USB Upgrade

HP ScanJet 6200C/6250C Scanners

This file contains information about upgrading USB to use with your scanner. If    the HP installation program stated that no scanner interface was found, but you know you have USB, your PC needs to be upgraded. Follow the instructions in this file to upgrade USB.

NOTE: For most users, the scanner is installed automatically and no upgrade is required.

Contents

How to run the USB upgrade program

Why the USB upgrade is included

How to run the USB upgrade program

Follow these instructions:

1)	Print this file. Refer to the instructions as you run the installation.

2)	Close all open applications within Windows.

3) 	On the HP Scanning Software CD, open the usb folder and run setup.exe.

To do this, right click on Start ->

-        choose Explore ->

-        choose the drive letter of your CD (such as E:\)

-        click to display the contents of the CD

-        double click to display the contents of the folder USB

-        double-click on setup.exe

4) 	Follow the instructions on the screen. Reboot your system as directed (you may

have to reboot more than once). When you have finished upgrading USB and you

have rebooted the PC, the system will display messages that it has now automatically

detected USB.

5) 	Run the scanning software installation once again. To run again, open My Computer

and double-click on the CD drive letter. (It is in the root directory on the HP Scanning

Software CD and is called Setup.exe).

6) 	Refer to the Setup Poster to complete the installation.

Why the USB upgrade is included

Some PCs (especially older PCs) come from the factory with the USB BIOS disabled.

It must be enabled for you to use USB for the scanner.

Some versions of Windows operating systems need to be upgraded to use USB.

(For those of you familiar with Windows operating system releases, Win 95 versions 1111

and 1212 need to be upgraded to the equivalent of version 1214.)

NOTE: Windows NT 4.0 does not support USB. If you have Win NT 4.0, use the SCSI

connection for the scanner.