- GenRef

- v1.00

- MDOS Reference guide.

- Utility Library

- (C) Copyright 1989

J. Paul Charlton

- ALL RIGHTS RESERVED

- _________________________________________

- UTILITY CONTENTS

__________________________________________________________________________________

- Page

- Utility Overview..................................1

- Calling Utility Functions.........................1

- Validate Time.....................................2

- Read Time.........................................3

- Set Time..........................................4

- Validate Date.....................................5

- Read Date.........................................6

- Set Date..........................................7

- Julian Date.......................................8

- Day of Week.......................................9

- Parse Filename....................................10

- Load Task.........................................13

- Fork Task.........................................14

__________________________________________________________________________________

UTILITY OVERVIEW                             __________________________________________________________________________________

- The memory management routines in MDOS  are  provided  to

aid  a  programmer  in writing applications which are larger

than the 64 Kbytes directly  addressable  by  the  CPU's  16

address  lines.   They  also  serve the purpose of providing

each task with it's own private address space, separate from

other the memory accessible to other tasks.

__________________________________________________________________________________

CALLING UTILITY FUNCTIONS                      __________________________________________________________________________________

- The MDOS utility functions must be called from  within  a

machine code program running as a task under MDOS.  You pass

arguments  to  the  utility  functions  using  only  a   few

registers of your program's workspace.

- The MDOS utility functions are  invoked  from  a  machine

code  program  when  software  trap  number  zero (XOP 0) is

called with a library number of 9.  The calling program's R0

must  contain  the  opcode of the routine within the utility

library which  is  to  be  performed.   The  following  code

fragment  will  return  the  day  of the week to the calling

task.

LI        R0,7

XOP       @NINE,0

MOV       R1,@WEEKDA

* ...

WEEKDA    DATA      0         day of the week (1-7):(Sun-Sat)

* ...

NINE      DATA      9

* ...

UTIL - 2                            GenREF v1.00

__________________________________________________________________________________

VALIDATE TIME                 __________________________________________________________________________________

Function            This operation is used to check the time stored in

the  clock chip for validity.  It insures that the

minutes and seconds are in  the  range  0:59,  and

insures that the hours are in the range 0:23.

Parameters          R0        = 0 (opcode)

Results             EQ status

- Parameter description

EQ status           The equal status bit will be set if  the  time  is

valid,  allowing  you  to  perform a "JEQ time$ok"

right after the software trap.

GenREF v1.00                           UTIL - 3

__________________________________________________________________________________

READ TIME                      __________________________________________________________________________________

Function                 This operation reads the  time  of  day  from  the

clock  chip, and places it into your string buffer

as a formatted string,  with  colons  between  the

hours, minutes, and seconds.

Parameters               R0        = 1 (opcode)

R1        = buffer

Results                  Buffer contains time string "HH:MM:SS".

- Parameter description

Buffer                   The buffer address you pass for the  string  is  a

16-bit  address  within your task's linear address

space.  The buffer must be  ten  characters  long,

and  the  address  you  pass is the address of the

second character in the buffer.

On return,  the  first  character  of  the  buffer

(offset  0)  will contain a length byte.  The next

eight characters,  starting  at  the  address  you

specified, will contain the formatted time string.

The last character in the buffer (offset  9)  will

contain a zero byte, for a null terminated string.

UTIL - 4                            GenREF v1.00

__________________________________________________________________________________

SET TIME                 __________________________________________________________________________________

Function            This operation will set the clock chip  using  the

time  in  the  formatting string which the calling

task passes as an argument.

Parameters          R0        = 2 (opcode)

R1        = string

Results             EQ status

- Parameter description

String              The address you pass for the string  is  a  16-bit

address  within  your task's linear address space.

The address you pass is the address of the  second

character  in the buffer (the first text character

in the string.)

The first character in the string buffer must be a

length  byte, giving the number of text characters

in the string.  Any leading spaces in  the  string

will be ignored.

The text of the string  must  have  the  following

format:

[h]h:[m]m[:[s][s]]

EQ status           The equal status bit  will  be  set  if  the  time

string  is  valid,  allowing you to perform a "JEQ

time$ok" right after the software trap.  The clock

chip  is not altered unless the EQ status has been

returned.

GenREF v1.00                           UTIL - 5

__________________________________________________________________________________

VALIDATE DATE               __________________________________________________________________________________

Function                 This operation is used to check the date stored in

the  clock chip for validity.  It insures that the

month is in the range 1:12, the day of  the  month

is the range 1:MAX_DAYS[month], the year is in the

range 0:99, and that the day of the week based  on

the  month-day-year  in the clock chip agrees with

the day of the  week  stored  in  the  clock  chip

itself.

Parameters               R0        = 3 (opcode)

Results                  EQ status

- Parameter Description

EQ status                The equal status bit will be set if  the  date  is

valid,  allowing  you  to  perform a "JEQ date$ok"

right after the software trap.

UTIL - 6                            GenREF v1.00

__________________________________________________________________________________

READ DATE                 __________________________________________________________________________________

Function            This operation reads the date from the clock chip,

and  places  it  into  your  string  buffer  as  a

formatted string, with a dash between  the  month,

day, and year.

Parameters          R0        = 4 (opcode)

R1        = buffer

Results            Buffer contains date string "mm-dd-yy".

- Parameter Description

Buffer              The buffer address you pass for the  string  is  a

16-bit  address  within your task's linear address

space.  The buffer must be  ten  characters  long,

and  the  address  you  pass is the address of the

second character in the buffer.

On return,  the  first  character  of  the  buffer

(offset  0)  will contain a length byte.  The next

eight characters,  starting  at  the  address  you

specified,  will contain the formatted date string

"mm-dd-yy".  The  last  character  in  the  buffer

(offset  9)  will  contain a zero byte, for a null

terminated string.

GenREF v1.00                          UTIL - 7

__________________________________________________________________________________

SET DATE                      __________________________________________________________________________________

Function                 This operation will set the clock chip  using  the

date  in  the  formatting string which the calling

task passes as an argument.

Parameters               R0        = 5 (opcode)

R1        = string

Results                  EQ status

- Parameter description

string                   The address you pass for the string  is  a  16-bit

address  within  your task's linear address space.

The address you pass is the address of the  second

character  in the buffer (the first text character

in the string.)

The first character in the string buffer must be a

length  byte, giving the number of text characters

in the string.  Any leading spaces in  the  string

will be ignored.

The text of the string  must  have  the  following

format:

[m]m/[d]d[/[y][y]] [m]m-[d]d[-[y][y]]

EQ status                The equal status bit  will  be  set  if  the  date

string  is  valid,  allowing you to perform a "JEQ

date$ok" right after the software trap.  The clock

chip  is not altered unless the EQ status has been

returned.

UTIL - 8                            GenREF v1.00

__________________________________________________________________________________

JULIAN DATE                 __________________________________________________________________________________

Function            This  operation  performs  the   function   of   a

perpetual  calendar,  and  will  work  on any date

after January 1st, 1 AD.

Parameters          R0        = 6 (opcode)

R1        = month

R2        = day

R3        = year

Results             R1,R2     = julian date

- Parameter description

Year                This must be the full year, like "1989", not "89",

for  the  year  in  which  this  documentation was

written.

Julian date         This is the number of days since January 1st, 4712

- B.C.

GenREF v1.00                          UTIL - 9

__________________________________________________________________________________

DAY OF WEEK                      __________________________________________________________________________________

Function                 Returns the day of the week, from one (Sunday)  to

seven (Saturday).

Parameters               R0        = 7 (opcode)

Results                  R1        = weekday

- Parameter description

Weekday                  This is a sixteen bit integer with  a  value  from

>0001 (Sunday) to >0007 (Saturday).

UTIL - 10                          GenREF v1.00

__________________________________________________________________________________

PARSE FILENAME                 __________________________________________________________________________________

Function            This operation will  convert  a  logical  filename

descriptor   to  a  physical  filename  descriptor

recognized by the Device  Service  Routines.   For

disk  devices,  the  conversion  may depend on the

drive currently set for the task and  the  current

subdirectory   on  the  drive  (depending  on  the

ambiguity  left  in  the  name  by   the   calling

program.)

It  is  useful  when  you  wish   to   make   your

application program independent of which device it

was loaded from or when your application must  ask

a user for a filename.

Parameters          R0        = 8 (opcode)

R1        = logical name

R2        = physical name

R3        = alias flag

Results             R0        = delimiter

R1        = error code

EQ status

- Parameter description

Logical name        This is the address of the first character in  the

string to be converted to a physical device name.

At first, the name is compared to the names of all

character  devices  recognized  by  MDOS.   If the

string matches the name of any  of  the  character

devices,    the  string  will  be  copied  without

modification  to  the  specified   string   output

buffer.

- There  are  three  separators  regonized  by  this

routine  as  part  of a disk path name: COLON ":",

- PERIOD ".",  and  BACKSLASH  "\".   If  the  first

separator  found before a terminating delimiter is

a PERIOD, the entire string will be copied without

modification   to   the  specified  string  output

buffer.

- The following characters indicate the end  of  the

input  string  if they are not contained inside of

double-quote   marks:   SPACE,    COMMA,    SLASH,

SEMICOLON.    The   NIL   character  (>00)  always

terminates the input string,  even  if  there  are

unmatched  double  quotes  in  the  string.  These

characters are referred to as "terminal characters".

GenREF v1.00                      UTIL - 11

- The  terminal  characters  along  with  the  three

separator characters are known as "delimiters".

Remaining filenames are parsed as follows:

- Part A, drive alias. All characters parsed  during

this phase are ignored in subsequent phases of the

parsing.

caller R3    alias flag <>0                -> null string

"volume:"    + non-terminal                -> "WDS.volume."

"volume:"    + terminal character          -> "volume"

"n:"         + terminal character          -> "alias"

"n:"         + non-terminal, "\", or "."   -> "alias."

all others                                 -> "alias."

- Part B, current directory.  Using  the  characters

remaining in the input string after Part A.

"\"          is first character            -> null string

current dir  is NULL                       -> null string

"."          + terminator                  -> "CURDIR"

".."         + terminator                  -> "PARENTDIR"

".\"         + non-delimiters              -> "CURDIR."

"..\\"       + non-delimiters              -> "PARENTDIR."

all others                                 -> "CURDIR."

- Part C, file specifier.  This  is  the  characters

remaining  in  the  string after Part A and Part B

have been done.  These characters are copied  into

the  output  buffer  until  a BACKSLASH, QUOTE, or

terminator is found.  When a BACKSLASH  is  found,

it  is replaced by a PERIOD.  When a terminator is

found, parsing of the input string is stopped, and

the  address  of the terminator is returned to the

caller.  When a QUOTE  is  found,  all  characters

until  the  next  QUOTE or NIL in the input string

are  copied  to  the  output  buffer  (Unless  two

matching  QUOTES  were  adjacent to each other, in

which case a single QUOTE character will be placed

into the output buffer.)

- The resulting string in the output buffer is "PART

- A"  +  "PART B" + "PART C", and is returned to the

caller.

UTIL - 12                                GenREF v1.00

Phys. name          This is most useful when  it  specifies  the  name

length byte in a PAB.

Note that the  physical  name  can  use  the  same

buffer  as  the  logical  name,  and  will  simply

overwrite  the  logical  name  after  parsing   is

complete.   As  a  caller,  you  must  specify the

address of the length byte in your  string  output

buffer  with  this  parameter.  Before calling the

parse routine, you must set the length byte to the

maximum  length  allowed  for  the  output string,

which is returned in the form "<len><chars><nil>".

Alias flag          This  flag  must  be  set  to  zero   for   normal

processing.   If  this  flag  is non-zero, no disk

drive  alias  will  be  prepended  to  the  output

filename.  (This feature is used only by the CHDIR

command of MDOS at present.)

Delimiter           This is the address of the first character in your

input  string  which  wasn't  processed during the

generation of the filename.   Note  that  this  is

designed  in  such  a  fashion  that you generally

don't need your own  routine  to  parse  filenames

which  are  passed to your program as command line

parameters; just call the parse routine, check the

delimiter,  and  call  the parse routine again for

the next command line parameter.

Error code          This is set to zero if no errors were  encountered

while  parsing  the  filename.   This  is non-zero

under several  conditions:  the  resulting  output

string  was  too  long for your buffer, a COLON is

the first character in the input string,  a  drive

specified  with  "n:"  does  not  have an assigned

alias, or a directory specifier of the form "." or

".."   was  not  followed  by  a  BACKSLASH  or  a

terminator.

GenREF v1.00                         UTIL - 13

__________________________________________________________________________________

LOAD TASK                      __________________________________________________________________________________

Function                 This operation will load a chained  program  image

file  into  memory,  and  cause it to execute as a

task under MDOS.  Invocation of the new task  will

start  at address >0400 with a workspace of >F000,

and memory windows 0..6 of the task will initially

contain the data loaded from the program image.

Parameters               R0        = 9 (opcode)

R1        = physical name

Results                  R0        = error code

R1        = child page zero (physical page number)

>00E8     in child task contains the physical page number

of the parent's page zero

- Parameter description

Phys. name               This is the  address  of  the  length  byte  of  a

filename stored in the format "<len><chars>".

- Error code

0         = no error, task was loaded

1         = insufficient memory

2         = invalid filename

3         = image file found, with invalid header

- Image file header

An image file header  has  the  following  format,

compatible with GenLINK.

byte 0    if >00, last image in chain, otherwise bump

filename and load another image in the chain

byte 1    "G" (normal speed) or "F" (use fast memory)

byte 2,3  length of this image file

byte 4,5  load address of this image file

byte 6..len+6       image data bytes

UTIL - 14                           GenREF v1.00

__________________________________________________________________________________

FORK TASK                 __________________________________________________________________________________

Function            This operation causes the creation of a  new  task

under MDOS.  The new task (child task) is an exact

copy of the calling task (parent task).

The new task has a virtual  memory  address  space

which  has  one  physical  memory  page  for  each

physical page used by the  parent  task.   If  the

parent  task  was  using  shared memory, the child

task can also  use  the  same  shared  memory  and

communicate with the parent task.

Further note:  The terms "parent" and "child"  are

used  as  a convenience in differentiating between

the calling task and the newly created  task.   In

MDOS itself, there is no concept of "task tree" or

parent-child relationship  as  there  is  in  some

other  operating  systems.  In MDOS, all tasks are

peers.

Parameters          R0        = 10 (opcode)

Results             parent task:

R0        = -1 (error)

= otherwise, this is the physical page number of the

child task's header page.

PC        = program execution continues with the instruction

after the XOP call.  This instruction must be a

single word instruction such as a Jump

instruction.

child task:

R0        = -1

PC        = program execution continues with the 2nd

instruction after the XOP call.  Note that the

instruction after the XOP call must be a single

word instruction.

>00E8     in child task contains the physical page number

of the parent's page zero

GenREF v1.00                  UTIL - 15

- Parameter Description

error                    An error code of -1 will be returned to the parent

task  if  there  is not enough memory available on

your system to create a clone of your task.

- Example code

LI        R0,10

XOP       @NINE,0

JMP       PARENT

CHILD     PRINT     "This is the child speaking..."

BLWP      @0         exit

PARENT    PRINT     “Child: be quiet!”

BLWP      @0