- GenRef

- v1.00

- MDOS Reference guide.

- Video Library

- (C) Copyright 2004

Beery W. Miller

ALL RIGHTS RESERVED

____________________________________________________________________________________

Video - CONTENTS               ____________________________________________________________________________________

Video Overview	4

Video Modes	4

Video Registers	5

Text Modes	7

Text1	7

Text2A	7

Text2B	8

MultiColor	8

MultiColor	8

Graphic Modes	9

Graphic1	9

Graphic2	9

Graphic3	10

Graphic4	10

Graphic5	11

Graphic6	11

Graphic7	12

Video Page Conceps	13

Video Sprites	14

Sprite Mode 1	14

Sprite Mode 2	14

Video Color Table	15

Calling Video Functions	16

Set Video Mode	17

Get Video Mode	18

Set Cursor Position	19

Get Cursor Position	20

Set Display Page	21

Get Display Page	22

Scroll Window Up	23

Scroll Window Down	24

Scroll Window Left	25

Scroll Window Right	26

Call Screen	27

Get Character Color	28

Set Border Color	29

Set Color Palette	30

Set Pixel Color	31

Get Pixel Color	32

Set Vector Color	33

Color Search	34

High Speed Block Move	35

High Speed Block Copy	36

Logical Block Copy	38

Block Scroll Up	39

Block Scroll Down	40

Block Scroll Left	41

Block Scroll Right	42

Sprite Define	43

Delete Sprite	44

Locate Sprite	45

Sprite Motion	46

Sprite Color	47

Define Sprite Pattern	48

Sprite Magnify	49

Sprite Pattern / Distance	50

Sprite Coincidence	51

Sprite Pattern Define / Get	52

Character Pattern Define/Get	53

Set Text Window	54

Get Text Window	55

Write TTY	56

Restore Character / Sprite Pattern	58

Set Text Color	59

Write Character String	60

Print Screen	61

Horizontal Character / Color (HCHAR)	62

Vertical Character / Color (VCHAR)	63

Horizontal Character (HCHAR)	64

Vertical Character (VCHAR)	65

Set Mouse	66

Get Mouse	67

Get Mouse Release	68

Make Sound	69

Musical Tone Frequencies	70

Sound Status	71

VWTR	72

VRFR	73

Get Tables	74

Get Palette Registers	75

____________________________________________________________________________________

VIDEO - OVERVIEW ____________________________________________________________________________________

The video management routines in MDOS are provided to aid a programmer in writing applications requiring video input and output.  The V9938 processor contained aboard the Geneve 9640 is upward compatible with the existing TMS9918A used in the TI-99/4A.  The V9938 was developed through the joint efforts of ASCII Corporation, Microsoft Inc., and YAMAHA.

The following functions are supported on the V9938.

- Full bit-mapped mode

- 80-column text display

- Access using x- and y- coordinates independent of the screen mode.

- Hardware commands internal the V9938 including AREA MOVE, LINE, SEARCH, RASTER OPERATION, etc.

- More sprites per horizontal line than the TMS9918A.

- Maximum 512 x 424 pixels, 16 colors

- Bit-mapped graphics

- Interfaces the bus mouse

- Maximum 8 sprites per horizontal line

- Logical operation function

- And more

____________________________________________________________________________________

VIDEO MODES           ____________________________________________________________________________________

The video management routines in MDOS support 11 different modes of display. The following table describes those modes and their properties.

| Mode | Size | Video Mode # | Colors | Sprite Mode | Number of Display Pages |
| --- | --- | --- | --- | --- | --- |
| TEXT1 | 40 x 24 | >0000 | 2 | N | 32 |
| TEXT2A | 80 x 24 | >0001 | 2 | N | 16 |
| MULTICOLOR | 64 x 48 | >0002 | 16 | 1 | 32 |
| GRAPHIC1 | 32 x 24 | >0003 | 16 | 1 | 32 |
| GRAPHIC2 | 32 x 24 | >0004 | 16 | 1 | 8 |
| GRAPHIC3 | 256 x 212 | >0005 | 16 | 2 | 8 |
| GRAPHIC4 | 512 x 212 | >0006 | 16 | 2 | 4 |
| GRAPHIC5 | 512 x 212 | >0007 | 4 | 2 | 4 |
| GRAPHIC6 | 512 x 212 | >0008 | 16 | 2 | 2 |
| GRAPHIC7 | 256 x 212 | >0009 | 256 | 2 | 2 |
| TEXT2B | 80 x 26.5 | >000A | 2 | N | 16 |

____________________________________________________________________________________

VIDEO REGISTERS ____________________________________________________________________________________

The V9938 uses 49 internal registers for its screen operations. These registers are referred to as "VDP registers" in this book. VDP registers are classified by function into three groups as described below. The control register group and status register group can be referred to using VDP(n) system variables. By using the MDOS Video XOP functions described later in this manual, we do not need to be concerned with directly accessing these registers.  Further information on direct programming of the V9938 can be obtained by obtaining a copy of the V9938 Technical Manual available on ftp.whtech.com as a PDF file or on http://map.tni.nl/ as either a PDF file or HTML document.

Control register group (R#0 to R#23, R#32 to R#46)

This is a read-only 8-bit register group controlling V9938 actions. Registers are expressed using the notation R#n. R#0 to R#23 are used to set the screen mode. R#32 to R#46 are used to execute VDP commands. These VDP commands will be described in detail later. Control registers R#24 to R#31 do not exist. The roles of the different control registers are listed below.

Control register list

____________________________________________________________

|  	| Corres- 	|	|

|  R#n   	| ponding	| Function	|

|         	| VDP(n)	|	|

|----------+-----------+--------------------------------------------------------------------------	|

| R#0  	| VDP(0)  	| mode register #0	|

| R#1  	| VDP(1)  	| mode register #1	|

| R#2  	| VDP(2)  	| pattern name table	|

| R#3  	| VDP(3)  	| colour table (LOW)	|

| R#4  	| VDP(4)  	| pattern generator table	|

| R#5  	| VDP(5)  	| sprite attribute table (LOW) 	|

| R#6  	| VDP(6)  	| sprite pattern generator table	|

| R#7  	| VDP(7)  	| border colour/character colour at text mode	|

| R#8  	| VDP(9)  	| mode register #2	|

| R#9  	| VDP(10) | mode register #3	|

| R#10 	| VDP(11) | colour table (HIGH)	|

| R#11 	| VDP(12) | sprite attribute table (HIGH)	|

| R#12 	| VDP(13) | character colour at text blinks	|

| R#13 	| VDP(14) | blinking period	|

| R#14 	| VDP(15) | VRAM access address (HIGH)	|

| R#15 	| VDP(16) | indirect specification of S#n	|

| R#16 	| VDP(17) | indirect specification of P#n	|

| R#17	| VDP(18) | indirect specification of R#n	|

| R#18 	| VDP(19) | screen location adjustment (ADJUST)	|

| R#19 	| VDP(20) | scanning line number when the interrupt occurs  	|

| R#20	| VDP(21) | colour burst signal 1	|

| R#21 	| VDP(22) | colour burst signal 2	|

| R#22 	| VDP(23) | colour burst signal 3	|

| R#23 	| VDP(24) | screen hard scroll	|

_______________________________________________________________

|  	| Corres- 	|	|

|  R#n   	| ponding	| Function	|

|         	| VDP(n)	|	|

|----------+-----------+--------------------------------------------------------------------------------	|

| R#32 	| VDP(33)	| SX: X-coordinate to be transferred (LOW)	|

| R#33 	| VDP(34)	| SX: X-coordinate to be transferred (HIGH)	|

| R#34	| VDP(35)	| SY: Y-coordinate to be transferred (LOW)	|

| R#35 	| VDP(36) | SY: Y-coordinate to be transferred (HIGH)	|

| R#36 	| VDP(37) | DX: X-coordinate to be transferred to (LOW)	|

| R#37 	| VDP(38) | DX: X-coordinate to be transferred to (HIGH) 	|

| R#38 	| VDP(39) | DY: Y-coordinate to be transferred to (LOW)	|

| R#39 	| VDP(40) | DY: Y-coordinate to be transferred to (HIGH) 	|

| R#40 	| VDP(41) | NX: num. of dots to be transferred in X direction (LOW)  	|

| R#41 	| VDP(42) | NX: num. of dots to be transferred in X direction (HIGH)	|

| R#42 	| VDP(43) | NY: num. of dots to be transferred in Y direction (LOW)  	|

| R#43 	| VDP(44) | NY: num. of dots to be transferred in Y direction (HIGH) 	|

| R#44 	| VDP(45) | CLR: for transferring data to CPU	|

| R#45 	| VDP(46) | ARG: bank switching between VRAM and expanded VRAM|

| R#46	| VDP(47)	| CMR: send VDP command	|

Status register (S#0 to S#9)

This is a read-only 8-bit register group which reads data from the V9938.   Registers are expressed using the notation S#n. The functions of the registers are listed below.

Status register list

_______________________________________________________________

|  	| Corres- 	|	|

| S#n   	| ponding	| Function	|

|         	| VDP(n)	|	|

|----------+-----------+--------------------------------------------------------------------------------	|

| S#0  	| VDP(8)  	| interrupt information	|

| S#1  	| VDP(-1) 	| interrupt information	|

| S#2  	| VDP(-2) 	| DP command control information/etc.	|

| S#3  	| VDP(-3) 	| coordinate detected (LOW)	|

| S#4  	| VDP(-4) 	| coordinate detected (HIGH)	|

| S#5  	| VDP(-5) 	| coordinate detected (LOW)	|

| S#6  	| VDP(-6) 	| coordinate detected (HIGH)	|

| S#7  	| VDP(-7) 	| data obtained by VDP command 	|

| S#8 	| VDP(-8) 	| X-coordinate obtained by search command (LOW)	|

| S#9  	| VDP(-9) 	| X-coordinate obtained by search command (HIGH)	|

Colour palette register group (P#0 to P#15)

These registers are used to set the colour palette. Registers are expressed using the notation P#n where 'n' is the palette number which represents one of 512 colours. Each palette register has 9 bits allowing three bits to be used for each RGB colour (red, green, and blue).

Text Modes

Video Mode – TEXT1

Characteristics	Pattern Size		6 dots (w) x 8 dots (h)

Patterns			256 types

Screen pattern count	40 (w) x 24 (h) patterns

Pattern colors		Two colors out of 512 colors (per screen)

VRAM area per screen	4K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Pattern color code 1	High-order four bits of R#7

Pattern color code 0	Low-order four bits of R#7

Background color code	Low-order four bits of R#7

The area in which character fonts are stored is called the pattern generator table. This table is located in VRAM, and, although the font is defined by using 8 bytes for each character from the top of the table, the 2 low order bits of each byte representing the right two columns are not displayed on the screen. Thus, the size of one character is 6 x 8 pixels. Each character font set contains 256 different characters numbered from 0 to 255.

Video Mode – TEXT2A

Characteristics	Pattern Size		6 dots (w) x 8 dots (h)

Patterns			256 types

Screen pattern count	80 (w) x 24 (h) patterns

Pattern blinking		Possible for each character

Pattern colors		Two colors out of 512 colors (per screen)

Four if using blinking

VRAM area per screen	8K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Blink attributes	VRAM color table

Pattern color code 1	High-order four bits of R#7

Pattern color code 0	Low-order four bits of R#7

Background color code	Low-order four bits of R#7

Pattern color code 1	High-order four bits of R#12

(Used for blinking)

Pattern color code 0	Low-order four bits of R#12

(Used for blinking)

The area in which character fonts are stored is called the pattern generator table. This table is located in VRAM, and, although the font is defined by using 8 bytes for each character from the top of the table, the 2 low order bits of each byte representing the right two columns are not displayed on the screen. Thus, the size of one character is 6 x 8 pixels. Each character font set contains 256 different characters numbered from 0 to 255.

Video Mode – TEXT2B

Characteristics	Pattern Size		6 dots (w) x 8 dots (h)

Patterns			256 types

Screen pattern count	80 (w) x 26.5 (h) patterns

Pattern blinking		Possible for each character

Pattern colors		Two colors out of 512 colors (per screen)

Four if using blinking

VRAM area per screen	8K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Blink attributes	VRAM color table

Pattern color code 1	High-order four bits of R#7

Pattern color code 0	Low-order four bits of R#7

Background color code	Low-order four bits of R#7

Pattern color code 1	High-order four bits of R#12

(Used for blinking)

Pattern color code 0	Low-order four bits of R#12

(Used for blinking)

The area in which character fonts are stored is called the pattern generator table. This table is located in VRAM, and, although the font is defined by using 8 bytes for each character from the top of the table, the 2 low order bits of each byte representing the right two columns are not displayed on the screen. Thus, the size of one character is 6 x 8 pixels. Each character font set contains 256 different characters numbered from 0 to 255.

MultiColor Mode

Video Mode – MULTICOLOR

Characteristics	Screen composition		64 (w) x 48 (h) color blocks

Color blocks		16 colors out of 512 colors

Sprite mode			Sprite Mode 1

VRAM area per screen	4K bytes

Controls	Color block color code	VRAM pattern generator table

Color block location	VRAM pattern name table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table

VRAM sprite pattern table

The pattern generator table is an area that stores the colors of the color blocks.  Each pattern is made up of four color blocks.  These patterns are approximately 8 x 8 when the dots available for the screen display area is 256 x 192 dots.  In MULTICOLOR mode, two bytes are used for each pattern, and each pattern includes four color blocks.

Graphic Modes

Video Mode – GRAPHIC1

Characteristics	Pattern Size		8 dots (w) x 8 dots (h)

Patterns			256 types

Screen pattern count	32 (w) x 24 (h) patterns

Pattern colors		16 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 1

VRAM area per screen	4K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Pattern color codes 1 & 0	Can be specified as a group for each

8-pattern set, in the VRAM color table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern generator table is an area that stores the pattern fonts.  Each pattern has a number from 0 to 255.  The font for each pattern is constructed from 8 bytes.  The pattern name table is composed of one byte for each screen pattern.  Each byte specifies a unique pattern.

Video Mode – GRAPHIC2

Characteristics	Pattern Size		8 dots (w) x 8 dots (h)

Patterns			768 types

Screen pattern count	32 (w) x 24 (h) patterns

Pattern colors		16 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 1

VRAM area per screen	16K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Pattern color codes 1 & 0	Can be specified as a group for each

raster, in the VRAM color table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern generator table is an area that stores the pattern fonts.  Each pattern group has a number from 0 to 255; and since each group may have three members, 768 patterns may be specified.  The font for each pattern is constructed from 8 bytes.

Video Mode – GRAPHIC3

Characteristics	Pattern Size		8 dots (w) x 8 dots (h)

Patterns			768 types

Screen pattern count	32 (w) x 24 (h) patterns

Pattern colors		16 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 2

VRAM area per screen	16K

Controls	Pattern font	VRAM pattern generator table

Screen pattern location	VRAM pattern name table

Pattern color codes 1 & 0	Can be specified as a group for each

raster, in the VRAM color table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern generator table is an area that stores the pattern fonts.  Each pattern group has a number from 0 to 255; and since each group may have three members, 768 patterns may be specified.  The font for each pattern is constructed from 8 bytes.

Video Mode – GRAPHIC4

Characteristics	Bit-mapped Graphics Mode

Screen Size			256 (w) x 212 (h) dots

256 (w) x 192 (h) dots

Screen colors		16 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 2

VRAM area per screen	32K

Controls	Graphics	VRAM pattern name table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern name table is composed of one byte for every two dots on the screen.  A color can be assigned for each dot from a selection of 16 colors out of 512 colors.

Video Mode – GRAPHIC5

Characteristics	Bit-mapped Graphics Mode

Screen Size			512 (w) x 212 (h) dots

512 (w) x 192 (h) dots

Screen colors		4 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 2

VRAM area per screen	32K

Controls	Graphics	VRAM pattern name table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern name table is composed of one byte for every four dots on the screen.  A color can be assigned for each dot from a selection of 4 colors out of 512 colors.  A hardware tiling function processes the sprite and background colors.  For these colors, you can specify four bits; however, of these four bits, the higher-order two bits specify the color code of the even dots, and the lower-order two bits specify the color code of the odd dots of the x-coordinate (0 to 511).  The size of one dot of a sprite is approximately twice that of a graphic dot; however, when this tiling function is used, one dot of a sprite may be displayed in two colors.  The even and odd dots of the background color may also be specified in the same manner.

Video Mode – GRAPHIC6

Characteristics	Bit-mapped Graphics Mode

Screen Size			512 (w) x 212 (h) dots

512 (w) x 192 (h) dots

Screen colors		16 colors out of 512 colors (per screen)

Sprite mode			Sprite Mode 2

VRAM area per screen	128K (Two screens)

Controls	Graphics	VRAM pattern name table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern name table is composed of one byte for every two dots on the screen.  A color can be assigned for each dot from a selection of 16 colors out of 512 colors.  .

Video Mode – GRAPHIC7

Characteristics	Bit-mapped Graphics Mode

Screen Size			256 (w) x 212 (h) dots

256 (w) x 192 (h) dots

Screen colors		256 colors  (per screen)

Sprite mode			Sprite Mode 2

VRAM area per screen	128K (Two screens)

Controls	Graphics	VRAM pattern name table

Background color code	Low-order four bits of R#7

Sprites	VRAM sprite attribute table, VRAM sprite

Pattern table

The pattern name table is composed of one byte for every dot on the screen.  A color can be assigned for each dot from a selection of 256 colors.  .

VIDEO PAGE CONCEPTS

The parameters used for the V9938 are all x-y coordinates.  In other words, the internal command processor of the V9938 accesses the entire VRAM area as x-y coordinates of the display mode.

When a screen is to be displayed, 212 lines of the same page are displayed.  Use the Set Display Page Video Opcode >04 to select the page.  The display modes and their relationships to the coordinates are shown in the table below.

GRAPHIC4                                                                                        GRAPHIC5

| (0,0) |  | (255,0) |  | (0,0) |  | (511,0) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Page 0 |  |  |  | Page 0 |  |
| (0,255) |  | (255,255) |  | (0,255) |  | (511,255) |
| (0,256) |  | (255,256) |  | (0,256) |  | (511,256) |
|  | Page 1 |  |  |  | Page 1 |  |
| (0,511) |  | (255,511) |  | (0,511) |  | (511,511) |
| (0,512) |  | (255,512) |  | (0,512) |  | (511,512) |
|  | Page 2 |  |  |  | Page 2 |  |
| (0,767) |  | (255,767) |  | (0,767) |  | (511,767) |
| (0,768) |  | (255,768) |  | (0,768) |  | (511,768) |
|  | Page 3 |  |  |  | Page 3 |  |
| (0,1023) |  | (255,1023) |  | (0,1023) |  | (511,1023) |

GRAPHIC7                                                                                        GRAPHIC6

| (0,0) |  | (255,0) |  | (0,0) |  | (511,0) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Page 0 |  |  |  | Page 0 |  |
| (0,255) |  | (255,255) |  | (0,255) |  | (511,255) |
| (0,256) |  | (255,256) |  | (0,256) |  | (511,256) |
|  | Page 1 |  |  |  | Page 1 |  |
| (0,511) |  | (255,511) |  | (0,511) |  | (511,511) |

Video Sprites

VIDEO SPRITE MODES

Sprite Mode 1	In SPRITE MODE 1, there are 32 sprites, numbered >00 to >1F.  The sprites assigned the lower numbers have a higher priority.  On a single CRT horizontal line, up to 4 sprites with the highest priority are displayed, and the overlapping portions of sprites with lower priorities are not displayed.

When two sprites collide (their pattern color 1 portions have overlapped), this condition can be detected.

Characteristics	Sprite size		8 x 8 dots (normal)

16 x 16 dots (magnified)

Number of sprites	32 sprites

Sprite Mode 2	In SPRITE MODE 2, there are 32 sprites, numbered >00 to >1F.  The sprites assigned the lower numbers have a higher priority.  On a single CRT horizontal line, up to 8 sprites with the highest priority are displayed, and the overlapping portions of sprites with lower priorities are not displayed.

When two sprites collide (their pattern color 1 portions have overlapped), this condition can be detected.

The colors of the sprite may be specified for each horizontal line.

Characteristics	Sprite size		8 x 8 dots (normal)

16 x 16 dots (magnified)

Number of sprites	32 sprites

VIDEO COLOR TABLE

Color Name       Code    Pallette Register Settings

Red	Green	Blue

Transparent	>00	0	0	0

Black	>01	0	0	0

Medium Green	>02	1	6	1

Light Green	>03	3	7	3

Dark Blue	>04	1	1	7

Light Blue	>05	2	3	7

Dark Red	>06	5	1	1

Cyan	>07	2	6	7

Medium Red	>08	7	1	1

Light Red	>09	7	3	3

Dark Yellow	>0A	6	6	1

Light Yellow	>0B	6	6	4

Dark Green	>0C	1	4	1

Magenta	>0D	6	2	5

Gray	>0E	5	5	5

White	>0F	7	7	7

____________________________________________________________________________________

CALLING VIDEO FUNCTIONS

____________________________________________________________________________________

The MDOS Video Library must be called from within a machine code program running as a task under MDOS. You pass arguments to the Video Library via the calling registers.

The MDOS Video Library is invoked from a machine code program when software trap number zero (XOP 0) is called with a library number of 6. The calling program's R0 must contain the 16-bit subprogram  at the time of the XOP.  The following code fragment will set the video mode to 80 x 24 column text mode and write “This is a sample text string <CR/LF>” to the screen before exiting back to the prompt.

LI	R0,>0000	Set Video Mode

LI	R1,>0001	Text 2 mode

XOP	@SIX,0	Access subprogram

LI	R0,>0027	Write Text Routine

LI	R1,STR1	String to write

CLR	R2	Null terminated string

XOP	@SIX,0	Access subprogram

BLWP	@0	Exit

SIX	DATA	>0006

STR1	TEXT	“This is a sample text string”

BYTE	>0D,>0A,0	CR/LF/Null terminated

EVEN

In the preceding example, three hidden assumptions were made.  First it is assumed that STR1 is located on a page which is currently mapped into a memory page which has the same 16-bit address page number as its Virtual address page number (read the section on Memory Management.) The second assumption is that SIX is actually at the virtual address SIX, not in some overlay segment with a different virtual address.

Video Mode Library

Set Video Mode

Function	Sets video mode and returns the current border color.  For bitmap modes, the background color will be the border color in effect at time of call. This subprogram enables you to select the graphics or text mode that offers you the combination of text and/or graphics capabilities that best suits the particular needs of your program.

When you call the subprogram, the following occurs:

- Clears the entire screen

- Restores the default character definitions of all characters

- Restores the default foreground color and background color of all characters.

- Restores the default screen color.

- Deletes all sprites.

- Resets all sprites.

- Resets the sprite magnification level to 1

- Restores the default current position (X=0, Y=0)

Parameters	R0x =  >0000

R1x = 	Video Mode

Results	None

Parameter Description

Video Mode

| Mode | Size | Video Mode # |
| --- | --- | --- |
| TEXT1 | 40 x 24 | >0000 |
| TEXT2A | 80 x 24 | >0001 |
| MULTICOLOR | 64 x 48 | >0002 |
| GRAPHIC1 | 32 x 24 | >0003 |
| GRAPHIC2 | 32 x 24 | >0004 |
| GRAPHIC3 | 256 x 212 | >0005 |
| GRAPHIC4 | 512 x 212 | >0006 |
| GRAPHIC5 | 512 x 212 | >0007 |
| GRAPHIC6 | 512 x 212 | >0008 |
| GRAPHIC7 | 256 x 212 | >0009 |
| TEXT2B | 80 x 26.5 | >000A |

Get Video Mode

Function	Returns the video mode presently and parameters of the video mode.

Parameters	R0x = >0001

Results	R0x = Returned video mode

R1x = Number of columns

R2x = Number of rows

R3x = Number of Graphic columns (pixels)

R4x = Number of Graphical rows (pixels)

R5x = Current page offset (in pixel rows, use for chip commands)

R6x = Color of screen border

R7h = Foreground color of text

R7l  = Background color of text

Parameter Description

Set Cursor Position

Function	Sets the current cursor position to the designated location on the display screen.

Parameters	R0x = >0002

R1x = Row Number

R2x = Column Number

Results	None

Parameter Description

Get Cursor Position

Function	Gets the current position of the cursor on the display screen.

Parameters	R0x = >0003

Results	R0x = Returned row number

R1x = Returned column number

Parameter Description

Set Display Page

Function	Set’s the current display page for the video mode presently being used.

Parameters	R0x = >0004

R1x = Page number

Results	None

Parameter Description

Get Display Page

Function	Returns the current display page number.

Parameters	R0x = >0005

Results	R0x = Display page number

Parameter Description

Scroll Window Up

Function	Scrolls the currently defined window up a defined number of lines.

Parameters	R0x = >0006

R1x = Number of lines to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Character value for blank lines

R7h  = Foreground color for blank lines

R7l   = Background color for blank lines

Results	None

Parameter Description

Scroll Window Down

Function	Scrolls the currently defined window down a defined number of lines.

Parameters	R0x = >0007

R1x = Number of lines to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Character value for blank lines

R7h  = Foreground color for blank lines

R7l   = Background color for blank lines

Results	None

Parameter Description

Scroll Window Left

Function	Scrolls the currently defined window left a defined number of lines.

Parameters	R0x = >0008

R1x = Number of lines to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Character value for blank lines

R7h  = Foreground color for blank lines

R7l   = Background color for blank lines

Results	None

Parameter Description

Scroll Window Right

Function	Scrolls the currently defined window right a defined number of lines.

Parameters	R0x = >0009

R1x = Number of lines to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Character value for blank lines

R7h  = Foreground color for blank lines

R7l   = Background color for blank lines

Results	None

Parameter Description

Call Screen

Function	Set’s the background and foreground color of the currently displayed screen. The screen color is the color of the border and the color displayed when transparent is specified as the foreground or background color of a character or pixel.

Parameters	R0x = >000A

R1h =  Foreground color (if R3 = 0)

R1l  =  Background color

R2x =  Character Set # (if mode 3)

R3x =  Flag (0 is change foreground, !0 is leave it alone)

Results	None

Parameter Description

Background color	A numeric expression whose value specifies a screen color from among the 16 available colors.

Foreground color	A numeric expression whose value specifies a screen color from among the 16 available colors.

Get Character Color

Function	Returns the character and the foreground and background color from a defined position on the screen.

Parameters	R0x = >000B

R1x =  Row

R2x =  Column

Results	R0x = ASCII character read from screen

R1h = Foreground color for character

R1l  = Background color for character

Parameter Description

Set Border Color

Function	Set’s the border color for the screen

Parameters	R0x =  >000C

R1x =  Color to render border

Results	None

Parameter Description

Set Color Palette

Function	Set’s the color Palette

Parameters	R0x = >000D

R1x =  Palette register number

R2x =  Color to put into palette register

R2:  =  xxxx | xGGG | xBBB | xRRR as a 16 bit word

Results	None

Parameter Description

Set Pixel Color

Function	Set Pixel Color of a point on a graphic mode screen with a logical operation done on the data of the dot tht is already dislayed.

Parameters	R0x = >000E

R1x = X coordinate of pixel

R2x = Y coordinate of pixel

R3h = Foreground color to render pixel

R3l  = Background color to render pixel in graphics mode 2 and 3.

R4h  = Logic operation to be performed

Results	None

Parameter Description

Logic Operation

Summary of Logical Operations

| Name | Operation | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- |
| IMP | DC = SC | 0 | 0 | 0 | 0 |
| AND | DC = SC*DC | 0 | 0 | 0 | 1 |
| OR | DC = SC+DC | 0 | 0 | 1 | 0 |
| EOR | DC = !SC*DC +SC*!DC | 0 | 0 | 1 | 1 |
| NOT | DC = !SC | 0 | 1 | 0 | 0 |
| --- |  | 0 | 1 | 0 | 1 |
| --- |  | 0 | 1 | 1 | 0 |
| --- |  | 0 | 1 | 1 | 1 |
| TIMP | If SC=0 then DC=DC else DC=SC | 1 | 0 | 0 | 0 |
| TAND | If SC=0 then DC=DC else DC=SC*DC | 1 | 0 | 0 | 1 |
| TOR | If SC=0 then DC=DC else DC=SC+DC | 1 | 0 | 1 | 0 |
| TEOR | If SC=0 then DC=DC else DC=!SC*DC +SC*!DC | 1 | 0 | 1 | 1 |
| TNOT | If SC=0 then DC=DC else DC=!SC | 1 | 1 | 0 | 0 |
| --- |  | 1 | 1 | 0 | 1 |
| --- |  | 1 | 1 | 1 | 0 |
| --- |  | 1 | 1 | 1 | 1 |

* SC = Source Color code

* DC = Destination Color code

* EOR = Exclusive OR

Get Pixel Color

Function	Returns the pixel color for the defined location.

Parameters	R0x = >000F

R1x =  X coordinate of pixel

R2x =  Y coordinate of pixel

Results	R0h = Returned foreground color of pixel

R0l  = Returned background color of pixel in graphics mode 2 & 3

Parameter Description

Set Vector Color

Function	The LINE command or Set Vector command draws a straight line between two points with a logical operation.  The line drawn is the hypotenuse that results after the long and short sides of a triangle are defined.  The two sides are defined as distances from a single point.

Parameters	R0x = >10

R1x = X coordinate of first pixel

R2x = Y coordinate of first pixel

R3x = X coordinate of second pixel

R4x = Y coordinate of second pixel

R5h = Foreground color to render vector

R5l  = Background color to render vector in graphic modes 2 & 3

R6l  = Logic operation to be performed

Results	None

Parameter Description

Logic Operation

Summary of Logical Operations

| Name | Operation | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- |
| IMP | DC = SC | 0 | 0 | 0 | 0 |
| AND | DC = SC*DC | 0 | 0 | 0 | 1 |
| OR | DC = SC+DC | 0 | 0 | 1 | 0 |
| EOR | DC = !SC*DC +SC*!DC | 0 | 0 | 1 | 1 |
| NOT | DC = !SC | 0 | 1 | 0 | 0 |
| --- |  | 0 | 1 | 0 | 1 |
| --- |  | 0 | 1 | 1 | 0 |
| --- |  | 0 | 1 | 1 | 1 |
| TIMP | If SC=0 then DC=DC else DC=SC | 1 | 0 | 0 | 0 |
| TAND | If SC=0 then DC=DC else DC=SC*DC | 1 | 0 | 0 | 1 |
| TOR | If SC=0 then DC=DC else DC=SC+DC | 1 | 0 | 1 | 0 |
| TEOR | If SC=0 then DC=DC else DC=!SC*DC +SC*!DC | 1 | 0 | 1 | 1 |
| TNOT | If SC=0 then DC=DC else DC=!SC | 1 | 1 | 0 | 0 |
| --- |  | 1 | 1 | 0 | 1 |
| --- |  | 1 | 1 | 1 | 0 |
| --- |  | 1 | 1 | 1 | 1 |

* SC = Source Color code

* DC = Destination Color code

* EOR = Exclusive OR

Color Search

Function	The SEARCH command searches for a border color in the Video RAM to the right or left of a basic point.

Parameters	R0x = >0011

R1x = X coordinate of source point

R2x = Y coordinate of source point

R3l  = Color for search

R3h = Direction for search (>00 = LEFT, >FF = RIGHT)

Results	EQ status

R0x = X coordinate of location where color was found

R1x = Y coordinate of location where color was found

Parameter Description

EQ Status	The equal status bit will be set if the function was valid and found the color, allowing you to perform a “JEQ Function$ok” right after the software trap.

High-Speed Block Move

Function	Move a portion of the screen contents from one destination to a second destination with VRAM.  Since the data to be transferred is done in units of one byte, there is a limitation, according to the display mode, on the value of X.

Parameters	R0x = >0012

R1x = Row number of upper left corner of source

R2x = Column number of upper left corner of source

R3x = Row number of upper left corner of destination

R4x = Column number of upper left corner of destination

R5x = Number of rows

R6x = Number of columns

R7l  = Pixel color for blank pixels

Results	None

Parameter Description

Note:  In Graphics Mode 4 & Graphics Mode 6, the lower one bit, and in Graphics 5 mode, the lower two bits, are lost.

High-Speed Block Copy

Function	Copies a portion of the screen contents from one destination to a second destination with VRAM.  Since the data to be transferred is done in units of one byte, there is a limitation, according to the display mode, on the value of X.

Parameters	R0x = >0013

R1x = Row number of upper left corner of source

R2x = Column number of upper left corner of source

R3x = Row number of upper left corner of destination

R4x = Column number of upper left corner of destination

R5x = Number of rows

R6x = Number of columns

Results	None

Parameter Description

Note:  In Graphics Mode 4 & Graphics Mode 6, the lower one bit, and in Graphics 5 mode, the lower two bits, are lost.

Logical Block Move

Function	Move a block of video from a source point to a destination point.  Since the data to be transferred is done in units of dots, logical operations may be done on the destination data.

Parameters	R0x = >0014

R1x = Row number of upper left corner of source

R2x = Column number of upper left corner of source

R3x = Row number of upper left corner of destination

R4x = Column number of upper left corner of destination

R5x = Number of rows

R6x = Number of columns

R7l  = Pixel color for blank pixels

R7h = Logic operation to be performed on destination

Results	None

Parameter Description

Logic Operation

Summary of Logical Operations

| Name | Operation | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- |
| IMP | DC = SC | 0 | 0 | 0 | 0 |
| AND | DC = SC*DC | 0 | 0 | 0 | 1 |
| OR | DC = SC+DC | 0 | 0 | 1 | 0 |
| EOR | DC = !SC*DC +SC*!DC | 0 | 0 | 1 | 1 |
| NOT | DC = !SC | 0 | 1 | 0 | 0 |
| --- |  | 0 | 1 | 0 | 1 |
| --- |  | 0 | 1 | 1 | 0 |
| --- |  | 0 | 1 | 1 | 1 |
| TIMP | If SC=0 then DC=DC else DC=SC | 1 | 0 | 0 | 0 |
| TAND | If SC=0 then DC=DC else DC=SC*DC | 1 | 0 | 0 | 1 |
| TOR | If SC=0 then DC=DC else DC=SC+DC | 1 | 0 | 1 | 0 |
| TEOR | If SC=0 then DC=DC else DC=!SC*DC +SC*!DC | 1 | 0 | 1 | 1 |
| TNOT | If SC=0 then DC=DC else DC=!SC | 1 | 1 | 0 | 0 |
| --- |  | 1 | 1 | 0 | 1 |
| --- |  | 1 | 1 | 1 | 0 |
| --- |  | 1 | 1 | 1 | 1 |

* SC = Source Color code

* DC = Destination Color code

* EOR = Exclusive OR

Logical Block Copy

Function	Copy a block of video from a source point to a destination point.  Since the data to be transferred is done in units of dots, logical operations may be done on the destination data.

Parameters	R0x = >0015

R1x = Row number of upper left corner of source

R2x = Column number of upper left corner of source

R3x = Row number of upper left corner of destination

R4x = Column number of upper left corner of destination

R5x = Number of rows

R6x = Number of columns

R7h = Logic operation to be performed on destination

Results	None

Parameter Description

Logic Operation

Summary of Logical Operations

| Name | Operation | A3 | A2 | A1 | A0 |
| --- | --- | --- | --- | --- | --- |
| IMP | DC = SC | 0 | 0 | 0 | 0 |
| AND | DC = SC*DC | 0 | 0 | 0 | 1 |
| OR | DC = SC+DC | 0 | 0 | 1 | 0 |
| EOR | DC = !SC*DC +SC*!DC | 0 | 0 | 1 | 1 |
| NOT | DC = !SC | 0 | 1 | 0 | 0 |
| --- |  | 0 | 1 | 0 | 1 |
| --- |  | 0 | 1 | 1 | 0 |
| --- |  | 0 | 1 | 1 | 1 |
| TIMP | If SC=0 then DC=DC else DC=SC | 1 | 0 | 0 | 0 |
| TAND | If SC=0 then DC=DC else DC=SC*DC | 1 | 0 | 0 | 1 |
| TOR | If SC=0 then DC=DC else DC=SC+DC | 1 | 0 | 1 | 0 |
| TEOR | If SC=0 then DC=DC else DC=!SC*DC +SC*!DC | 1 | 0 | 1 | 1 |
| TNOT | If SC=0 then DC=DC else DC=!SC | 1 | 1 | 0 | 0 |
| --- |  | 1 | 1 | 0 | 1 |
| --- |  | 1 | 1 | 1 | 0 |
| --- |  | 1 | 1 | 1 | 1 |

* SC = Source Color code

* DC = Destination Color code

* EOR = Exclusive OR

Block Scroll Up

Function	Scroll a block of video on the screen up.

Parameters	R0x = >0016

R1x = Number of pixels to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Pixel color for blank lines

Results	None

Parameter Description

Block Scroll Down

Function	Scroll a block of video on the screen down.

Parameters	R0x = >0017

R1x = Number of pixels to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Pixel color for blank lines

Results	None

Parameter Description

Block Scroll Left

Function	Scroll a block of video on the screen left.

Parameters	R0x = >0018

R1x = Number of pixels to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Pixel color for blank lines

Results	None

Parameter Description

Block Scroll Right

Function	Scroll a block of video on the screen right.

Parameters	R0x = >0019

R1x = Number of pixels to scroll

R2x = Row number of upper left corner

R3x = Column number of upper left corner

R4x = Row number of lower right corner

R5x = Column number of lower right corner

R6x = Pixel color for blank lines

Results	None

Parameter Description

Sprite Define

Function	Sprites are graphics that can be assigned any valid color and placed anywhere on the screen.  Sprites treat the screen as a grid 256 pixels high and 256 pixels wide.  However, only the first 192 pixels are visible on the screen.

You can create up to 32 sprites in all GRAPHICS modes except TEXT modes, which do not allow sprites.

Sprites can be set in motion in any direction at a variety of speeds.  A sprite continues its motion until it is specifically changed by the program.  Because sprites move from pixel to pixel, their motion can be smoother than that of characters, which can be moved only one character position (6 or 8 pixels) at a time.

Sprites “pass over” characters on the screen.  When two or more sprites are coincident (occupying the same screen pixel position), the sprite with the lowest sprite-number covers the other sprite(s).

Parameters	R0x = >001A

R1x = Pointer to sprite data

R2x = Number of sprites to define

Results	None

Parameter Description

Sprite Data Mode 1	1st word in list is Sprite # (Base 0)

2nd word in list is Character Code 0-255

3rd word in list is Position

4th word in list is X-Velocity

5th word in list is Y-Velocity

6th word in list is Color

Sprite Data Mode 2	1st word in list is Sprite # (Base 0)

2nd word in list is Character Code 0-255

3rd word in list is Position

4th word in list is X-Velocity

5th word in list is Y-Velocity

Next 16 words in list are colors

Character code	A numeric expression with a value from 0 to 255, specifying the character that defines the sprite pattern.  If you magnify the change the sprite’s size, sprite definition includes the character specified by the character code and three additional characters.

Delete Sprite

Function	The DELSPRITE subprogram enables you to delete one or more sprites.

Parameters	R0x = >001B

R1x = Pointer to list of sprite #’s

R2x = Number of sprites to delete (>FFFF for all)

Results	None

Parameter Description

Locate Sprite

Function	The LOCATE subprogram enables you to change the location of one or more sprites.  This subprogram can cause a sprite that has been deleted to reappear.

Parameters	R0x = >001C

R1x = Pointer to location data

R2x = Number of sprites to locate

Results	None

Parameter Description

Sprite Motion

Function	Place a sprite in motion.  The MOTION subprogram is used to specify the row-velocity and column-velocity of a sprite. If both the row- and column-velocities are zero, the sprite is stationary. A positive row-velocity moves the sprite down and a negative value moves it up. A positive column-velocity moves the sprite to the right and a negative value moves it to the left. If both row-velocity and column-velocity are nonzero, the sprite moves smoothly at an angle in a direction determined by the actual values.

When a moving sprite reaches an edge of the screen, it disappears.  The sprite reappears in the corresponding position at the opposite edge of the screen.

Parameters	R0x = >001D

R1x = Pointer to motion data

R2x = Number of sprites to put in motion

Results	None

Parameter Description

Sprite Color

Function	Define sprite color

Parameters	R0x = >001E

R1x = Pointer to color data

R2x = Number of sprites to color

Results	None

Parameter Description

Define Sprite Pattern

Function	Define sprite pattern.  The SPRITE subprogram creates sprites. Sprites are graphics which have a color and a location anywhere on the screen. They can be set in motion in any direction at a variety of speeds, and continue their motion until it is changed by the program or the program stops. They move more smoothly than the usual character which jumps from one screen position to another.

Parameters	R0x = >001F

R1x = Pointer to pattern # data.

R2x = Number of sprites to pattern

Results	None

Parameter Description

Sprite-number 	It is a numeric expression from 1 to 28. If the value is that of a sprite that is already defined, the old sprite is deleted and replaced by the new sprite. If the old sprite has a row- or column-velocity, and no new one is specified, the new sprite retains the old velocities.

Sprites pass over fixed characters on the screen. When two or more sprites are coincident, the sprite with the lowest sprite number covers the other sprites. While five or more sprites are on the same screen row, the one(s) with the highest sprite number(s) disappear.

Sprite Magnify

Function	Magnify a sprite’s size on the screen

Parameters	R0x = >0020

R1x =  Magnification factor (1-4, just like extended basic)

Results	None

Parameter Description

Magnification Factor 1	A magnification-factor of 1 causes all sprites to be single size and unmagnified. This means that each sprite is defined only by the character specified when the sprite was created and takes up just one character position on the screen.

Magnification Factor 2	A magnification-factor of 2 causes all sprites to be single size and magnified. This means that each sprite is defined only by the character specified when it was created, but takes up four character positions on the screen. Each dot position in the character specified expands to occupy four dot positions on the screen. The expansion from a magnification-factor of 1 is down and to the right.

Magnification Factor 3	A magnification-factor of 3 causes all sprites to be double size and unmagnified. This means that each sprite is defined by four character positions that include the character specified. The first character is the one specified when the sprite was created if its number is evenly divisible by four. or the next smallest number that is evenly divisible by four. That character is the upper left quarter of the sprite. The next character is the lower left quarter of the sprite. The next character is the upper right quarter of the sprite. The final character is the lower right quarter of the sprite. The character specified when the sprite was created is one of the four that makes up the sprite. The sprite occupies four character positions on the screen.

Magnification Factor 4	A magnification-factor of 4 causes all sprites to be double size and magnified. This means that each sprite is defined by four character positions that include the character specified. The first character is the one specified when the sprite was created if its number is evenly divisible by four, or the next smallest number that is evenly divisible by four. That character is the upper left quarter of the sprite. The next character is the lower left quarter of the sprite. The next character is the upper right quarter of the sprite. The final character is the lower right quarter of the sprite. The character specified when the sprite was created is one of the four that makes up the sprite. The sprite occupies sixteen character positions on the screen. The expansion from a magnification-factor of 3 is down and to the right.

Sprite Position and Sprite Distance

Function	Returns the square of the distance between two sprites.  The POSITION subprogram returns the position of the specified sprite(s) in the given dot-row(s) and dot-column(s) as numbers from 1 to 256.  If the square of the distance is greater than 32,767, the number returned is 32,767.

The distance between two sprites is considered to be the distance between the upper-left corners of the sprite.  The number returned is the square of the distance.

The distance between a sprite and a screen pixel is considered to be the distance between the upper-left corner of the sprite and the specified pixel.  The number returned is the square of the distance.

Parameters	R0x = >0021

R1x = Number of sprite to get position data

R2x = Type of Distance, 0 for none, 1 for Sprite, 2 for location

R3x = Number of second sprite (type 1), or Pixel row (type 2)

R4x = Pixel column (type 2)

Results	R0x = Returned row of sprite

R1x = Returned column of sprite

R2x = Distance (if second sprite number was given)

Parameter Description

Sprite Coincidence

Function	The COINCIDENCE subprogram enables you to ascertain if sprites are coincident (in conjunction) with each other or a specified screen pixel.

The exact conditions that constitute a coincidence vary depending on whether you are testing for the coincidence of two sprites, a sprite and a screen pixel, or all sprites.

If the sprites are moving very quickly, it may occasionally fail to detect a coincidence.

Two sprites are considered to be coincident if the upper-left corners of the sprites are within a specified number of pixels (coincidence checks) of each other. A coincidence exists if the distance between the pixels in the upper-left corners of the two sprites is less than or equal to the value of the coincidence checks.

A sprite is considered to be coincident with a screen pixel if the upper-left corner of the sprite is within a specified number of pixels (coincidence checks) of the screen pixel or if any pixel in the sprite occupies the screen pixel location.

Parameters	R0x = >0022

R1x = Type (0 = sprites, 1=locations, 2 = any two sprites)

R2x = Number of coincidence checks

R3x = Pointer to test field

R4x = Pointer to result field

Results	R0x = Number of coincidences detected

Parameter Description

Sprite Pattern Define or Get

Function	Define or Get Sprite pattern definitions

Parameters	R0x = >0023

R1x = CPU address of sprite pattern definitions

R2x = Number of sprite patterns to define or get

R3x = Starting pattern number

R4x = 0 if Define, >FFFF if Get

Results	None

Parameter Description

Sprites are created by turning some dots "on" and leaving others "off". The space character (ASCII code 32) is a character with all the dots turned "off". Turning all the dots "on" produces a solid block. The color of the on dots is the foreground color. The color of the off dots is the background color.

| BLOCKS |  |  |  | Binary Code 0=Off: 1=On | Hexadecimal Code |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0000 | 0 |
|  |  |  |  | 0001 | 1 |
|  |  |  |  | 0010 | 2 |
|  |  |  |  | 0011 | 3 |
|  |  |  |  | 0100 | 4 |
|  |  |  |  | 0101 | 5 |
|  |  |  |  | 0110 | 6 |
|  |  |  |  | 0111 | 7 |
|  |  |  |  | 1000 | 8 |
|  |  |  |  | 1001 | 9 |
|  |  |  |  | 1010 | A |
|  |  |  |  | 1011 | B |
|  |  |  |  | 1100 | C |
|  |  |  |  | 1101 | D |
|  |  |  |  | 1110 | E |
|  |  |  |  | 1111 | F |

Character Pattern Define or Get

Function	Define or get a character pattern definition. The CHAR subprogram allows you to define special graphics characters. You can redefine all 0 through 255 character definitions.

Parameters	R0x = >0024

R1x = CPU address of character pattern definitions

R2x = Number of patterns to define or get

R3x = Starting pattern number

R4x = 0 if Define, >FFFF if get

Results	None

Parameter Description

Characters are created by turning some dots "on" and leaving others "off". The space character (ASCII code 32) is a character with all the dots turned "off". Turning all the dots "on" produces a solid block. The color of the on dots is the foreground color. The color of the off dots is the background color.

| BLOCKS |  |  |  | Binary Code 0=Off: 1=On | Hexadecimal Code |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0000 | 0 |
|  |  |  |  | 0001 | 1 |
|  |  |  |  | 0010 | 2 |
|  |  |  |  | 0011 | 3 |
|  |  |  |  | 0100 | 4 |
|  |  |  |  | 0101 | 5 |
|  |  |  |  | 0110 | 6 |
|  |  |  |  | 0111 | 7 |
|  |  |  |  | 1000 | 8 |
|  |  |  |  | 1001 | 9 |
|  |  |  |  | 1010 | A |
|  |  |  |  | 1011 | B |
|  |  |  |  | 1100 | C |
|  |  |  |  | 1101 | D |
|  |  |  |  | 1110 | E |
|  |  |  |  | 1111 | F |

Set Text Window

Function	Define a text window on the screen enabling you to define screen margins.  The margins you specify define a screen window that affects the operation of several instructions.

The margins cannot “overlap”; that is, the position of the top margin must be higher on the screen than the bottom margin, and the position of the left margin must be farther left on the screen than the right margin.

The valid range for margin location varies according to the graphic mode you are in.  In all modes, the margins can extend to the edges of the screen.

Parameters	R0x = >0025

R1x = Top row

R2x = Left column

R3x = Bottom row

R4x = Right column

Results	None

Parameter Description

Get Text Window

Function	Get the text window screen definitions

Parameters	R0x = >0026

Results		R0x = Top row

R1x = Left column

R2x = Bottom row

R3x = Right column None

Parameter Description

Write TTY

Function	Write characters or strings to the screen.

Parameters	R0x = >0027

R1x = CPU address of string

R2x = Number of characters in string (0 = null terminated string)

Results	None

Parameter Description

Special Characters

>1B,>43,>2f,>2b	Control sequence to set color foreground (f) and background (b) colors.

>1B,>3D,>2r,>2c	Control sequence to set row (r) and column (c) position.

>0D			Carriage return

>01			Home cursor position

>08			Backspace

>09			Tab (8 characters)

>0A or >0B		Linefeed

>0C or >1A		Clear Screen

ASCII Codes	The following predefined characters may be printed or displayed on the screen.

| 30 | (cursor) | 63 | ? (question mark) |
| --- | --- | --- | --- |
| 31 | (edge character) | 64 | @ (at sign) |
| 32 | (space) | 65 | A |
| 33 | ! (exclamation point) | 66 | B |
| 34 | " (quote) | 67 | C |
| 35 | # (number or pound sign) | 68 | D |
| 36 | $ (dollar) | 69 | E |
| 37 | % (percent) | 70 | F |
| 38 | & (ampersand) | 71 | G |
| 39 | ‘ (apostrophe) | 72 | H |
| 40 | ( (open parenthesis) | 73 | I |
| 41 | ) (close parenthesis) | 74 | J |
| 42 | * (asterisk) | 75 | K |
| 43 | + (plus) | 76 | L |
| 44 | , (comma) | 77 | M |
| 45 | - (minus) | 78 | N |
| 46 | . (period) | 79 | O |
| 47 | / (slash) | 80 | P |
| 48 | 0 | 81 | Q |
| 49 | 1 | 82 | R |
| 50 | 2 | 83 | S |
| 51 | 3 | 84 | T |
| 52 | 4 | 85 | U |
| 53 | 5 | 86 | V |
| 54 | 6 | 87 | W |
| 55 | 7 | 88 | X |
| 56 | 8 | 89 | Y |
| 57 | 9 | 90 | Z |
| 58 | : (colon) | 91 | [ (open bracket) |
| 59 | ; (semicolon) | 92 | \ (reverse slash) |
| 60 | < (less than) | 93 | ] (close bracket) |
| 61 | = (equals) | 94 | ^ (exponentiation) |
| 62 | > (greater than) | 95 | _ (underline) |

| 96 | ` (accent grave) | 112 | p |
| --- | --- | --- | --- |
| 97 | a | 113 | q |
| 98 | b | 114 | r |
| 99 | c | 115 | s |
| 100 | d | 116 | t |
| 101 | e | 117 | u |
| 102 | f | 118 | v |
| 103 | g | 119 | w |
| 104 | h | 120 | x |
| 105 | i | 121 | y |
| 106 | j | 122 | z |
| 107 | k | 123 | { (left brace) |
| 108 | l | 124 | ¦ (vertical bar) |
| 109 | m | 125 | } (right brace) |
| 110 | n | 126 | ~ (tilde) |
| 111 | o | 127 | DEL (appears as blank) |

Restore Character or Sprite Pattern

Function	Restore Character or Sprite Pattern to it’s original definition.

Parameters	R0x = >0028

R1x = Flag (0 Sprite, <>0 Character)

Results	None

Parameter Description

Set Text Color

Function	The COLOR subprogram enables you to specify the colors of characters.  In general, each character has two colors.  The color of the pixels that make up the character itself is the foreground-color; the color of the pixels that occupy the rest of the character position on the screen is the background-color.  If a color is transparent, the color actually displayed is the color specified by the SCREEN subprogram (>000A).

Parameters	R0x = >0029

R1h = Foreground color for text

R1l  = Background color for text

Results	None

Parameter Description

Write Character String

Function	Write character string

Parameters	R0x = >002A

R1x = Address of string

R2x = Number of characters in string

R3x = 0 if change cursor position, >FFFF if leave cursor at beginning

Results	None

Parameter Description

Print Screen

Function	Print screen to defined printer.

Parameters	R0x = >002B

R1x = 0 for shades, 1 for outline

R2x = 0 for normal density (double), 1 for hi density (quad)

Results	None

Parameter Description

Horizontal Character Color

Function	Write characters to screen horizontally. The HCHAR subprogram displays a character anywhere on the display screen and optionally repeats it horizontally. The character with the ASCII value of character-code is placed in the position described by row and column and is repeated horizontally repetition times.

Parameters	R0x = >002C

R1x = Row

R2x = Column

R3x = ASCII character to write to screen

R4x = Number of times to write character and color

R5h = Foreground color for character

R5l  = Background color for character

Results	None

Parameter Description

None

Vertical Character Color

Function	Write characters to screen vertically. The VCHAR subprogram displays a character anywhere on the display screen and optionally repeats it vertically. The character with the ASCII value of character-code is placed in the position described by row and column and is repeated vertically repetition times.

Parameters	R0x = >002D

R1x = Row

R2x = Column

R3x = ASCII character to write to screen

R4x = Number of times to write character and color

R5h = Foreground color for character

R5l  = Background color for character

Results	None

Parameter Description

None

Horizontal Character

Function	Write characters to screen horizontally. The HCHAR subprogram displays a character anywhere on the display screen and optionally repeats it horizontally. The character with the ASCII value of character-code is placed in the position described by row and column and is repeated horizontally repetition times.

Parameters	R0x = >002E

R1x = Row

R2x = Column

R3x = ASCII character to write to screen

R4x = Number of times to write character and color

Results	None

Parameter Description

None

Vertical Character

Function	Write characters to screen vertically. The VCHAR subprogram displays a character anywhere on the display screen and optionally repeats it horizontally. The character with the ASCII value of character-code is placed in the position described by row and column and is repeated horizontally repetition times.

Parameters	R0x = >002F

R1x = Row

R2x = Column

R3x = ASCII character to write to screen

R4x = Number of times to write character and color

Results	None

Parameter Description

None

Set Mouse

Function	Set mouse speed and position

Parameters	R0x = >0030

R1x = New X position for mouse

R2x = New Y position for mouse

R3x = Scale factor for mouse speed (0 to 7) 0 = fastest

Results	None

Parameter Description

Get Mouse Speed

Function	Set mouse speed and position

Parameters	R0x = >0031

R1x = Returned X position for mouse

R2x = Returned Y position for mouse

R3x = b1 b2 b3 0 xxxx xxxx xxxx (highest bits)

b1 = left   	1 = down

b2 = middle	1 = down

b3 = right	1 = down

Results	None

Parameter Description

Get Mouse Release

Function	Get Mouse release data.

Parameters	R0x = >0032

Results		R1x = Returned X displacement since last call to opcode >31 or >32

R2x = Returned Y displacement since last call to opcode >31 or >32

R3x = b1 b2 b3 0 xxxx xxxx xxxx (highest bits)

b1 = left   	1 = down

b2 = middle	1 = down

b3 = right	1 = down

Parameter Description

Make Sound

Function	Make a sound through the sound generator chip on the Geneve 9640.

Parameters	R0x = >0033

R1x = Generator 1 frequency in Hz

R2x = Generator 2 frequency in Hz

R3x = Generator 3 frequency in Hz

R4h = Attenuation for Generator 1 (0 to 15)

R4l  = Attenuation for Generator 2 (0 to 15)

R5h = Attenuation for Generator 3 (0 to 15)

R6h = Control for noise generator:  bits = 0000 0xyz

x = 0 for periodic noise

x = 1 for white noise

yz = 00 = 6991 Hz

yz = 01 = 3496 Hz

yz = 10 = 1738 Hz

yz = 11 = Same Hz as Generator 3

R6l =  Attenuation for Noise Generator

R7x = Duration of noise in 60th seconds.

Results	None

Parameter Description

The SOUND subprogram tells the computer to produce tones or noise. The values given control three aspects of the sound: Duration; frequency; and volume.

| Value | Range | Description |
| --- | --- | --- |
| Duration | 1 to 4250 -1 to -4250 | The length of the sound in thousandths of a second |
| Frequency | (Tone) 110 to 44733 (Noise) -1 to -8 | What sound is played |

Duration is from .001 to 4.250 seconds, although it may vary up to 1/60th of a second. The computer continues performing program statements while a sound is being played. When you call the SOUND subprogram, the computer waits until the previous sound has been completed before performing the new CALL SOUND. However, if a negative duration is specified, the previous sound is stopped and the new one is begun immediately.

Frequency specifies the frequency of the note to be played with a value from 110 to 44733. (NOTE: This range goes higher than the range of human hearing. People vary in their ability to hear high notes, but generally the highest is approximately a value of 10000.) The actual frequency produced by the computer may vary up to 10 percent. The table below lists lists the frequencies of some musical tones.common notes.

Musical Tone Frequencies

The following table gives the frequencies (rounded to integers) of four octaves of the tempered scale (one half step between notes). While this list does not represent the entire range of tones that the computer can produce, it can be helpful for programming music.

| Frequency | Note | Frequency | Note |
| --- | --- | --- | --- |
| 110 | A | 440 | A (Above Middle C) |
| 117 | A# | 466 | A# |
| 123 | B | 494 | B |
| 131 | C | 523 | C (High C) |
| 139 | C# | 554 | C# |
| 147 | D | 587 | D |
| 156 | D# | 622 | D# |
| 165 | E | 659 | E |
| 175 | F | 698 | F |
| 185 | F# | 740 | F# |
| 196 | G | 784 | G |
| 208 | G# | 831 | G# |
| 220 | A | 880 | A (Above High C) |
| 233 | A# | 932 | A# |
| 247 | B | 988 | B |
| 262 | C (Middle C) | 1047 | C |
| 277 | C# | 1109 | C# |
| 294 | D | 1175 | D |
| 311 | D# | 1245 | D# |
| 330 | E | 1319 | E |
| 349 | F | 1397 | F |
| 370 | F# | 1480 | F# |
| 392 | G | 1568 | G |
| 415 | G# | 1661 | G# |
| 440 | A (Above Middle C) | 1760 | A |

Sound Status

Function	Determine status of the sound generator

Parameters	R0x = >0034

Results	EQ bit set if no sound is in progress.

Parameter Description

VWTR

Function	Video Write to Register command with register save.

Parameters	R0x = >0035

R1x = VDP Register #

R2l  = Value to put into VDP register

Results	None

Parameter Description

.

VRFR

Function	Video Read From Register, actually a read from stored values.

Parameters	R0x = >0036

R1x = VDP register #

Results	R0l  = Value read from register

Parameter Description

Get Tables

Function	Get Tables

Parameters	R0x = >0037

R1x = Pointer to user data to put copy of tables (24 bytes)

Results	None

Parameter Description

Tables			CTABLE	Data	0,0

PTABLE	Data	0,0

SCRIMG	Data	0,0

SPRATT	Data	0,0

SPRPAT	Data	0,0

SPRCOL	Data	0,0

Get Palette Registers

Function	Get Pallette Registers

Parameters	R0x = >0038

R1x = Pointer in user data, to put copy of Palette registers (32 bytes)

Results	None

Parameter Description