- GenRef

- v1.00

- MDOS Reference guide.

- DSR Library

- (C) Copyright 1989

J. Paul Charlton

ALL RIGHTS RESERVED

___________________________________________________________________________________

DSR - CONTENTS               ___________________________________________________________________________________

Page

Overview..........................................1

Available devices.................................1

Calling DSR functions.............................1

Open..............................................3

Close.............................................8

Read..............................................10

Write.............................................13

Restore...........................................16

Load..............................................18

Save..............................................20

Create Directory..................................23

Delete............................................25

Delete record.....................................27

Status............................................28

File ID Data Structure............................30

Bread.............................................32

Bwrite............................................35

Protect...........................................38

Rename............................................39

Format............................................42

___________________________________________________________________________________

DSR OVERVIEW

___________________________________________________________________________________

The MDOS device drivers  (Device  Service  Routines)  are

used  to  transfer  data from your program to a mass storage

device, from a mass storage device to your program;  and  to

transfer  data between your program and a character oriented

device like  a  terminal,  modem  or  printer.   The  device

drivers are also used to control device characteristics such

as baud rate, creating subdirectories, and initializing  new

media.

___________________________________________________________________________________

Available Devices               ___________________________________________________________________________________

DSK1 ... DSK9                 DSK1 through DSK9 are block oriented  devices

with a floppy disk directory structure.  Each

of these devices is limited to 3200  sectors.

The  assignment  of  these  names  to  actual

drives/ramdisks can vary  depending  on  your

hardware configuration.

HDS1 ... HDS3                 HDS1 through HDS3 are block oriented  devices

with  a  winchester disk directory structure.

See  your  HFDC  owner's  manual   for   more

information on these devices.

SCS1 ... SCS3                 SCS1 through SCS3 are block oriented  devices

with  a  winchester disk directory structure.

See  your  SCSI  owner's  manual   for   more

information on these devices.

RS232,                        RS232    through    RS232/4    are     serial

RS232/1, RS232/2              bidirectional asynchronous character devices,

RS232/3, RS232/4              both input and output is buffer with spoolers

whose size you configured with your AUUTOEXEC

file.

PIO,                          PIO through PIO/2 are parallel  bidirectional

PIO/1, PIO/2                  character  devices, usually used to send data

to a  printer.   Both  input  and  output  is

buffered  with  spoolers  whose  size was set

with your AUTOEXEC file.

WDS1 ... WDS4                 WDS1 through WDS4 are  used  to  address  the

Personality   Card  winchester  disk  system.

These devices do not  support  all  MDOS  DSR

operations.

___________________________________________________________________________________

CALLING DSR FUNCTIONS

___________________________________________________________________________________

The MDOS device drivers must  be  called  from  within  a

machine code program running as a task under MDOS.  You pass

arguments to the DSR using a small area of memory known as a

PAB (Peripheral Access Block.) The primary parts of a PAB are

DSR-2                            GenREF v1.00

an  opcode,  an error flag, and a full filename, other areas

of the PAB have meanings which  are  specific  to  the  file

operation you wish to perform.

The MDOS device drivers are invoked from a  machine  code

program  when  software  trap  number zero (XOP 0) is called

with a library number of 8.  The calling program's  R0  must

contain  the  16-bit  address  of the PAB at the time of the

XOP.  The calling program's R0 must be located between >A000

and >FFD8 in memory, and should be located in the PAD RAM at

>F000 if possible.  The following code fragment will  rename

a file on floppy drive #1.

LI        R0,PABADR

XOP       @EIGHT,0

MOVB      @PABADR+2,R0    check for error

JNE       ERROR

*

BL        @PRINT

TEXT      'File rename successful'

BYTE      >0D,>0A,0

*

* ...

*

NEWNAM    TEXT      'NEWFILE'

PABADR    BYTE      >0D       rename opcode

BYTE      >00

DATA      0

DATA      NEWNAM    assumes NEWNAM is mapped in

DATA      0,0,0,0

DATA      NAMLEN

NAME      TEXT      'DSK1.OLDFILE'

NAMLEN    EQU       $-NAME

*

In the preceding example, three hidden  assumptions  were

made.  First, it is assumed that the program's registers are

correctly located, with the Workspace  Pointer  register  of

the 9995 containing a value from >A000 to >FFD8.  Second, it

is assumed that  PABADR  is  located  on  a  page  which  is

currently  mapped  into a memory page which has the same 16-

bit address page number as its Virtual address  page  number

(read   the   section  on  Memory  Management.)   The  third

assumption is that NEWNAM is actually at the virtual address

NEWNAM, not in some overlay segment with a different virtual

address.

Always check these three assumptions in your own programs!!!

GenREF v1.00                             DSR-3

___________________________________________________________________________________

OPEN

___________________________________________________________________________________

Function                 For block devices, such as hard  disk  and  floppy

disk,  this  operation  must  be used to prepare a

structured file for access  with  READ  and  WRITE

opcodes.    You   must   close   the  file  before

terminating your task.  No other task in  MDOS  is

permitted  to open the file while your task has it

open.

For character devices, such as RS232 and PIO, this

operation  is  only  used  to change the operating

modes of the device port you specified as part  of

the   file   name.   Multiple  tasks  are  allowed

simultaneous  access  to   the   RS232   and   PIO

ports...user  beware.   You  do not need to open a

character device before reading and writing to it.

Pab format               Parameters passed to OPEN

byte offset   size      parameter

0             1 byte    opcode = >00

1             1 byte    mode flags

6             2 bytes   records to reserve for newly

created file

8             2 bytes   record length

15            1 byte    name length

16            string    file name

Parameters returned from OPEN

byte offset   size      parameter

2             1 byte    error code

8             2 bytes   your record length

12            2 bytes   actual record length

Parameter description

Opcode                   >00 is the opcode for the  OPEN  function  in  the DSR.

This opcode only applies to structured files  with

the  display  or internal attributes. You will get

an error if you attempt to open  a  non-structured

file in this manner.

DSR-4                                   GenREF v1.00

Flag byte

Bit       Meaning

0 (lsb)   0 =  Sequential  access   file,   you   must   use

sequential  access  for  files  with variable

record lengths.

1 =  relative access file, this can only  be  used

with fixed record length files.

2,1       00 = Update mode, can  only  be  used  with  fixed

record length files.  You are allowed to READ

and WRITE records to the file.

01 = Output mode, this is used  to  create  a  new

file.   You are only allowed to WRITE records

to this file.  If the file already existed on

disk,  the  old  contents of the file will be

forgotten.  You will get an error if there is

already  a  Protected  file with the name you

specified.

10 = Input mode, this is used prepare for  reading

from an existing file.  You will get an error

if the file doesn't already exist.

11 = Append mode, this  is  used  to  prepare  for

writing  to  the  end of an existing file, or

for creating a new file.   You  will  get  an

error  if  you  try  to use this with a fixed

record length file.

3         0 =  Use display format data

1 =  Use internal format data

This bit has no effect on the  data  actually

stored  in  the  file.  It is provided by the

programmer as an indication of  the  type  of

data  in  the file.  You will get an error if

this flag does not match the attributes of  a

file  you  are  opening for update, input, or

append modes.

4         0=   File will use variable record lengths

1 =  File will use fixed record lengths

This flag will affect how data is  stored  in

the  file,  and  whether you can use relative

record access within the file.

5,6       00   RESERVED, set to ZERO.

7         0    IGNORED

1    For RS232 and PIO devices only, setting  this

bit  to  one  will cause the driver to change

the mode of the  port  as  specified  in  the

switches  you placed after the device name in

the filename.  The mode will be set after all

data  currently in the output spooler for the

specified device has been processed.

GenREF v1.00                           DSR-5

Error code

>00            No error occurred, the file is open and ready

for reads and writes.

>20            Write Protection violation.  A file you  were

trying  to open for UPDATE, APPEND, or OUTPUT

could not be created because the floppy  disk

has  a  write-protect tab.  Alternatively, an

existing file which you were trying  to  open

for   UPDATE,   APPEND,  or  OUTPUT  has  the

"protected"  attribute   bit   set   in   its

directory entry.

>40            Invalid attributes.  You specified  a  record

length  which  was  different than that of an

existing file.  Or, you tried to open a fixed

file  in APPEND mode.  Or, the attributes you

specified in the mode byte do not agree  with

the  file  attributes  of an existing file on

the disk.

>80            Out of space.   There  are  not  enough  free

sectors  on the device to create the file you

specified;   or,   the   directory   on   the

specified  device  already has the maximum of

127 entries; or, there  are  too  many  files

open in MDOS, by your task and other tasks.

>C0            Media   error.    For   some   reason,   MDOS

encountered   an   unrecoverable  error  when

trying to access the specified  device.   For

floppy  disks, this could mean that the drive

is empty.  For hard disks,  this  means  that

there  is no device present, or that MDOS was

unable to read a needed sector from the  hard

drive.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if a file  you  specified

for  INPUT mode does not yet exist.  You will

also  get  this  error  if  any  subdirectory

specified  as  part  of the filename does not

exist on the specified device.

Reserved Records              If this parameter is set to zero  during  the

OPEN  call,  no  space will be reserved for a

newly created file.  You can reserve  sectors

for  a file when you open the file by setting

this parameter to the number of  records  you

expect  to  put  into  a  newly created file.

Reserving space  during  the  OPEN  call  can

greatly  decrease  the  time  needed to write

records to a new file since MDOS doesn't have

to spend time allocating more sectors for the

file each time you want to write more records

to  the  file.  For files with record lengths

less  than  255  characters,  the  number  of

sectors   reserved  is  calculated  with  the

following  formula:    Sectors   =   Records/

INT(256/Record_Size).

DSR-6                                   GenREF v1.00

For files with record lengths longer than 255

characters, the number of sectors reserved is

calculated  with  the  formula:   Sectors   =

INT((Records*Record_Size)/256).  Note that if

you specified a  record  length  of  ZERO,  a

record  length  of  80  will be used by MDOS.

For variable files with record  lengths  less

than  255,  the  record  size  is  the record

length plus one.  For fixed files, the record

size  is  the same as the record length.  For

variable files with records  lengths  greater

than  255,  the  record  size  is  the record

length plus two.

Record Length            This parameter specifies the  default  record

length  for  a file.  If you are creating the

file, and this parameter has  a  zero  value,

MDOS   will   use   a  record  length  of  80

characters.   If  the  file  already  exists,

passing  a zero value for this parameter will

cause MDOS to use the record  length  of  the

existing  file.   If the file already exists,

and you specify a  non-zero  value  for  this

parameter  which is different from the record

length of the file, you will  get  an  error.

Specifying  a zero value will always open any

structured file.

On return, this will be the same as  you  set

it,  unless  you  initially  specified a zero

length.  If you specified a zero length,  and

the  file already existed, this will have the

true record  length  of  the  file.   If  you

specified   a   zero  length,  and  you  were

creating a new file, this will have  a  value

of 80.

Actual Length            On return, if the file  you  were  trying  to

access already existed, this will contain the

record length of the file.  This is  returned

even  if  you  get an error for specifying an

incorrect record  length  for  a  file  which

already  exists.   If  you  just  created the

file, this will contain your record length.

Filename length          This is a count of the number  of  characters

in the filename string.

Filename string          For block devices, such  as  disks  and  hard

disks,  this  string must contain the name of

the device you wish to access, followed by  a

list   of  the  subdirectories  separated  by

periods, followed by the name of the file you

wish        to        access.       (Example:

"HDS1.SOURCE.UTIL.EXAMPLE")   The  length  of

the  name,  including  the period separators,

GenREF v1.00                       DSR-7

Must be limited to 40 characters. For the OPEN call,

MDOS  will  not  create directories as specified in

your pathname.  If the specified directories did not

exist, you will get an error.  Floppy disks are currently

limited to one subdirectory level.  The individual names

for the subdirectories and your filename are limited to

ten characters, you will get an error if you try to use

more  than  ten characters.

For character devices, such as RS232 and PIO, this string

must contain the name of the device you wish to set for,

followed by a list of switches you wish to set separated

by periods. (Example: "RS232/2.BA=9600.DA=8.PA=N.CR")

Note that the switches are set  only  if  you  set  bit

seven of the flag byte to a one. The switches will not

take affect until all output previously loaded into the

spooler for the specified device has been processed.

PIO                      The following switch extensions may  be  used

with the PIO ports.

CR        Turn  off  carriage  returns  and  line-feeds

after each variable record sent.

LF        Turn  off  line-feeds  after  each   variable

record  sent,  each  variable record is still

followed by a carriage return.

NU        Print nulls after  each  variable  record  to

allow for a low slew-rate printer.

IB        Reconfigure  the  spooler  to   recognize   a

printer   with   an  Inverted-Busy  handshake

signal.  (If your  printer  doesn't  seem  to

work with MDOS, try turning this switch on.)

HS        Reconfigure the spooler  to  perform  a  full

handshake with the printer for each byte sent

(instead of just strobe.)  (If  your  printer

doesn't  seem  to work with MDOS, try turning

this switch on.)

RS232                    The following switch extensions may  be  used

with the RS232 ports.

CR        Turn  off  carriage  returns  and  line-feeds

after each variable record sent.

LF        Turn  off  line-feeds  after  each   variable

record  sent,  each  variable record is still

followed by a carriage return.

NU        Print nulls after  each  variable  record  to

allow for a low slew-rate printer.

BA=baudrate    (110,300,600,1200,2400,4800,9600,19200)

DA=databits    (7,8)

PA=parity      (O,E,N) for (Odd,Even,None), respectively.

TW        Use two stopbits on transmission  instead  of

one stopbit.  (If your device doesn't seem to

work with MDOS, try turning this switch on.)

CH        Check parity on each input character.

DSR-8                                       GenREF v1.00

___________________________________________________________________________________

CLOSE               ___________________________________________________________________________________

FUNCTION                      This operation must be performed to close  an

open  file.   It will cause MDOS to write any

modified file  buffers  associated  with  the

file; and if the file was modified, MDOS will

write out a new copy of the file's  directory

entry.

You must perform this operation on every file

you  have open before causing your program to

terminate.

This operation has no effect on  a  character

devices such as RS232 and PIO.

PAB format                    Parameters passed to CLOSE

byte offset   size      parameter

0             1 byte    opcode = >01

15            1 byte    name length

16            string    file name

Parameters returned from CLOSE

byte offset   size      parameter

2             1 byte    error code

GenREF v1.00                           DSR-9

Parameter description

Opcode         >01 is the opcode for the CLOSE  function  in

the DSR.

Error code

>00            No error occurred, the file is now closed, or

the file wasn't even open.

>20            Write Protection violation.  MDOS was  unable

to  flush  the  file's buffers to disk.  This

should  only  happen  if  the  user  switched

floppy disks while the file was open.

>C0            Media   error.    For   some   reason,   MDOS

encountered   an   unrecoverable  error  when

trying to flush the buffers for the specified

file.  For floppy disks, this could mean that

the user prematurely removed  the  disk  from

the drive.  Alternatively, MDOS was unable to

locate  the  place  on  the  disk  where  the

flushed data was to be written.

>E0            General purpose error.  An error which didn't

fit any of the previous descriptions.

GenREF v1.00                            DSR-10

___________________________________________________________________________________

READ               ___________________________________________________________________________________

Function                      This operation is used to transfer data  from

the  specified  device/file  to  a buffer you

specify.  For block devices  such  as  floppy

and hard disk, the file must already be open.

You can read from a character device such  as

RS232 at any time, no OPEN operation needs to

be performed.

Reading from a character device  is  designed

to  accept  input  from  a  user  typing at a

terminal.  For variable record length  files,

MDOS    will    interpret   certain   control

characters (described later in this  section)

input from the device as editing commands for

the   current   input   line,   and   display

appropriate changes on the user's terminal.

PAB format                    Parameters passed to READ

byte offset   size      parameter

0             1 byte    opcode = >02

3             3 bytes   buffer address

6             2 bytes   record number

8             2 bytes   record length

10            1 byte    CPU/VDP flag

15            1 byte    name length

16            string    file name

Parameters returned from READ

byte offset   size      parameter

2             1 byte    error code

6             2 bytes   next record number

11            1 byte    ZERO

12            2 bytes   character count

DSR-11                                 GenREF v1.00

Parameter description

Opcode         >02 is the opcode for the  READ  function  in

the DSR.

Error code

>00            No error occurred, the read was successful.

>A0            You attempted to read past  the  end  of  the

file's  previous  contents.  Your buffer does

not contain valid data.

>C0            For block devices,  media  error.   For  some

reason,  MDOS  encountered  an  unrecoverable

error  when  trying  read   data   from   the

specified  file. For floppy disks, this could

mean that the user  prematurely  removed  the

disk from the drive.  Alternatively, MDOS was

unable to locate the place on the disk  where

the data was to be read from.

For RS232  character  devices,  the  hardware

detected  an  error such as incorrect parity,

byte too long, character buffer  overflow  (a

character was lost.)

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if the file you specified

is not currently open.

Buffer Address This is where  you  specify  the  address  to

which data is to be transferred.

For transfers to VDP RAM, only the lowest  17

bits of the 3 bytes are significant.

For transfers to CPU RAM, only the lowest  21

bits of the 3 bytes are significant.  For CPU

RAM transfers, the  lowest  13  bits  are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU  address  your  task  will use to

examine the data, depending on how your  task

has  altered  the map of its execution memory

pages (see the section on Memory Management.)

Record Number  The record number is  only  valid  for  Fixed

record  files.   You will get an error if you

set the record number higher than the highest

record  ever  written  to  the  file.   It is

possible to read a buffer full of garbage  if

you  specify  a record number which has never

been written to.

GenREF v1.00                  DSR-12

On  return,  the  record  number   has   been

Incremented by one from the value you passed.

Note  that no  distinction  is made between

Relative and Sequential  access  for  Fixed  files.

All access   to   fixed   files   is  treated  as

sequential  unless  you  change  the   record

number within your program.

Record Length    For block devices, your  program  should  not

alter this value.

For character devices, set this value to  the

length of your input buffer.

CPU/VDP Flag     If  this  byte  is   zero,   data   will   be

transferred   to   a  buffer  in  the  memory

belonging  to  your  task,  at  the   address

specified  in  the  buffer  address.  If this

byte is non-zero, data will be transferred to

VDP RAM.

Character Count  On  return,  this  contains  the  number   of

characters  placed  into  your  input buffer.

For fixed files on  block  devices,  this  is

always  the same as the record length for the

file.  If you were reading fixed records from

a character device, this value may be smaller

than  the  record  length  if  no  input  was

available.

NOTE:            For variable record  length  files  on  block

devices,  this  count  can actually be larger

than  the  specified   record   length,   for

compatibility with existing applications like

TI-Extended Basic. (The  notable  example  is

I/V  163  Merge  format files, which can have

records longer than 163 characters.)

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For block devices, such  as  disks  and  hard

disks, this string must contain the name of a

file which you currently  have  opened.   For

character devices, this is simply the name of

the device, all switches after the  name  are

ignored.

EDIT Characters  Editing  characters  available  on  character

input devices.

ENTER     code >0D  terminates input

ECHO      code >12  redisplays current input line

DEL       code >7F  deletes last character in line

DSR-13                               GenREF v1.00

___________________________________________________________________________________

WRITE          ___________________________________________________________________________________

Function                 This operation is used to transfer data  from

a   buffer   you  specify  to  the  specified

device/file.   For  block  devices  such   as

floppy  and  hard disk, the file must already

be open.

You can write to a character device  such  as

RS232  or  PIO at any time, no OPEN operation

needs to be performed.

Writing to  a  character  device  with  fixed

record   lengths   causes   the   number   of

characters you specified to be written to the

device,   no  end  of  line  markers  (except

possibly  NULLS)  are  added  to  the   data.

Writing  to a character device with  variable

record   lengths   causes   the   number   of

characters you specified to be written to the

device,  possibly  followed  by  a   carriage

return and a linefeed, just a linefeed, or no

extra characters, depending on  the  switches

you  set  with  an  OPEN  call.  NULLS may be

added to your data if  you  turned  NULLS  on

with an OPEN call.

PAB format               Parameters passed to WRITE

byte offset   size      parameter

0             1 byte    opcode = >03

3             3 bytes   buffer address

6             2 bytes   record number

8             2 bytes   record length

10            1 byte    CPU/VDP flag

15            1 byte    name length

16            string    file name

Parameters returned from WRITE

byte offset   size      parameter

2             1 byte    error code

6             2 bytes   next record number

GenREF v1.00                        DSR-14

Parameter description

Opcode         >03 is the opcode for the WRITE  function  in

the DSR.

Error code

>00            No error occurred, the write was successful.

>20            Write protection.  MDOS was unable  to  write

data  to the file.  If the file was opened in

OUTPUT mode, this means  that  the  user  has

switched disks since the file was opened, and

that the new disk is write-protected.  For  a

file  opened  in  APPEND or UPDATE mode, this

probably  means  that  the  disk  was  write-

protected from the outset.

>80            Out of space.  The disk is full.

>C0            For block devices,  media  error.   For  some

reason,  MDOS  encountered  an  unrecoverable

error  when  trying  read  the  sector  usage

bitmap  information  or  trying to write data

into the file itself.  For floppy disks, this

could  mean that the user prematurely removed

the disk from the drive.  Alternatively, MDOS

was  unable  to  locate the place on the disk

where the data was to be written.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if the file you specified

is not currently open.

Buffer Address This is where you specify  the  address  from

which data is to be transferred.

For transfers from VDP RAM, only  the  lowest

17 bits of the 3 bytes are significant.

For transfers from CPU RAM, only  the  lowest

21  bits of the 3 bytes are significant.  For

CPU RAM transfers, the lowest 13 bits are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU  address  your  task  will use to

initialize the data, depending  on  how  your

task  has  altered  the  map of its execution

memory  pages  (see  the  section  on  Memory

Management.)

Record Number  The record number is  only  valid  for  Fixed

record  files.   Writing to a record past the

end of the current file contents  will  cause

the  file  to be expanded, possibly causing a

disk full error.

DSR-15                                  GenREF v1.00

On  return,  the  record  number   has   been

incremented by one from the value you passed.

Note that  no  distinction  is  made  between

Relative  and  Sequential  access  for  Fixed

files.  All access to fixed files is  treated

as  sequential  unless  you change the record

number within your program.

Record Length    For block devices, your  program  should  not

alter this value.

For character devices, set this value to  the

length of your output buffer.

CPU/VDP Flag     If  this  byte  is   zero,   data   will   be

transferred  from  a  buffer  in  the  memory

belonging  to  your  task,  at  the   address

specified  in  the  buffer  address.  If this

byte is non-zero, data  will  be  transferred

from VDP RAM.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For block devices, such  as  disks  and  hard

disks, this string must contain the name of a

file which you currently  have  opened.   For

character devices, this is simply the name of

the device, all switches after the  name  are

ignored.

GenREF v1.00                            DSR-16

___________________________________________________________________________________

RESTORE               ___________________________________________________________________________________

Function                      For block devices,  such  as  hard  disk  and

floppy  disk, this resets the file so the the

next record read will come from the beginning

of  the file, or the next record written will

go at the beginning of the  file.   The  only

exception  is  fixed  record  files  with the

relative  mode  on,  those  files   will   be

restored to the record number you specify.

This is the only opcode which makes a distinction

Between sequential and relative access fixed files.

PAB format                    Parameters passed to RESTORE.

byte offset   size      parameter

0             1 byte    opcode = >04

1             1 byte    mode flags

6             2 bytes   record number

15            1 byte    name length

16            string    file name

Parameters returned from RESTORE

byte offset   size      parameter

2             1 byte    error code

6             2 bytes   record number

Parameter description

Opcode         >04 is the opcode for the RESTORE function

This opcode must be used on a file which  was

opened  with  the  OPEN  opcode.  It does not

apply to character devices such as RS232  and

PIO.

Flag byte      Only the least significant bit has meaning to

the restore opcode, and only for fixed files.

If the least significant bit is set  to  one,

for  a  fixed  record  file, the file pointer

will be moved to the record you specified  in

the   record  number.   Otherwise,  the  file

pointer will be moved to  the  start  of  the

file.

DSR-17                                 GenREF v1.00

Error code

>60            Bad opcode. You attempted to use the  RESTORE

operation on a character device.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if the file you specified

is not currently open.

Record number  For fixed files with the relative access mode

bit  set,  this  is  the  record  number  for

subsequent  access  to  the  file.   This  is

pretty  useless,  because  you can always set

the record number on any read or write.

Upon   return,   this   contains   zero   for

sequential access fixed files, and the record

number  you  specified  for  relative  access

fixed files.

Filename length

This is a count of the number  of  characters

in the filename string.

Filename string

For block devices, such  as  disks  and  hard

disks, this string must contain the name of a

file which you currently have opened.

GenREF v1.00                            DSR-18

___________________________________________________________________________________

LOAD               ___________________________________________________________________________________

Function                      For block devices,  such  as  hard  disk  and

floppy disk, this operation is used to read a

program image file into memory at the  buffer

address you specified.

For character devices, this operation is  not

implemented, but there was some thought about

using the 1K  XMODEM  protocol  to  implement

this operation.

PAB format                    Parameters passed to LOAD

byte offset   size      parameter

0             1 byte    opcode = >05

3             3 bytes   buffer address

10            1 byte    CPU/VDP flag

11            3 bytes   buffer size

15            1 byte    name length

16            string    file name

Parameters returned from LOAD

byte offset   size      parameter

2             1 byte    error code

6             1 byte    >00

7             3 bytes   image size

Parameter description

Opcode         >05 is the opcode for the  LOAD  function  in

the DSR.

This opcode can only be used to  transfer  an

entire  program  image  from a block oriented

storage device to memory.  You  will  get  an

error  if  you  try to load any other type of

file.

Error code

>40            Bad attributes.  You attempted to load a file

which wasn't a program image.

>60            Bad opcode.  You attempted  to  load  from  a

character  oriented  device  such as RS232 or

PIO.

>80            Buffer overrun.  The image file  on  disk  is

larger  than  the  maximum  buffer  size  you

specified.

>C0            Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to locate or read from  the  specified

file.  This could mean that a floppy drive is

empty, or there is a bad sector on the floppy drive.

DSR-19                                    GenREF v1.00

>E0              General purpose error. An error which didn't fit

any of the previous descriptions. You will get this

error if the file does not exist on the specified device.

Buffer Address   This is where  you  specify  the  address  to which

data is to be transferred.

For transfers to VDP RAM, only the lowest 17 bits

of the 3 bytes are significant.

For transfers to CPU RAM, only the lowest 21 bits

of the 3 bytes are significant. For CPU RAM transfers,

the lowest 13 bits are an offset into one of your

task's memory pages, and the other 8 bits specify

which of your task's pages the transfer starts on.

This address is not necessarily the  same  as  the

16-bit CPU address your task will use to examine the

data, depending on how your task has altered the map

of its execution memory pages (see the section on

Memory Management.)

Image Size       These three bytes give the actual size of the image

file on disk. This value is returned to you even if the

file was not loaded because it was larger than your

buffer.  Note that image files loaded from the WDSx

personality  card  winchester  are limited to 16384 -

(buffer_address MOD  8192) bytes in length.

CPU/VDP Flag     If  this byte is zero, data will be transferred to a

buffer  in  the  memory belonging to your task, at

the address specified in the buffer address.  If this

byte is non-zero, data will be transferred to VDP RAM.

Buffer Size      You use these three bytes to inform  MDOS  of

the  maximum number of characters you wish to

load as an image file.  If the image file  is

longer  than  the size you specify here, none

of the image file will  be  loaded,  and  you

will get an error.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For block devices, such  as  disks  and  hard

disks,  this  string must contain the name of

the device you wish to access, followed by  a

list   of  the  subdirectories  separated  by

periods, followed by the name of the file you

wish to access. (Example: "HDS1.SOURCE.UTIL.EXAMPLE").

The length of the name, including the  period

separators, must be limited to 40 characters.

This file must already exist  on  the  device

you are accessing.

GenREF v1.00                            DSR-20

___________________________________________________________________________________

SAVE          ___________________________________________________________________________________

Function                 For block devices,  such  as  hard  disk  and

floppy  disk, this operation is used to write

a program image file to disk from  memory  at

the  buffer  address  you  specified.  If the

file doesn't already exist, a new  file  will

be  created if there is room on the disk.  If

the  file  already  exists,  it  must  be  an

unprotected  program  image file, or you will

get an error.  When you are saving  an  image

file,  any  subdirectories  specified  in the

filename must already exist, they will not be

created  for  you.   You will get an error if

the specified subdirectories do  not  already

exist.

For character devices, this operation is  not

implemented, but there was some thought about

using the 1K  XMODEM  protocol  to  implement

this operation.

PAB format               Parameters passed to SAVE for image files.

byte offset   size      parameter

0             1 byte    opcode = >06

3             3 bytes   buffer address

10            1 byte    CPU/VDP flag

11            3 bytes   buffer size

15            1 byte    name length

16            string    file name

Parameters returned from SAVE.

byte offset   size      parameter

2             1 byte    error code

Parameter description

Opcode         >06 is the opcode for the  SAVE  function  in

the DSR.

This opcode can be used to transfer an entire

program image from memory to a block oriented

storage device.

GenREF v1.00                       DSR-21

Error code

>20            Write Protection violation.  The  file  could

not  be created because the floppy disk has a

write-protect tab.  Alternatively,  the  file

already   exists,  and  has  the  "protected"

attribute set in its directory entry.

>40            Bad attributes.  You attempted to  save  over

an  existing  file  which  wasn't  a  program

image.

>60            Bad opcode.  You attempted to save  an  image

on  a character oriented device such as RS232

or PIO.

>80            Out of space.   There  are  not  enough  free

sectors  on the device to create the file you

specified;   or,   the   directory   on   the

specified  device  already has the maximum of

127 entries.

>C0            For block devices,  media  error.   For  some

reason,  MDOS  encountered  an  unrecoverable

error  when  trying  read  the  sector  usage

bitmap  information  or  trying to write data

into the file itself.  For floppy disks, this

could  mean that the user prematurely removed

the disk from the drive.  Alternatively, MDOS

was  unable  to  locate the place on the disk

where the data was to be written.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will  get  this  error  if  any  subdirectory

specified  as  part  of the filename does not

exist on the specified device.

Buffer Address This is where you specify  the  address  from

which data is to be transferred.

For transfers from VDP RAM, only  the  lowest

17 bits of the 3 bytes are significant.

For transfers from CPU RAM, only  the  lowest

21  bits of the 3 bytes are significant.  For

CPU RAM transfers, the lowest 13 bits are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU  address  your  task  will use to

initialize the data, depending  on  how  your

task  has  altered  the  map of its execution

memory  pages  (see  the  section  on  Memory

Management.)

DSR-22                                  GenREF v1.00

CPU/VDP Flag     If  this  byte  is   zero,   data   will   be

transferred  from  a  buffer  in  the  memory

belonging  to  your  task,  at  the   address

specified  in  the  buffer  address.  If this

byte is non-zero, data  will  be  transferred

from VDP RAM.

Buffer Size      You use these three bytes to inform  MDOS  of

the  number of characters you wish to save as

an image file.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For block devices, such  as  disks  and  hard

disks,  this  string must contain the name of

the device you wish to access, followed by  a

list   of  the  subdirectories  separated  by

periods, followed by the name of the file you

wish      to     save     into.     (Example:

"HDS1.SOURCE.UTIL.EXAMPLE")   The  length  of

the  name,  including  the period separators,

must be limited to 40 characters.

GenREF v1.00                            DSR-25

___________________________________________________________________________________

CREATE DIRECTORY               ___________________________________________________________________________________

Function                      For block devices, this operation is used  to

create  a  new  subdirectory on the specified

device.  You can create a new subdirectory by

using  a  filename  which  ends  in  a period

separator   character.    Any    intermediate

subdirectories  specified  in  your  filename

must already exist, they will not be  created

for       you.        A      filename      of

"HDS1.LEVEL1.LEVEL2.LEVEL3." would  create  a

subdirectory    named    "LEVEL3"   only   if

subdirectories "LEVEL1" and "LEVEL2"  already

existed  on "HDS1".  Note that only one level

of    subdirectories,    containing     three

subdirectories,  is  allowed on a floppy disk

device.

For character devices, this operation is  not

implemented.

PAB format                    Parameters passed to CREATE DIRECTORY

byte offset   size      parameter

0             1 byte    opcode = >06

15            1 byte    name length

16            string    file name

Parameters returned from CREATE DIRECTORY

byte offset   size      parameter

2             1 byte    error code

6             1 byte    >00

Parameter description

Opcode         >06 is the opcode for  the  CREATE  DIRECTORY

function in the DSR.  This is the same opcode

as the SAVE function  of  the  DSR,  the  DSR

distinguishes between the two uses by looking

for a period character  at  the  end  of  the

filename you specified.

DSR-26                                   GenREF v1.00

Error code

>20              Write Protection violation.  The subdirectory

could  not  be  created because a floppy disk

has a write-protect tab.

>60              Bad  opcode.   You  attempted  to  create   a

subdirectory  on  a character oriented device

such as RS232 or PIO.

>80              Out of space.   There  are  not  enough  free

sectors   on   the   device   to  create  the

subdirectory   you   specified;    or,    the

directory on the specified device already has

the maximum of 114 subdirectory entries.

>C0              For block devices,  media  error.   For  some

reason,  MDOS  encountered  an  unrecoverable

error  when  trying  read  the  sector  usage

bitmap  information  or  trying to write data

into the  subdirectory  itself.   For  floppy

disks,   this   could   mean  that  the  user

prematurely removed the disk from the  drive.

Alternatively,  MDOS was unable to locate the

place on the disk where the data  was  to  be

written.

>E0              General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will  get  this  error  if  any  subdirectory

specified as part of the filename (other than

the last one in the  filename,  the  one  you

wish   to  create)  does  not  exist  on  the

specified device.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For block devices, such  as  disks  and  hard

disks,  this  string must contain the name of

the device you wish to access, followed by  a

list   of  the  subdirectories  separated  by

periods,  followed  by  the   name   of   the

subdirectory  you wish to create, followed by

another          period.            (Example:

"HDS1.SOURCE.UTIL.NEWDIR.")   The  length  of

the name, including  the  period  separators,

must be limited to 40 characters.

GenREF v1.00                             DSR-25

___________________________________________________________________________________

DELETE

___________________________________________________________________________________

Function                      This operation serves three purposes.

For block devices,  such  as  hard  disk  and

floppy  disk,  this  can  be used to delete a

file  from  a  directory,  returning  sectors

allocated to the file into available space on

the disk.

On block devices, this operation can also  be

used  to remove a subdirectory from a disk if

the subdirectory is empty.

For character devices, this operation can  be

used  to  flush  the  output  spooler for the

device, causing all characters still  in  the

output spooler to be purged.

PAB format                    Parameters passed to DELETE.

byte offset   size      parameter

0             1 byte    opcode = >07

15            1 byte    name length

16            string    file name

Parameters returned from DELETE

byte offset   size      parameter

2             1 byte    error code

Parameter description

Opcode         >07 is the opcode for the DELETE function.

Error code

>20            Write Protection violation.  For  files,  the

specified  file  could not be deleted because

the disk is  physically  write-protected,  or

the "protected" mode bit is set in the file's

directory  entry.   For  subdirectories,  the

directory  could  not  be deleted because the

disk is physically write  protected,  or  the

directory  still  has  subordinate  files and

directories within it (the directory  is  not

empty.)

DSR-26                                  GenREF v1.00

>C0              For block devices,  media  error.   For  some

reason,  MDOS  encountered  an  unrecoverable

error when trying to update the sector  usage

bitmap  information  or  trying to write data

into the  subdirectory  itself.   For  floppy

disks,   this   could   mean  that  the  user

prematurely removed the disk from the  drive.

Alternatively,  MDOS was unable to locate the

place on the disk where the data  was  to  be

written.

>E0              General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will  get  this  error  if  any  subdirectory

specified as part of the filename (other than

the last one in the  filename,  the  one  you

wish   to  create)  does  not  exist  on  the

specified device.  You  will  also  get  this

error  if the specified file/subdirectory did

not exist.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  To delete files on  block  devices,  such  as

disks   and  hard  disks,  this  string  must

contain the name of the device  you  wish  to

access,   followed   by   a   list   of   the

subdirectories separated by periods, followed

by  the  name of the file you wish to delete.

(Example:  "HDS1.SOURCE.UTIL.EXAMPLE")    The

length  of  the  name,  including  the period

separators, must be limited to 40 characters.

To delete a subdirectory from a block device,

such  as  disks  and  hard disks, this string

must contain the name of the device you  wish

to   access,   followed  by  a  list  of  the

subdirectories separated by periods, followed

by  the  name of the subdirectory you wish to

create,   followed   by    another    period.

(Example:    "HDS1.SOURCE.UTIL.NEWDIR.")  The

length of  the  name,  including  the  period

separators, must be limited to 40 characters.

To purge the output spooler  on  a  character

device such as RS232 or PIO, this string must

contain the name of the device.  Any switches

and  other  characters  following  the device

name will be ignored.

GenREF v1.00                             DSR-27

___________________________________________________________________________________

DELETE RECORD

___________________________________________________________________________________

Function                      This operation is not  currently  implemented

in MDOS.  It would be used to delete a record

from a key indexed file (possibly implemented

as a B-Tree.)

PAB format                    Parameters passed to DELETE RECORD.

byte offset   size      parameter

0             1 byte    opcode = >08

15            1 byte    name length

16            string    file name

Parameters returned from DELETE RECORD

byte offset   size      parameter

2             1 byte    error code = >60

Parameter description

Opcode         >08 is  the  opcode  for  the  DELETE  RECORD

function.

Error code

>60            Bad opcode.  This opcode is  not  implemented

in MDOS.

Filename length

This is a count of the number  of  characters

in the filename string.

Filename string

This would be the name of  a  currently  open

file on a block device.

DSR-30                             GenREF v1.00

___________________________________________________________________________________

STATUS          ___________________________________________________________________________________

Function                 For block devices,  such  as  hard  disk  and

floppy   disk,   this   operation  gives  you

information  about  the   attributes   of   a

specified   file,   as  well  as  information

regarding space available on the disk.

PAB format               Parameters passed to STATUS.

byte offset   size      parameter

0             1 byte    opcode = >09

15            1 byte    name length

16            string    file name

Parameters returned from STATUS

byte offset   size      parameter

2             1 byte    error code

14            1 byte    attribute byte

Parameter description

Opcode         >09 is the opcode for the STATUS function.

This opcode can be used on any file, even  if

it  is  open.  It does not apply to character

devices such as RS232 and PIO.

Error code

>60            Bad opcode. You attempted to use  the  STATUS

operation on a character device.

>C0            Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to locate or read from  the  specified

file.  This could mean that a floppy drive is

empty, or there is a bad sector on the floppy

drive.

>E0            General purpose error.  An error which didn't

fit any of the previous descriptions.

GenREF v1.00                          DSR-29

Attribute byte

Bit       Meaning

0 (lsb)   0 =  Not at end of file.

1=   At end of open file,  read  is  not  possible

without  EOF error.  Write is possible if the

file was opened for writing.

1         0=   There is room on the disk to expand the file.

1=   Disk is full, there is no room  to  make  the

file larger.

2         0=   If the file is a  data  file,  it  has  fixed

length records.

1=   If the file is a data file, is  has  variable

length records.

3         0=   If file exists, it is a data file.

1=   If file exists, it is a program image file.

4         0=   If the file  is  a  data  file,  it  contains

display format data.

1=   If the file  is  a  data  file,  it  contains

internal format data.

5              This bit is not used.

6         0=   If the file exists, it is  protected  against

write operations.

1=   If the  file  exists,  write  operations  are

allowed.

7 (msb)   0=   File exists,  all  other  bits  specify  file

attributes.

1=   File does not exist, all other bits  must  be

ignored.

Filename length

This is a count of the number  of  characters

in the filename string.

Filename string

For block devices, such  as  disks  and  hard

disks, this string must contain the name of a

file   for   which   you   want   to   obtain

information.

DSR-30                              GenREF v1.00

___________________________________________________________________________________

FILE ID

___________________________________________________________________________________

Function                 The file ID can be transferred from a file to

your program via the BREAD function.  You can

cause MDOS to create a new file with  all  of

the  characteristics in a file ID provided by

your task by use of the BWRITE function.  The

file  ID is transferred between MDOS and your

task is the buffer whose address you  specify

in the "buffer address" field of the PAB used

by BREAD and BWRITE.

The file ID tells  you  and  MDOS  everything

there  is  to  know about the file other than

where it is actually located on the disk.

File ID format in buffer

offset         size

0              2 bytes   Extended record length for files with records

longer  than  255  bytes.   All  16  bits are

significant, so  a  record  whose  length  is

specified here can be from 256 to 65535 bytes

long.

2              1 byte    File status flag bits.

(lsb)          0=        Data file.

1=        Program image file.

1              0=        If data file, contains display format data.

1=        If data file, contains internal format data.

2              RESERVED

3              0=        File is not protected.

1=        File is write protected.

4-6            RESERVED

(msb)          0=        If data file, contains fixed length records.

1=        If  data  file,  contains   variable   length

records.

3              1 byte    For data files, this is the number of records

which  can fit into on one sector.  This byte

is zero for program images  files  and  files

with record lengths longer than 256 bytes.

4              2 bytes   This is the 16 least significant bits of a 20

bit number representing the number of sectors

reserved  for  the  file.   The   four   most

significant  bits  are  at  offset  18 in the

buffer.

6              1 byte    This is the number of bytes used in the  last

sector  of  the  file.   A zero value in this

byte means that all 256  bytes  in  the  last

sector are used.  This is used in determining

the length of program  image  files,  and  to

determine the EOF for variable record length files.

GenREF v1.00                          DSR-31

7              1 byte    This is the record length for data files with

records shorter than 256 bytes.  This is zero

for program image files and data  files  with

records longer than 255 bytes.

8              2 bytes   These two bytes must be reversed to form a 16

bit  number which is the least significant 16

bits of a 20 bit number used to determine the

EOF   mark   for   files.    The   four  most

significant bits are  at  offset  19  in  the

buffer.

For files with fixed record lengths, this  is

one  greater  than  the  highest  record ever

written to in the file.

For program image files, this is  the  number

of  sectors actually used in the file.  This,

in conjunction with the number of bytes  used

in  the last sector, is used to determine the

number of bytes in the program image.

For variable record files, this is simply the

number  of sectors actually used in the file,

and is only used when you want to APPEND data

to the file.

10             2 bytes   Date of file creation.  This has three fields

packed  into  the  16  available bits.  The 7

most significant bits are the last two digits

of  the year.  The next 4 bits are the month.

The 5 least significant bits are the  day  of

the  month.   The  date  12 May 1988 would be

encoded as (88*512 + 5*32 + 12) = >B0AC

12             2 bytes   Time of file creation.  This has three fields

packed  into  the  16  available bits.  The 5

most significant bits are the hour, from 0 to

23.   The  next 6 bits are the minute, from 0

to 59.  The 5 least significant bits are  the

seconds,  divided  by  2.  The  time 10:58:13

would be encoded as (10*2048 + 58*32 +  13/2)

= >5746

14             2 bytes   Date of  file  update.  Same  format  as  the

creation date.

16             2 bytes   Time of file  update.   Same  format  as  the

creation time.

18             1 byte    This contains the most significant 4 bits  of

the number of sectors reserved for the file.

19             1 byte    This contains the most significant 4 bits  of

the number of sectors used by the file.

GenREF v1.00                             DSR-32

___________________________________________________________________________________

BREAD               ___________________________________________________________________________________

Function                      For block devices,  such  as  hard  disk  and

floppy  disk,  this operation is used to read

sectors directly from the disk or from within

any  type  of  file  located on the disk into

memory at the buffer address  you  specified.

It  can  also  be  used  to read subdirectory

headers from a floppy disk.

With files on a block device, if you  specify

a sector count of zero, this can also be used

to read the file's ID  into  the  buffer  you

specified.

The values returned in the PAB are defined to

make error recovery routines easy to code.

PAB format                    Parameters passed to BREAD.

Byte offset   size      parameter

0             1 byte    opcode = >0A

3             3 bytes   buffer address

6             2 bytes   LSW sector offset

10            1 byte    CPU/VDP flag

12            2 bytes   sector count

14            1 byte    MSB sector offset

15            1 byte    name length

16            string    file name

Parameters returned from BREAD.

byte offset   size      parameter

2             1 byte    error code

3             3 bytes   updated buffer address

6             2 bytes   updated LSW sector offset

12            2 bytes   remaining sectors

14            1 byte    updated MSB sector offset

Parameter description

Opcode         >0A is the opcode for the BREAD  function  in

the DSR.

DSR-33                                 GenREF v1.00

Error code

>60            Bad opcode.  You attempted  to  read  from  a

character oriented device such as RS232 or PIO.

>A0            Read past end of file.  When you are  reading

sectors  directly  from a disk, this error is

reported when you attempted to read a  sector

number  higher  than  any sector on the disk.

When you are reading from a file, this  error

is reported to you when you attempt to read a

sector past the last sector reserved for file

(not at the last sector used by the file.)

>C0            Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to locate or read from  the  specified

file.  This could mean that a floppy drive is

empty, or there is a bad sector on the  block

device.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error  if  the  file  does  not

exist on the specified device.

Buffer Address This is where  you  specify  the  address  to

which data is to be transferred.

For transfers to VDP RAM, only the lowest  17

bits of the 3 bytes are significant.

For transfers to CPU RAM, only the lowest  21

bits of the 3 bytes are significant.  For CPU

RAM transfers, the  lowest  13  bits  are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU  address  your  task  will use to

examine the data, depending on how your  task

has  altered  the map of its execution memory

pages (see the section on Memory Management.)

On return from  the  BREAD  function  with  a

non-zero  sector  count,  the  buffer address

will be updated to point to the first byte in

memory  after  the  data you just loaded.  If

there  were  no  errors  this  will  be  your

initial  buffer  address + 256 * sectors.  If

there were errors, this  will  point  to  the

where  the first bad sector encountered would

have been loaded.

Sector offset  The two bytes at offset 6 in the  PAB,  along

with the byte at offset 14 in the PAB, form a

20 bit sector  offset  within  the  specified

file  or  device.  Sector numbering starts at

zero, not at one.

GenREF v1.00                         DSR-34

On return from the BREAD  function,  this  is

updated  to  contain the offset of the sector

in   the   file   after   the   last   sector

successfully  read.   If  there  was an error

while reading sectors, this will contain  the

offset of the sector which MDOS was unable to read.

CPU/VDP Flag      If this byte is zero, data will be transferred

To a buffer in the memory belonging to your task,

at the address specified in the buffer address.

If this byte is non-zero, data will be transferred

to VDP RAM.

Sector count      To read sectors from a disk or from a file, this must

be set to a non-zero number which tells MDOS how many

sectors you want to read. To read a file ID from a

file into your buffer, you must set this to zero.

On return, this will contain  the  number  of

sectors not read due to an error condition.

Filename length   This is a count of the number  of  characters

in the filename string.

Filename string   For access to a file on a block device,  such

as  disk  or  hard  disk,  this  string  must

contain the name of the device  you  wish  to

access,   followed   by   a   list   of   the

subdirectories separated by periods, followed

by  the  name of the file you wish to access.

(Example:  "HDS1.SOURCE.UTIL.EXAMPLE")    The

length  of  the  name,  including  the period

separators, must be limited to 40 characters.

This  file  must  already exist on the device

you are accessing.  You must specify a sector

count of zero if you want to read the file ID

into your buffer.

For direct  access  to  sectors  on  a  block

device,  this string must contain the name of

the device you wish to access, followed by  a

period. (Example: "HDS1.")

For direct access to the pointers of a floppy

disk  subdirectory,  this string must contain

the name of the floppy disk device,  followed

by  a  period,  followed  by the subdirectory

name, followed by another  period.  (Example:

"DSK1.SUBDIR.")    For   this   subdirectory,

reading from sector zero or sector  one  will

return information unique to that subdirectory

in the same format as sector zero and  sector

one  of  the main floppy disk.  Access to any

sector beyond sector one will simply read the

corresponding  sector from the disk, as if no

subdirectory had been specified.

GenREF v1.00                            DSR-35

___________________________________________________________________________________

BWRITE

___________________________________________________________________________________

Function                      For block devices,  such  as  hard  disk  and

floppy  disk, this operation is used to write

sectors directly to the disk or into any type

of  file  located  on the disk from memory at

the buffer address you specified.

With files on a block device, if you  specify

a sector count of zero, this can also be used

to create a new file with the characteristics

described  in  a  file  ID  in the buffer you

specified.

The values returned in the PAB are defined to

make error recovery routines easy to code.

PAB format                    Parameters passed to BWRITE.

byte offset   size      parameter

0             1 byte    opcode = >0B

3             3 bytes   buffer address

6             2 bytes   LSW sector offset

10            1 byte    CPU/VDP flag

12            2 bytes   sector count

14            1 byte    MSB sector offset

15            1 byte    name length

16            string    file name

Parameters returned from BWRITE.

byte offset   size      parameter

2             1 byte    error code

3             3 bytes   updated buffer address

6             2 bytes   updated LSW sector offset

12            2 bytes   remaining sectors

14            1 byte    updated MSB sector offset

Parameter description

Opcode         >0B is the opcode for the BWRITE function  in

the DSR.

DSR-36                                  GenREF v1.00

Error code

>60            Bad opcode.  You  attempted  to  write  to  a

character oriented device  such as RS232 or PIO.

>80            Disk full.  When  you  were  creating  a  new

file,  there  weren't  enough sectors left on

the disk to create the file.   Alternatively,

the  specified  subdirectory  already had 127

file entries, and your new file could not  be

added to the directory.

>A0            Write past end of file.  When you are writing

sectors  directly  to  a  disk, this error is

reported when you attempted  to  write  to  a

sector  number  higher than any sector on the

disk.  When you are writing to a  file,  this

error  is reported to you when you attempt to

write  to  a  sector  past  the  last  sector

reserved  for  file  (not  at the last sector

used by the file.)

>C0            Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to locate the specified file, or while

trying  to  write to the file, or when trying

to find more  sectors  for  the  file.   This

could  mean  that a floppy drive is empty, or

there is a bad sector on the block device.

>E0            General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error  if  the  file  does  not

exist  on  the  specified  device (unless you

were just creating the file.)

Buffer Address This is where you specify  the  address  from

which data is to be transferred.

For transfers from VDP RAM, only  the  lowest

17 bits of the 3 bytes are significant.

For transfers from CPU RAM, only  the  lowest

21  bits of the 3 bytes are significant.  For

CPU RAM transfers, the lowest 13 bits are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU  address  your  task  will use to

initialize the data, depending  on  how  your

task  has  altered  the  map of its execution

memory  pages  (see  the  section  on  Memory

Management.)

On return from the  BWRITE  function  with  a

non-zero  sector  count,  the  buffer address

will be updated to point to the first byte in

memory  after  the  data  you just saved.  If

there were no errors this will be your initial

GenREF v1.00                         DSR-37

buffer  address  +  256  * sectors.  If there

were errors, this will  point  to  the  first

sector  of  data in your buffer which was not

saved to the block device.

Sector offset    The two bytes at offset 6 in the PAB, along with

the byte at offset 14 in the PAB, form a 20 bit

sector offset within the specified file or device.

Sector numbering starts at zero, not at one.

On return from the BWRITE function, this is updated

to contain the offset of the sector in the file

after the last sector successfully  written.

If there was an error while writing sectors, this

will contain the offset of the sector which MDOS

was unable to write data into.

CPU/VDP Flag     If this byte is zero, data will be transferred

from a buffer in the memory belonging to your

task, at the address specified in the buffer

address. If this byte is non-zero, data  will

be  transferred from VDP RAM.

Sector count     To write sectors to a disk or to a file, this

must  be set to a non-zero number which tells

MDOS how many sectors you want to write.

To create a new file, with the file  ID  from

your buffer, you must set this to zero.

On return, this will contain  the  number  of

sectors not written due to an error condition.

Filename length  This is a count of the number  of  characters

in the filename string.

Filename string  For access to a file on a block device, such as

disk or hard disk, this string must contain the

name of the device you wish to access, followed

by a list of the subdirectories separated by

periods, followed by the name of the file you

wish to access.(Example: "HDS1.SOURCE.UTIL.EXAMPLE")

The length of  the name, including the period

separators, must be limited to 40 characters.

This  file  must  already exist on the device

you are accessing  (unless  you  are  in  the

process  of  creating  the file with your own

file ID.)  You must specify a sector count of

zero  if  you  want to create a new file with

the ID information in your buffer.

For direct  access  to  sectors  on  a  block

device,  this string must contain the name of

the device you wish to access, followed by  a

period. (Example: "HDS1.")

GenREF v1.00                            DSR-38

___________________________________________________________________________________

PROTECT               ___________________________________________________________________________________

Function                      For block devices,  such  as  hard  disk  and

floppy  disk,  this  operation can be used to

remove  or  set  write-protection   for   the

specified file.

PAB format                    Parameters passed to PROTECT.

byte offset   size      parameter

0             1 byte    opcode = >0C

2             1 byte    protection flag.

15            1 byte    name length

16            string    file name

Parameters returned from PROTECT.

byte offset   size      parameter

2             1 byte    error code

Parameter description

Opcode           >0C is the opcode for the PROTECT function.

This opcode should only be  used  on  a  file

which is not open.

Protection flag  If this byte is zero, the specified file will

be  made  into  an unprotected file.  If this

byte is non-zero,  the  file  will  have  the

write-protect bit set in its directory entry.

Error code

>20              Write Protection violation.  For  files,  the

protection of the specified file could not be

changed  because  the  disk   is   physically

write-protected.

>60              Bad opcode. You attempted to use the  PROTECT

operation on a character device.

>C0              Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to  locate  or  update  the  directory

entry  for  the  specified  file.  This could

mean that a floppy drive is empty,  or  there

is a bad sector on the floppy drive.

>E0              General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if the file you specified

does not exist on the specified device.

DSR-39                              GenREF v1.00

___________________________________________________________________________________

RENAME          ___________________________________________________________________________________

Function                 For block devices,  such  as  hard  disk  and

floppy  disk,  this  operation can be used to

rename  a  file  or  a  subdirectory  on  the

device.   It  can  also be used to change the

volume label on  a  block  device.   After  a

rename  operation  of a file or subdirectory,

the filename in the PAB will  be  updated  to

reflect   the   new   name  of  the  file  or

subdirectory.

PAB format               Parameters passed to RENAME.

byte offset   size      parameter

0             1 byte    opcode = >0D

3             3 bytes   buffer address

10            1 byte    CPU/VDP flag

15            1 byte    name length

16            string    file name

@buffer       10 bytes  new file name

Parameters returned from RENAME

byte offset   size      parameter

2             1 byte    error code

15            1 byte    name length

16            string    new file name

Parameter description

Opcode         >0D is the opcode for the RENAME function.

This opcode should only be  used  on  a  file

which is not open.

GenREF v1.00                    DSR-40

Error code

>20              Write Protection violation.  The name of  the

specified  file  or subdirectory could not be

changed  because  the  disk   is   physically

write-protected.

>60              Bad opcode. You attempted to use  the  RENAME

operation on a character device.

>C0              Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to  locate  or  update  the  directory

entry  for  the  specified  file.  This could

mean that a floppy drive is empty,  or  there

is a bad sector on the floppy drive.

>E0              General purpose error.  An error which didn't

fit  any  of  the previous descriptions.  You

will get this error if the file you specified

does not exist on the specified device.

Buffer Address   This is where you specify  the  address  from

which  MDOS  will obtain the new name for the

file.

For transfers from VDP RAM, only  the  lowest

17 bits of the 3 bytes are significant.

For transfers from CPU RAM, only  the  lowest

21  bits of the 3 bytes are significant.  For

CPU RAM transfers, the lowest 13 bits are  an

offset  into one of your task's memory pages,

and the other 8 bits specify  which  of  your

task's  pages  the  transfer starts on.  This

address is not necessarily the  same  as  the

16-bit  CPU address your task will use to set

the filename, depending on how your task  has

altered the map of its execution memory pages

(see the section on Memory Management.)

New name         The buffer must contain  ten  characters  for

the  new  name  of the file or directory.  If

the name is shorter than ten characters,  you

must  add  enough trailing spaces to the name

to make the buffer contain ten characters.

CPU/VDP Flag     If  this  byte  is   zero,   data   will   be

transferred   to   a  buffer  in  the  memory

belonging  to  your  task,  at  the   address

specified  in  the  buffer  address.  If this

byte is non-zero, data will be transferred to

VDP RAM.

Filename length  This is a count of the number  of  characters

in the filename string.

On return from  the  RENAME  operation,  this

contains the length of the new filename.

DSR-41                                GenREF v1.00

Filename string  To rename a file on a block  device  such  as

disks   and  hard  disks,  this  string  must

contain the name of the device  you  wish  to

access,   followed   by   a   list   of   the

subdirectories separated by periods, followed

by  the  name of the file you wish to rename.

(Example: "HDS1.SOURCE.UTIL.FILE") The length

of the name, including the period separators,

must be limited to 40 characters.

To rename a subdirectory  on a  block  device

such  as  disks  and  hard disks, this string

must contain the name of the device you  wish

to   access,   followed  by  a  list  of  the

subdirectories separated by periods, followed

by  the  name of the subdirectory you wish to

rename,   followed   by    another    period.

(Example:   "HDS1.SOURCE.UTIL.DIRECT.")   The

length of  the  name,  including  the  period

separators, must be limited to 40 characters.

To change the name of a  volume  in  a  block

device,  this  string  must  contain only the

name of  the  device,  and  the  buffer  must

contain the new name for the volume.

GenREF v1.00                            DSR-42

___________________________________________________________________________________

FORMAT              ___________________________________________________________________________________

Function                      For floppy disk block devices, this operation

is  used to initialize blank diskettes, or to

reinitialize  used  diskettes  (erasing   the

previous contents of the disk.)

PAB format

Parameters passed to FORMAT.

byte offset   size      parameter

0             1 byte    opcode = >0E

3             1 byte    tracks

4             1 byte    skew

5             1 byte    interlace

6             1 byte    density

7             1 byte    sides

15            1 byte    name length

16            string    device name

Parameters returned from FORMAT

byte offset   size      parameter

2             1 byte    error code

8             2 bytes   sector count

11            1 byte    actual sides

12            1 byte    sectors per track

Parameter description

Opcode         >0E is the opcode for the FORMAT function.

This opcode should only be used on a device

which does not have any open files.

DSR-43                                  GenREF v1.00

Error code

>20            Write Protection violation.  The name of  the

specified  file  or subdirectory could not be

changed  because  the  disk   is   physically

write-protected.

>60            Bad opcode. You attempted to use  the  FORMAT

operation   on   a  character  or  winchester

device.

>C0            Media   error.    For   some   reason,   MDOS

encountered   an  unrecoverable  error  while

trying to initialize the  disk.   This  could

mean that a floppy drive is empty, there is a

bad sector on the floppy drive, or  the  disk

drive is defective.

>E0            General purpose error.  An error which didn't

fit any of the previous descriptions.

Tracks         This is the number  of  tracks  per  side  to

format  on  the specified device.  For double

sided disks, this must be either  40  or  80.

For single sided disks, this can range from 1

to 40, or 80.  Track counts between 40 and 80

can not be used.  Track counts larger than 80

cannot be used.

Skew           This is the rotational delay between the last

sector  on a specified track, and sector zero

on the  next  track,  specified  in  sectors.

This  delay  can  be optimized so that sector

zero of the subsequent track is ready as soon

as  a  head  step operation between tracks is

complete, eliminating an  extra  rotation  of

the  floppy disk after a head step.  One disk

revolution is 200 milliseconds on a  standard

5.25"  floppy  disk.  The optimum delay for a

single-track   head   step   is   about    35

milliseconds.   The optimum skew is generally

(35/200) multiplied by the sectors per track,

rounded up to the next highest integer.

Skew is NOT  the  spacing  between  the  zero

sector on consecutive tracks.

Interlace      Interlace is used to adjust rotational delays

between consecutively numbered sectors on the

same track.  After reading a sector from  the

disk, MDOS will immediately process data from

the sector, returning  information  from  the

sector to the calling task.  In the time MDOS

spends processing the data, the disk is still

turning and several sectors may pass the head

of the disk drive.  If one of  those  sectors

which went by as MDOS was processing the data

happened to be  the  next  sector  that  MDOS

needed, MDOS would have to wait for the disk to

GenREF v1.00                       DSR-44

spin around again before it could  read  that

sector  from  the disk.  Interlace is used to

specify how many sectors are likely to go  by

as  MDOS  is  processing  the current sector.

Interlace is the number of revolutions of the

disk  it  would  take  MDOS to read an entire

track,  assuming   that   MDOS   could   read

consecutively numbered sectors from the track

without the extra rotational delay.  With  an

interlace   of   2,   consecutively  numbered

sectors on a track would have another  sector

between  them,  a sector which MDOS will miss

as it is processing the data from  the  first

sector.

Density        Setting this byte to >02 will cause  MDOS  to

format  the disk with 18 sectors per track on

a double-density  controller  card.   Setting

this  byte to any other value will cause MDOS

to format the disk with 9 sectors  per  track

on all controller cards.

Sides          Setting this byte to >02 will cause  MDOS  to

format both sides of a disk in a double-sided

drive.  If the drive is not double-sided, and

you  set  this  byte to >02, only one side of

the disk will  be  formatted.   Setting  this

byte  to  any value other than >02 will cause

MDOS to only format one side of the disk.

Sector Count   On return from the FORMAT operation, this  is

a  16  bit integer which indicates the number

of sectors available on a  freshly  formatted

diskette.

Actual sides   On return from the FORMAT operation, this  is

the  number  of  sides  of the disk that MDOS

formatted.  For a single  sided  drive,  this

will  always  be  >01.   For  a  double sided

drive, the number here  will  depend  on  the

number of sides you asked to be formatted.

Sectors        On return from  the  FORMAT  operation,  this

byte  will  contain  a >09 for single density

disks, and >12 for double density disks.

Device name length

This is a count of the number  of  characters

in the filename string.

Device name    For block devices, such  as  disks  and  hard

disks,  this  string must contain the name of

the device you wish to format, followed by  a

period. (Example: "DSK1.")