# NCP5208 2-IN-1 DDR POWER CONTROLLER SPICE MODELING

# 5.4.1		Scope

The purpose of this design is to model NCP5208 DDR-I/II Termination Regulator.

| Spice Design: | DDR- I/II Termination Regulator |
| --- | --- |
| Performed by: | Kehinde Omolayo |
| Model Last Rev Date: | 2-15-2004 |
| ON Semi Publication Order Number: | ## NCP5208/D |
| ON Semi Revision | ## October, 2004-Rev. 2 |
| SPICE File | NCP5208S.LIB (for IsSpice version) NCP5208P.LIB (for PSpice version) |

# 5.4.2		Functional Description

NCP5208 is a two quadrant linear regulator: it sinks and sources 1.5Adc. The device provides active termination for DDR-I/II SDRAM. In addition to over current limit protection, the regulator is precisely designed to tract half the voltage of VDDQ input and generates active low power good signal (POK) when the system is in regulation.

The device also provides active low shutdown input (SD) that tri-state the VTT output. Soft start feature is also included.

# 5.4.3		Assumptions

- The temperature for this model is 25 degrees centigrade.

- Thermal Shutdown is not included

- Analog supply input (AVIN) is greater or equal to power supply input (PVIN)

# 5.4.4		 Methods of modeling

IsSpice4 and Pspice were used to model the device. The IsSpice net list is as follows and the Pspice version is available upon request:

* IsSpice Model Editor

* |=======================================================

* |                     NCP5208        REV - 01/11/04

* |                (IsSPICE VERSION)

* |                ON SEMICONDUCTOR

* |          DDR-I/II TERMINATION REGULATOR

* |

* | This model was developed for ON Semiconductor by:

* | OK Power Systems

* |

* |

* |

* | Copyright 2004 OK Power Systems

* | All Rights Reserved

* |

* | The content of this model is subject to change

* | without notice and may not be modified or altered

* | without permission from ON Semiconductor. This model

* | has been carefully checked and is believed to be

* | accurate, however neither OK Power Systems nor

* | ON Semiconductor assume liability for the use of

* | this model or the results obtained from using it.

* |

* | For more information regarding modeling services,

* | model libraries or simulation products, please

* | call at (631) 654-0253 ext 116 or 106.

* |

* |=======================================================

*Designed by Kehinde Omolayo 2-15-2004

.SUBCKT NCP5208 1 2 3 4 5 6 7 8

*DDR-I/II TERMINATION REGULATOR

*Terminal Identification

*POK=1 GND=2 VFB=3 SD=4 VDDQ=5 AVIN=6 PVIN=7 VTT=8

.MODEL POK5203M NMOS Level=1 CBD=8.44P CBS=10.1P CGBO=21.6N

+ CGDO=2N CGSO=2.4N GAMMA=.893 IS=250P KP=19.3M LAMBDA=22M

+ MJ=.46 PB=.8 PHI=.75 RS=0 VTO=1.1

.MODEL H5203D D CJO=2P

.MODEL L5203D D CJO=2P N=0.1

.SUBCKT RS1500 2 3 4 5

C1 11 0 10P

R3 10 8 144.269

C3 8 0 10P

B1 10 0 V=V(2)>1 ? V(5)>1 ? 0 : 2

B2 16 0 V=V(3)>1 ? V(4)>1 ? 0 : 2

B5 4 0 V=V(8)>1 ? 2 : 0

B6 5 0 V=V(11)>1 ? 2 : 0

R1 16 11 144.269

.ENDS

X1 26 10 11 28 RS1500

B1 26 0 V=V(10)<1 ? 2 : V(4)>1.24+V(2) ? 0 : 2

B2 10 0 V=V(4)<0.69+V(2) ? 0 : V(9)

V1 9 0 PULSE 0 2

R1 4 2 1MEG

I1 6 4 DC=5.5U

D1 4 6 H5203D

B3 6 2 I=1m*(V(6)-V(2))*V(11)

R2 2 5 50K

B4 25 2 V=V(8)>(0.48*V(5)+0.52*V(2)) ?

+V(8)<(0.52*V(5)+0.48*V(2)) ? 5 : 0

M1 1 32 2 2 POK5203M

C1 24 0 2.778N

R3 12 16 10K

B5 24 0 I=V(11)<1 ? 1M : -453.46N*V(12)*(EXP(2.24*V(12))-1)

V2 21 19

D2 27 20 H5203D

D3 30 20 H5203D

G1 20 0 11 0 0.1025

V3 18 27

V4 30 17

E1 18 0 19 8 10

E2 0 17 19 8 10

R4 19 8 1MEG

B6 19 8 I=10*(I(V4)+I(V3))

E3 21 2 22 0 1

R5 23 22 10k

C2 22 0 1n

B7 23 0 V=(V(7)-V(2))/(1+EXP(V(13)))

B8 0 13 V=70*(1-EXP(V(14)))/(1+EXP(V(14)))/

+(1+EXP(-5.3*(V(6)-V(2)-0.944)))

B9 0 14 V=3*(V(16)-V(15)+V(2))

R6 3 2 170MEG

R7 2 8 1MEG

B11 7 2 I=I(V2)>0 ? I(V2) : 0

E4 12 0 5 2 0.5

D4 24 12 H5203D

D5 0 24 L5203D

D6 16 29 L5203D

E5 29 0 24 0 1

D7 8 7 H5203D

D8 2 8 H5203D

E6 15 3 0 17 0.74

R9 32 25 20

.ENDS

# 5.4.5		 Model Tests

The IsSpice test fixture for VTT start-up and shutdown modes is shown in figure 5.4.1, and the Spice net List is shown in figure 5.4.2. The SPICE simulation waveforms are shown in figure 5.4.3.

Figure 5.4.1. Start-up/shutdown application Schematic

Figure 5.4.2. PWM APPLICATION Net List

.TRAN 0.1U 5M

.PRINT  TRAN VOUT

.PRINT  TRAN SD

.PRINT  TRAN POK

X1 5 0 1 6 4 2 2 1 NCP5208

C1 1 3 0.1U

RLOAD 1 0 0.833

V1 4 0 DC=2.5

V2 2 0 DC=2.5

R2 3 0 1

V3 6 0 PULSE 0 2 0 1.5M 1.5M 2M

R3 2 5 10K

.END

Figure 5.4.3: Start-up/shutdown application waveforms

# 5.4.6 		Conclusions and Recommendations

The application shutdown input signal was ramped up to verify the enable input trip points. The model enable upper level is 1.25V and the lower level is 0.69. The model hystereis is 0.492V, and it is within 0.40Vmin-0.68Vmax specification.