#include <pic18.h>

//#include <init_vi.h>

#define bitset(var,bitno) ((var) |= 1 << (bitno))

#define bitclr(var,bitno) ((var) &= ~(1 << (bitno)))

unsigned int channel_number;

typedef enum {forward, reverse, brake} action;

typedef struct channel_info {

char chan_num;  //

unsigned int timer_start;  // start time, usually

unsigned int timer_end;	// read from tmr 1 at interrupt time

unsigned int duration;  // timer calcualted duration

unsigned int duty_cycle;  // percent that the pulse is hi and gets decremented

unsigned int int_duty_cycle;  // percent that the pulse is hi and gets decremented

action direction ; // forward, reverse, brake

char sync_count;   //  counts number of sync pulses

char sync;   //  if this is 1, then sync ed

} channel;

channel chan[6];  // there are 6 structures, one for each channel

unsigned int temp_dc[6];

//used by interrupt to populate the right structure

// increments at end of interrupt

unsigned int operating_channel;

unsigned int count;

void interrupt ISR(void)

{

// get the channel

if (operating_channel < 1)

{

TMR0H = 0;

TMR0L = 0; // set timer to 0

chan[operating_channel].timer_start = TMR0;

}

else

{

chan[operating_channel-1].timer_end = TMR0;

TMR0H = 0;

TMR0L = 0;

if (operating_channel < 6)  chan[operating_channel].timer_start = TMR0;

}

operating_channel++;

bitclr (INTCON,1); //clear RB0 interrupt flag

return;

}

void main(void)

{

int i;

count = 0;

// Oscillator control register

IRCF0 = 1;  // 111 = 8MHz osc frequency

IRCF1 = 1;

IRCF2 = 1;

SCS1 = 1;  // select internal osc but only get 8MHz

SCS0 = 0;

SCS1 = 0;  // select primary clock to get 32MHz, works !

SCS0 = 0;

OSCTUNE = 0x40;  //enable PLL to get 32MHz Osc

//Timer 0

T0PS0 = 1;

T0PS1 = 0;

T0PS2 = 0;

TMR0ON = 1;

T08BIT = 0;

T0CS = 0;

PSA = 0;

//PSA = 1;  prescale off

//PORT B

//RB0 - RB2  interrupts

//RB3 - RB7 other pins like OSC etc.

PORTB = 0x00;  //init the port

ADCON1 = 0xff;

TRISB = TRISB | 0x01;  //bit 1 to bit 3 are output

PORTC = 0x00;

TRISC = 0x00;  // BITs o/p

PORTD = 0x00;

TRISD = 0x00;

IDLEN = 0;  //oscon register: run mode enable

// do interrupts last....  do not want them going off too soon.

// rcon register

//GIEL = 0; //peripheral interrupt - disabled

INTCON2 = 0x40;  // RISING EDGE ON INT 0

//  int 1, int 2 - disabled,

INTCON3 = 0x00;

INTCON = 0x90; //enable interrupt RB0

IPEN = 0;

GIEH = 1; // global enable interrupt  1-enable 0-disable

while (1) {

INTCON = 0x90; //enable interrupt RB0

INTCON2 = 0x40;  // RISING EDGE ON INT 0

//  int 1, int 2 - disabled,

INTCON3 = 0x00;

chan[0].duty_cycle = temp_dc[0];

chan[1].duty_cycle = temp_dc[1];

chan[2].duty_cycle = temp_dc[2];

chan[3].duty_cycle = temp_dc[3];

chan[4].duty_cycle = temp_dc[4];

chan[5].duty_cycle = temp_dc[5];

for (i=0; i<100; i++)

{

channel_number = 0;

if 	(chan[channel_number].duty_cycle > 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTC = LATC | 0b00000001;

PORTC = LATC & 0b11111101;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11111110;

PORTC = LATC | 0b00000010;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b00000000;

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTC = LATC & 0b11111110;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11111101;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b11111100;

}

chan[channel_number].duty_cycle = 0;

}

//------------------------------------------------------

channel_number = 1;

if 	(chan[channel_number].duty_cycle != 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTC = LATC | 0b00000100;

PORTC = LATC & 0b11110111;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11111011;

PORTC = LATC | 0b00001000;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b00000000;  //needed prevent FET meltdowm

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTC = LATC & 0b11111011;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11110111;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b11110011;  //needed prevent FET meltdowm

}

chan[channel_number].duty_cycle = 0;

}

//------------------------------------------------------

channel_number = 2;

if 	(chan[channel_number].duty_cycle != 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTC = LATC | 0b00010000;

PORTC = LATC & 0b11011111;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11101111;

PORTC = LATC | 0b00100000;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b00000000;

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTC = LATC & 0b11101111;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b11011111;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b11001111;

}

chan[channel_number].duty_cycle = 0;

}

//------------------------------------------------------

channel_number = 3;

if 	(chan[channel_number].duty_cycle != 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTC = LATC | 0b01000000;

PORTC = LATC & 0b01111111;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b10111111;

PORTC = LATC | 0b10000000;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b00000000;

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTC = LATC & 0b10111111;

}

else if (chan[channel_number].direction == reverse) {

PORTC = LATC & 0b01111111;

}

else if (chan[channel_number].direction == brake) {

PORTC = LATC & 0b00111111;

}

chan[channel_number].duty_cycle = 0;

}

//------------------------------------------------------

channel_number = 4;

if 	(chan[channel_number].duty_cycle != 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTD = LATD | 0b00000001;

PORTD = LATD & 0b11111101;

}

else if (chan[channel_number].direction == reverse) {

PORTD = LATD & 0b11111110;

PORTD = LATD | 0b00000010;

}

else if (chan[channel_number].direction == brake) {

PORTD = LATD & 0b00000000;

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTD = LATD & 0b11111110;

}

else if (chan[channel_number].direction == reverse) {

PORTD = LATD & 0b11111101;

}

else if (chan[channel_number].direction == brake) {

PORTD = LATD & 0b11111100;

}

chan[channel_number].duty_cycle = 0;

}

//------------------------------------------------------

channel_number = 5;

if 	(chan[channel_number].duty_cycle != 0 )  // then the line is active

{

if (chan[channel_number].direction == forward) {

PORTD = LATD | 0b00000100;

PORTD = LATD & 0b11110111;

}

else if (chan[channel_number].direction == reverse) {

PORTD = LATD & 0b11111011;

PORTD = LATD | 0b00001000;

}

else if (chan[channel_number].direction == brake) {

PORTD = LATD & 0b00000000;

}

chan[channel_number].duty_cycle--;  //decrement count

}

else   // set the line lo, inactive

{

if (chan[channel_number].direction == forward) {

PORTD = LATD & 0b11111011;

}

else if (chan[channel_number].direction == reverse) {

PORTD = LATD & 0b11110111;

}

else if (chan[channel_number].direction == brake) {

PORTD = LATD & 0b11110011;

}

chan[channel_number].duty_cycle = 0;

}

// read the input bit

if (RB0 == 0)  chan[0].sync_count++;

else chan[0].sync_count = 0;

if ( (chan[0].sync_count > 50) && (chan[0].sync_count < 60)  )   // if #2

{

chan[0].sync = 1;

operating_channel = 0; // set this for the first channel when the interrupt is called

if (chan[0].sync_count == 51)  {  // if #1

for ( channel_number = 0; channel_number < 6; channel_number++ )

{

chan[channel_number].duration = chan[channel_number].timer_end - chan[channel_number].timer_start;

if (chan[channel_number].duration > 3100 )

{

chan[channel_number].direction = forward;

temp_dc[channel_number] = \

( ( chan[channel_number].duration - 3000 ) ) / 7;

if (temp_dc[channel_number] > 100) temp_dc[channel_number] = 100;

}

else if (chan[channel_number].duration < 2900 )

{

chan[channel_number].direction = reverse;

temp_dc[channel_number] = \

( ( 3000 - chan[channel_number].duration ) ) / 7;

if (temp_dc[channel_number] > 100) temp_dc[channel_number] = 100;

}

else

{

chan[channel_number].direction = brake;

temp_dc[channel_number] = 0;

}

} // end of for

}  // end of if #1

}  // end of if #2

} // end of for loop  (  i )

} //end of while

}