## Additional notes available on the Web site:

## Setting Up UrJTAG for a new device

Note that UrJTAG is still under development and has a few issues. For instance, while the detect command finds the instruction register length, it is still necessary to manually enter it with “instruction length 8”. These little glitches can be difficult for a new user yet they really are pretty minor. It turns out that if the UrJTAG directory structure is properly setup this issue goes away. Since you might want to include a new FPGA or CPLD the following description details the steps needed for UrJTAG to access the *.bsd files.

UrJTAG looks for the BSDL files associated with a particular device identifier read from the chip. The BSDL files are stored in the directory “c:/usr/local/share/urjtag/MANUFACTURER/PARTS/*.bsd“. The directory hierarchy, starting with the manufacture defines one directory for each manufacturer. Within the manufacturer directory there are directories for each of the manufacturer’s parts. Finally within the individual part directories there is a BSDL file associated with the explicit part. UrJTAG reads the 32 bit device identifier register that is provided by all JTAG devices; from this value the fields are parsed to identify the manufacture, part and device stepping. Bits 11-1 define the manufacturer, bits 27-12 define the part and finally bits 31-28 define the device stepping.

Example:

Assuming the Chameleon is used the CPLD device is an XC9572XL.

UrJTAG reads the device identifier an retrieves the following 32 bit result

Device Id: 	01011001011000000100000010010011 (0x0000000059604093)

Parsing the string UrJTAG determines the following

Manufacturer 	xxxxxxxxxxxxxxxxxxxxx00001001001x

Part		xxxx10010110000001xxxxxxxxxxxxxx

Stepping		0101xxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Searching the manufacturer directory UrJTAG finds a MANUFACTURER file which defines the PARTS directory for the bit code.

c:/usr/local/share/urjtag/  		00001001001 => Xilinx

Searching the Xilinx directory UrJTAG finds a PARTS file which defines the part directory for the bit code.

c:/usr/local/share/urjtag/Xilinx  		10010110000001 => XC9572XL

Finally, searching the STEPPINGS file UrJTAG finds a BSDL file for the bit code.

c:/usr/local/share/urjtag/Xilinx/XC9572XL  0101 => Stepping1 xc9572xl_vq64.bsd

## Programming the AVR to supply CLKO to the CPLD

The first images MKIIJTAGV1.4.svf or MKIIISPV1.4.svf, when programmed into the CPLD allows the Chameleon to be directly connected to the MKII ICE for JTAG or ISP programming. This is necessary because the default configuration of the AVR does not support CLKO. The CLKO bit must be enabled to use the 16MHz processor clock within the CPLD. The test image is ChameleonV1.4_Test.svf depends on CLKO being programmed. The MKIIJTAGV1.4.svf is a great development support if you want to use an ICE emulator. It is also possible to use the inexpensive AVR Dragon5 using the MKIIJTAGV1.4.svf to access the AVR for programming and emulation. Note that a straight through 14 to 10 pin ribbon cable is used. Pins 11-14 are left unconnected.

How to de-brick the Chameleon

In normal operation the R9 jumper is installed however if you load an unusable image into the CPLD it is possible to force the AVR into a permanent reset state. To recover from this error it is possible to remove R9 to release the AVR RST line and allow the AVR to re-program the hardware as required. Isolating the CPLD from the AVR allows you to boot the AVR and use the ‘Flip’ loader to re-program the CPLD.