Command Set for the Iso-Pi Board

Communications Parameters:

The serial data link uses the Raspberry Pi’s ttyAMA0 serial port. The communication parameters used are the Raspberry Pi’s default parameters: 115,200 Baud, 8 bits, No parity. There are no provisions for Hardware handshaking on the Raspberry Pi, when using the Wheezy distro of the Linux OS. Therefore the ATMega328 MCU on the Iso-Pi uses an interrupt-driven, buffered  input routine which is capable of keeping up with this high baud rate.

Note however, that the ‘mega328’s input buffer size is only 40 characters. This is more than adequate to handle any of the defined commands and their associated parameters. However the program running on the Raspberry Pi must wait until it receives an acknowledgment string back from the Iso-Pi, before sending any further commands.

Acknowledgement string:

“OK”  <CR> <LF>    		          if the command was recognized and executed

OR

“ER”  Ernumber  <CR> <LF>          if the command  was  not recognized or

a parameter was out of range. Ernumber is

a numeric ASCII string corresponding to the

specific error encountered. Applicable ernumbers are shown with the individual command descriptions

In the case of commands that return a value back to the host, this value will be sent as an ASCII  numeric string, followed  by a  <CR> <LF> . Following this response, the normal acknowledgement string, shown above, will be sent.

While Linux uses the <LF> as its termination character, for the acknowledgement, I send a <CR> as well, as this makes it much easier to interact with the ISO-Pi when using a terminal program, such as Minicom, on the Raspberry Pi. This puts every command issued, and its results, on a separate line in the terminal display screen.

NOTE:  As mentioned in Part 2 of the associated article, the Raspberry Pi uses ttyAMA0 as a default console port. That means that, upon boot-up, the hundreds of lines of status information that you see on your HDMI monitor, will also sent out to the ttyAMA0 serial port. You must follow the instructions in the article to disconnect ttyAMA0 from the default console, or all of this extraneous data will be received by the Iso-Pi board, and it will be confused/overwhelmed by it.

Iso-Pi Commands

Iso-Pi commands consist of single letter mnemonics to define the type of operation to be performed. In some cases, this mnemonic may also be followed by one or more additional single letter mnemonics which break the command into different sub-categories. Next, there may be a numeric parameter, if applicable. This parameter is always sent as a numeric ASCII string.

All commands must be terminated by a <CR> character. Unlike Linux, the ATmega328 is running a BASCOM/AVR BASIC program, and the <CR> character is what is recognized as a terminator in BASIC.

ADC Command. This performs a conversion using the Iso-Pi’s 4-channel high resolution sigma-delta ADC. To use this very versatile ADC converter, you must specify the Channel number that you want to read, the conversion rate, as well as the ADC PGA gain that you desire. You should  refer to the MCP3424’s  datasheet for full details, but the various ADC commands are as follows:

AC num  <CR>

This sets up the ADC to read a specific channel number where num is an ASCII character from “0” to “3” corresponding to Channels 1-4. If you specify a number outside this range, an ernumber of “1” will be returned.

AG num  <CR>

This specifies the Gain of the ADC’s PGA. num is an ASCII character from “0” to “3” and selects gains of 1,2,4, or 8. If you specify a number outside this range, an ernumber of “2” will be returned.

AR num <CR>

This specifies the sample rate of the ADC. num is an ASCII character from “0” to “3” corresponding to sample rates of 240,60,15 and3.75 sps respectively. The faster the sample rate, the lower the resolution is: 12 bits at 240 sps up to 18 bits at 3.75 sps. If you specify a number outside this range, an ernumber of “3” will be returned.

AI  CR>

This takes an ADC reading using the parameters selected above. If the parameters are the same from one reading to the next, you do not have to keep re-specifying them.

This command will return a numeric value, in the form of an ASCII string. The ADC is a bipolar ADC, so the value returned will be a signed integer of 12,14,16 or 18 bit resolution, depending upon the sample rate selected above.

The AI command only takes one reading at a time.

Stepper motor commands. If desired, you can hook up a unipolar stepper motor up to the Micrel MIC5821 Driver on the Iso-Pi. The 8 outputs should be strapped together to form 4 discrete outputs ( Out1,Out2 hooked together, Out3,4 etc.): This gives a 1A per phase capacity.

BF < CR>

Move the stepper motor one step forward

BR < CR>

Move the stepper motor one step in reverse

BT num < CR>

Define the step rate in us per step (used in conjunction with the BM command)

Num must be an integer in the 0-32767 range or an ernum of 15 will be returned

How low num can be set will depend upon your stepper motor’s characteristics.

BS num < CR>

Define the number of steps to move (used in conjunction with the BM command)

Num must be an integer in the -32768 to 32767 range or an ernum of 14 will be returned

The sign of num will determine the direction of motion

BM < CR>

This command will start the stepper motor turning using the BT, BS parameters specified above. Note that the motor’s speed will ramp up to the step rate chosen with the BT command, and not use that step rate immediately, allowing for the inherent inertia of stepper motors. When the motor has completed the move, the OK acknowledgement will be sent- NOT BEFORE!

Micrel High Power Driver commands. The Micrel MIC5821 Driver on the Iso-Pi can be used as 8 discrete Low side current sinks (when not being used as the stepper motor controller as described above).

C num   < CR>

num  is a numeric ASCII string in the range 0-255. This byte value is output on the OUT1         to OUT8 pins with the following result:

For each bit position where the value is a logic “1”, the corresponding OUT pin will be in the conducting state, i.e. it will be sinking current to ground. Thus, it should be considered an inverting driver, as each high input will result in a low output (which is the conducting state).

If num is outside the range 0-255, a ernum of 16 will be returned.

Dual 12-bit DAC commands. The Iso-Pi contains a MCP4822 dual 12-bit voltage output DAC, each channel of which can be controlled independently. The full scale (4095) output voltage is 2.048 volts (although the ‘Mega328’s firmware could be modified to use the DAC’s high range, which provides 4.096 volt range).

DA num < CR>

Load DACA with value num

Where num is in the range 0-4096

DB num < CR>

Load DACB with value num

Where num is in the range 0-4096

If num is outside the range 0-4096, an ernum of 4 will be returned.

Rotary Encoder commands. If a Rotary encoder is hooked up, it can be read by the following commands. Note that not all encoders operate the same- the Bourns ECW1D-B24-BC0024 was used to test the Iso-Pi routines. The rotary encoder routine is interrupt-driven. Therefore, regardless of what other commands are being executed by the Iso-Pi, the rotary encoder will still be monitored accurately.

EZ < CR>

This command zeros out the rotary encoder position variable at the current rotary encoder position.

EV < CR>

This command reads the present rotary encoder position. This is returned as an ASCII string representing an integer in the -32768 to +32767 range. This number represents the number of encoder steps moved since the last EZ command. The direction of rotation is represented by the sign of the number returned.

EM < CR>

This command returns a flag representing whether the encoder has moved since the last time the EZ command was sent. If there was no motion, a “0” is returned, else a “1” is returned.

Key Pad commands.  If a 4 X 4 Matrix key pad is connected to the MCP23S17 Port A, it can be scanned and read out by the following commands.

KI < CR>

This is the keyboard initialize command and must be sent first, to setup the MCP23S17 PORT A properly.

KG < CR>

This command will start a keyboard scan and will return a numeric ASCII string representing a  number in the 1 to 16 range, corresponding to the key that is pressed. This is a BLOCKING command- it will not return a result until a key is pressed, and NO further commands will be recognized by the Iso-Pi, until a key is pressed.

If this command is issued in error, or no keyboard is present and working properly, the Iso-Pi will be unresponsive to further commands until it is powered down and restarted.

KF < CR>

This command will start a keyboard scan and return a numeric ASCII string representing a  number in the 1 to 16 range, if a key is presently being pressed. If no keys are being pressed, a “0” will be returned. This is not a BLOCKING command as the KG command is, and is therefore safer to use in most circumstances.

KN < CR>

This command will check the key pad, and will not return until there are NO keys pressed. This command would be used in conjunction with the KF command to allow one to distinguish one key press from another. This is a BLOCKING command- it will not return until all keys are released. The Iso-Pi will accept no more commands until this command completes.

DC motor commands.  Up to two DC motors can be connected to the Iso-Pi’s L298 H-Bridge driver. The speed of the motors is set by an 8-bit PWM operating at 563 Hz. The following commands control each motor independently.

MAS num  < CR>

This command controls the speed of the motor connected to the first H-bridge driver in the L298:  num is an integer in the 0-255 range, where 255 is full power, and 0 is no power. The lowest value that can be used for num, before the motor stalls, will depend upon the motor and its load.

MAF  < CR>

This makes the motor turn in the forward direction

MAR  < CR>

This makes the motor turn in the reverse direction

MAB  < CR>

This places a short across the motor terminals, which acts as a  brake to slow the motor down rapidly.

Depending upon your application, you would either set the speed to 0, to stop rotation gradually, or invoke the MAB command to stop it more quickly. It would not be advisable to switch directions instantaneously, by leaving the speed as-is, and just sending a command of the opposite direction.

For the motor connected to the 2nd section of the L298, all of the commands above should be replaced with MBx  ( plus num if applicable)

MCP23S17 Dual 8-bit Parallel Port commands. The MCP23S17 is a very versatile dual 8-bit parallel port IC, interfaced to the ATmega328 via the high speed SPI bus. So, its response time is limited basically by the speed of the 115,200 baud serial link to the Raspberry Pi. I have attempted to provide most of the functionality of the MCP23S17, except for the functions which issue an interrupt when pins change. This functionality does not lend itself to the nature of  the serial link between the Raspberry Pi and Iso-Pi.

PORT A section

PAM  num <CR>

This sets the Port A mode as follows

num =0:  Input mode, no polarity change in input pin, no pull-up

num =1:  Input mode,  polarity change in input pin, no pull-up

num =2:  Input mode, no polarity change in input pin, pull-up enabled

num =3:  Input mode,  polarity change in input pin, pull-up enabled

num =4:  Output mode

PAO num <CR>

Sends value num out to PORT A

PAI <CR>

Returns the current state of  PORT A as a numeric string in the range 0-255, followed by

<CR> <LF>  (and  followed by the normal OK acknowledgement string )

PORT B section. All of the above commands apply, but every command now starts with

PB rather than PA

Servo Motor commands: If an RC servo is connected to the Servo connector, it can be controlled by the following commands.

SI  <CR>

This command initializes the servo function in the firmware, and must be called prior to issuing any other servo commands.

SS  num  <CR>

This command  makes the servo move to the position defined by num: where num is a numeric ASCII string in the range of 0.0 to 100.0 (representing the percentage of full servo motion)

This command will not issue any error if num is outside this range, but will merely limit the values to 0.0 or 100.0 respectively.

Identification command.

? <CR>

If a “?” command is sent, the unit will respond with the identification string

“Raspberry Pi I/O board”  <CR>  <LF> (followed by the normal OK acknowledgement)