X10 Fan Speed control modification

Concept by Russell Salerno 73707,1105

Circuit by Bob Garry 71574,1023

Background

----------

The X10 fan mod grew out of my frustration with the lack of a decent X10 fan motor speed control.  Like many others, I wanted X-10 control of both the fan speed and lighting level.  Several home automation dealers offer a Levitton "fan speed/low voltage lighting" control, but this unit often causes the fan motor to emit excessive humming noise.  A little research turned up the answers.

Ceiling fans use inductive ("squirrel cage") motors.  As it turns out, there are two popular methods for controlling the speed of inductive motors.  One involves switching various capacitors in series with the motor.  This is the method used in just about every "silent" fan control.  The capacitor method works well for any fan, but offers only discrete steps of speed control, and is unavailable in a commercial X-10 unit.

The second method uses a triac, which chops off increasing bits of the AC cycle to decrease the amount of total power supplied to the motor (or light), thus slowing the motor.  This method offers infinite control settings, and is used in the popular X-10 speed controls and dimmers.  There is a problem with using this method to control fan speed, however.  Because the triac switches during the non-zero portion of the AC cycle, at rate of 120 times a second, it generates a very steep "edge" in the voltage going to the fan.  This edge causes the windings in the fan motor to vibrate, which is then heard as hum.  Whether the hum is obectionable depends on the specific design of the motor, and the tolerance of the room occupants to the noise.

I've heard claims that certain "expensive" fans have tighter windings and therefore will not hum with a triac-based control, but if you already have fans or can't spend $400 on a new one, this doesn't really matter.

About a year ago, myself and a very sharp fellow here on CIS by the name of Bob Garry, worked out a modification to a standard X-10 dimmer which would control a capacitor-based fan speed circuit.  (OK, well, Bob did most of the real work!)  It's actually a little more than a simple mod:  it consists of a controller unit which is driven from a modified X10 dimmer.  The dimmer requires this modification in order for it to properly drive the controller.

IMPORTANT

---------

This project uses potentially lethal line voltage, and requires expertise in electronics, house wiring techniques, and local and federal electrical codes.  If you are unsure of your skills in any of these areas, seek help from a qualified source.

Controller description

----------------------

The controller is a circuit which senses the average output voltage of the dimmer, and uses that reading to control a group of solid state relays.  The relays are part of a capcitive speed control circuit, which provides totally silent control of the fan.

The controller will resolve the eight steps of X10 resolution (off + 7 speeds) into five steps of fan control (off + 4 speeds.)  This was done to reduce the physical size of the controller module, as the relays and capacitors are the largest components in the circuit.  One of the design goals was to design the controller to be small enough to fit in the wiring box above the fan.  It's a tough fit, but it can be done.  Besides, four well-spaced fan speeds works out rather well for most environmental situations.

As the X10 dimmer setting is increased, the solid state relays will close in a binary count sequence:

relay 1    relay 2    relay 3   speed

off        off        off       off

off        off        on        low

off        on         off       med

off        on         on        med-high

on         off        off       high

Parts list

----------

C1		1uF, 250v (non-polarized)

C2		470uF, 6.3v

C3, C4		10uF, 6.3v

C5, C6		10uF, 250V (non-polarized)

D1-D4, D6	1N4004 diode

D5		1N4733A 5.1v Zener diode

D7, D8		1N4148 diode

K1-K3		Solid state relays:

Preferred Subsystems Products MP240D2 or

CP Clare "PowerDIP" PD1201

R1		22 ohm

R2		13k, 2W

R3		470k

R4		27k

R5		91k

R6-R13		20k

R14-R16		1.3k

R17, R18	200k

U1		LM339 Quad Comparitor

U2		74HC148 Priority Encoder

misc		enclosure, X10 incandescent dimmer module, etc.

Modifications to X10 Dimmer module:

----------------------------------

The triac used in the stock X10 dimmer has a holding current of 35mA.  All told, this means that the X10 device must see a load of at least 15 watts (approx) otherwise it will never turn on!  Since the controller box does not offer anywhere near this load (nor would we want it to for thermal reasons), the triac in the X10 module must be replaced with a lower "holding current" version.  For this application a holding current of <10 ma is required.  There are several triacs which will do the job, for example, a 1 Amp Teccor L401E5 triac, in a small TO-92 package, has a holding current of 5mA.

Once you have the X10 module open, you will be able to easily recognize the triac - it's connected to the aluminum chassis for heat sinking purposes.  The new triac does not have to be be heat sinked, but it must be properly connected electrically.  Looking down on the existing triac (leads facing away into the circuit board), with the heat sink side facing up, you have from left to right three pins, call them 1, 2 and 3.  Hold the Teccor triac, pins facing away from you, with the flat side down and the curved side up.  Pin 1 of the Teccor replaces pin 1 of the old triac.  Pin 2 of the Teccor replaces pin 3 of the old triac, and pin 3 of the Teccor replaces pin 2 of the old triac.  Consult the data sheet for the Teccor L401E5 and the old triac (typically will be an SGS-Thomson BTA 10.400C) for further details.

Connections

-----------

Typically, the dimmer will reside in a wall switch box and the controller will be in the wiring box above the fan, or in a separate wiring box.  Be extremely careful to insulate the metal box from the circuit!

The controller has four connections: hot, neutral, dimmer, and fan out.

Hot is connected to a constant 120V hot (unswitched) wire.  Note that you may not have an "always hot" wire at your fan.  If you are lucky enough to have separate light and fan lines already in your ceiling, you may be able to rewire things to provide the necessary connections.

The modded X10 dimmer is connected between hot and the "dimmer line out" input of the controller.

On the fan side, the fan motor is connected to "fan out."  The neutral side of the controller and fan are tied together with house neutral.

===========================================================================

Disclaimer:  This information is provided for informational purposes only.

The author assumes no responsibility for any damage, injury or any other

occurrance, mishap or event that may directly or indirectly result from the

use or misuse of any device or procedure described in this document.

===========================================================================