X-10 Professional Developers Library

For

C

=========================================

|                                       |

|               FUNCTS.DOC              |

|                                       |

=========================================

Version 3.0

Manual and Software Copyright 1993

Smart Home Technology

24123 94th Ave. S.

Kent,  WA  98031

(206) 813-8410

ALL RIGHTS RESERVED

THE X10 PROFESSIONAL DEVELOPERS LIBRARY

Here are the constants you need to be aware of when you are reading

through the different functions and commands.  They make your program

more readable and easier to remember than raw numbers.

-----------------------------------------------------------------------

| CONSTANTS FOR X-10 COMMANDS                                         |

-----------------------------------------------------------------------

ALL_LIGHTS_ON          Turn all lights on

ALL_UNITS_OFF          Turn all modules off, lamp and appliance

ON                     Turn module on

OFF                    Turn module off

BRIGHT                 Brighten light one level

DIM                    Dim light one level

-----------------------------------------------------------------------

| MISC USEFUL CONSTANTS AND DEFINES                                   |

-----------------------------------------------------------------------

TRUE  FALSE

YES   NO

SUN   MON   TUE   WED   THU   FRI   SAT

SWITCH  LAMP  APPL  CTRLER

TRANSMIT  RECEIVE

REFRESH  NO_REFRESH

-----------------------------------------------------------------------

| GLOBAL VARIABLES                                                    |

-----------------------------------------------------------------------

Dawn                   Time of day for first daylight

Dusk                   Time of day that darkness starts

PowerOutage            Whether there has been a power outage

ShowTransmit           Do you want to display the transmitted commands

Sunrise                Time of day the sunrises above the horizon

Sunset                 Time of day the sunsets below the horizon

X10Cmd                 Used to hold the received X-10 commands

X10Date                Used to hold the current date

X10Dow                 Used to hold a number for the day of the week

X10Time                Used to hold the current system time

-----------------------------------------------------------------------

|                                                                     |

|                                                                     |

|                    --==>> IMPORTANT NOTE <<==--                     |

|                                                                     |

|                                                                     |

|---------------------------------------------------------------------|

|                                                                     |

|                                                                     |

|                                                                     |

|          IN ORDER TO MAKE THE FUNCTIONS AND COMMANDS                |

|          AS FAST AND EFFICIENT AS POSSIBLE THERE IS                 |

|          NO CHECKING OF PARAMETERS.  WE HAVE LEFT IT                |

|          UP TO THE END USER TO PROVIDE VALID DATA TO                |

|          THE FUNCTIONS AND COMMANDS.                                |

|                                                                     |

|                                                                     |

|          IF THERE SEEMS TO BE A BIG PROBLEM WITH THIS               |

|          WE MAY INCLUDE SOME CHECKING IN FUTURE RELEASES.           |

|                                                                     |

|                                                                     |

|                                                                     |

|                                                                     |

-----------------------------------------------------------------------

add_to_group

FUNCTION       add_to_group

PURPOSE        Adds a list of X10 module addresses to the end of a

defined group.

SYNTAX         add_to_group(GROUP *group, char *unit_addr, ...)

PARAMETERS     GROUP *group    - Group structure to add the modules.

char *unit_addr - X10 module address.

...             - More addresses terminated by a NULL.

EXAMPLE        GROUP outside_lights;

add_to_group(&outside_lights, "O1", "O4", "P2", NULL);

turn_group(outside_lights, ON);

ASSUMES        List of X10 addresses must be terminated by a NULL.

Unit addr is within range of A-P and 1-16.

SIDE EFFECTS   Unpredictable if not terminated by a NULL.

RETURNS        Nothing

NOTES          Need to make sure you are passing the address of your

variable rather than by value.  In other words precede

your variable name with the & operator.

The functions that act on a group will do so in the

order that you add the modules.

add_to_time

FUNCTION       add_to_time

PURPOSE        Adds minutes to a time string to create a new time

string.  You can also subtract minutes by proceeding the

minutes with a minus sign (-).

SYNTAX         add_to_time(char *time_str, int min, char *new_time)

PARAMETERS     char *time_str  - Time to add the minutes to, a string

with a length of 6, ie: "02:00".

int min         - Number of minutes to add or subtract.

char *new_time  - Will contain the new time.  Should

also be a length of 6.

EXAMPLE        char YardZapperTime[6];

add_to_time(Sunset, -30, YardZapperTime);

ASSUMES        Nothing

SIDE EFFECTS   Stores the new time in the passed new_time variable

RETURNS        Nothing

brighten_selected_modules

FUNCTION       brighten_selected_modules

PURPOSE        Send a command to brighten pre-selected lamp modules

within a certain house code.

SYNTAX         brighten_selected_modules(char house, int steps)

PARAMETERS     char house - House code to send command.

int steps  - Number of times to send the bright command.

EXAMPLE        select_module("B1", "B2", "B9", NULL);

brighten_selected_modules('B', 2);

or

if (x10cmd_is("A", BRIGHT))

brighten_selected_modules('B', 2);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will turn on a module before brightening if its off.

RETURNS        Nothing

NOTES          This function is far different in the way it acts

compared to most of the other functions.  It is based on

steps (number of times) the bright command should be

sent out.  Not what level to set the modules to.  The

library will still keep track of the affected selected

modules level.

You send a character rather than a string for the house

code, 'A' and not "A".

cancel_timer

FUNCTION       cancel_timer

PURPOSE        Cancel a user defined X10 timer that was created with

the library function start_timer().

SYNTAX         cancel_timer(int num)

PARAMETERS     int num  - Number of the timer to cancel, 0-20.

EXAMPLE        cancel_timer(1);

ASSUMES        You opened communications to the TW523 module by using

the function call open_tw523() before you access the

timer functions.

SIDE EFFECTS   None

RETURNS        Nothing

close_tw523

FUNCTION       close_tw523

PURPOSE        Close communications to the TW523 module and return all

interrupt vectors to their original state.

SYNTAX         close_tw523(char *filename)

PARAMETERS     char *filename - Name of the file you are using to save

the status file in case of an outage.

EXAMPLE        #define OUTAGE_FILE "outage.sta";

close_tw523(OUTAGE_FILE);

or

close_tw523("outage.sta");

ASSUMES        Nothing

SIDE EFFECTS   Restores vector table to original state and deletes the

outage status file.

RETURNS        Nothing

date_is

FUNCTION       date_is

PURPOSE        Checks to see if the passed date matches the current

date held in the global variable X10Date.

SYNTAX         date_is(char *date)

PARAMETERS     char *date - A string containing the date to check.

Can be formatted as mm-dd or mm-dd-yyyy.

EXAMPLE        if (date_is("04-04") && time_is("10:00"))

flash_module("A1", 10);

if (date_is("10-25-1993") && time_is("02:00"))

refresh_entire_system();

ASSUMES        You are using the function get_x10date to stuff the

global variable X10Date with the current date.

SIDE EFFECTS   None

RETURNS        TRUE if passed date matches the date in X10Date else

FALSE.

NOTES          The get_x10time() function automatically initializes and

updates the X10Date variable.

delete_group

FUNCTION       delete_group

PURPOSE        Delete all the X10 addresses in an existing group.

SYNTAX         delete_group(GROUP *group)

PARAMETERS     GROUP *group - Group structure to delete.

EXAMPLE        delete_group(&outside_lights);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          Need to make sure you are passing the address of your

variable rather than by value.  In other words precede

your variable name with the & operator.

dim_group

FUNCTION       dim_group

PURPOSE        Dim a group of modules to a certain level.

SYNTAX         dim_group(GROUP group, int level)

PARAMETERS     GROUP group - Group structure of the modules.

int level   - Dim the modules to this level.

EXAMPLE        GROUP living_room;

add_to_group(&living_room, "L1", "L4", "L9", NULL);

dim_group(living_room, 10);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          You do not pass address of the variable for this

function.  In other words DO NOT precede the variable

with the & operator.

The function will act on the group in the same order

that you added the modules to the group.

dim_module

FUNCTION       dim_module

PURPOSE        Send a command to dim or brighten a lamp module.

SYNTAX         dim_module(char *unit_addr, int level)

PARAMETERS     char *unit_addr - X10 address of the module.

int level       - Dim the module to this level.

EXAMPLE        dim_module("A1", 8);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Will turn on a module before dimming if necessary

RETURNS        Nothing

NOTES          The number of levels available depends on the type of

module being dimmed.  A lamp module has a total of 21

steps and a wall switch has 17 steps.

A level of 1 is so small it could be considered Off.

If you want to bring the module to the maximum

brightness for its type send it a level of 21 or higher.

This command is also used to brighten a module.  Just

give it the level of brightness you would like the

module to be at and it will do whatever it takes to

achieve the final result.

dim_selected_modules

FUNCTION       dim_selected_modules

PURPOSE        Send a command to dim pre-selected lamp modules within a

certain house code.

SYNTAX         dim_selected_modules(char house, int steps)

PARAMETERS     char house - House code to send the command.

int steps  - Number of steps to dim the modules.

EXAMPLE        select_module("B1", "B3", NULL);

dim_selected_modules('B', 3);

or

if (x10cmd_is("A", DIM))

dim_selected_modules('B', 2);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will turn on a module before dimming if necessary

RETURNS        Nothing

NOTES          This function is far different in the way it acts

compared to most of the other functions.  It is based on

steps or number of times the bright command should be

sent out.  Not what level to set the modules to.  The

library still keeps track of the affected selected

modules level.

Note that you send a character rather than a string for

the house code, 'A' and not "A".

disable_com_interrupt

FUNCTION       disable_com_interrupt

PURPOSE        Disable the com port from generating interrupts.

SYNTAX         disable_com_interrupt()

PARAMETERS     None

EXAMPLE        disable_com_interup();

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

display_status

FUNCTION       display_status

PURPOSE        Quick and dirty display of the status of the modules for

a certain house code.  Intended for debugging not for

general use.

SYNTAX         display_status(char house)

PARAMETERS     char house - House code to display status.

EXAMPLE        if (x10cmd_is("A", ALL_LIGHTS_ON))

display_status('A');

or

if (KeyHit >= 'A' && KeyHit <= 'P') {

display_status(KeyHit);

KeyHit = NULL;

}

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Displays to the screen with printf.

RETURNS        Nothing

NOTES          You send a character rather than a string for the house

code, 'A' and not "A".

display_x10_globals

FUNCTION       display_x10_globals

PURPOSE        Quick and dirty screen display of global variables.

Intended for debugging and not for general use.

SYNTAX         display_x10_globals()

PARAMETERS     None

EXAMPLE        if (time_is("00:01"))

display_x10_globals();

or

if (KeyHit == 'G')

display_x10_globals();

KeyHit = NULL;

}

ASSUMES        Nothing

SIDE EFFECTS   Displays to the screen with printf.

RETURNS        Nothing

dow_is

FUNCTION       dow_is

PURPOSE        Checks to see if the passed day matches the current day

of the week.

SYNTAX         dow_is(int dow)

PARAMETERS     int dow - A numeric of the day of the week with Sunday

being 0 and Saturday being 6.

EXAMPLE        if (x10cmd_is("A1", ON) && dow_is(TUE))

turn_module("B1", ON);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        TRUE if passed day matches the current day of the week

else FALSE.

NOTES          The days of the week have been predefined as:

SUN  MON  TUE  WED  THU  FRI  SAT

dow_is_not

FUNCTION       dow_is_not

PURPOSE        Checks to see if the passed day does not match the

current day of the week.

SYNTAX         dow_is_not(int dow)

PARAMETERS     int dow - A numeric of the day of the week with Sunday

being 0 and Saturday being 6.

EXAMPLE        if (x10cmd_is("A1", ON) && dow_is_not(TUE))

turn_module("B1", ON);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        TRUE if passed day does not match the current day of the

week else FALSE.

NOTES          The days of the week have been predefined as:

SUN  MON  TUE  WED  THU  FRI  SAT

enable_com_interrupt

FUNCTION       enable_com_interrupt

PURPOSE        Enable the com port to generate interrupts.

SYNTAX         enable_com_interrupt()

PARAMETERS     None

EXAMPLE        enable_com_interrupt()

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

flash_module

FUNCTION       flash_module

PURPOSE        Send a command to flash a lamp module on/off.

SYNTAX         flash_module(char *unit_addr, int times)

PARAMETERS     char *unit_addr - X10 address of the module.

int times       - Number of times to flash the module.

EXAMPLE        flash_module("A1", 5);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Will automatically reset flashed modules to the state

they were in before the call, ie: if the module was

originally at brightness level 5 it will be returned to

that level after flashing.

RETURNS        Nothing

flash_selected_modules

FUNCTION       flash_selected_modules

PURPOSE        Send a command to flash pre-selected modules on/off

within a certain house code.

SYNTAX         flash_selected_modules(char house, int times)

PARAMETERS     char house - House code to send the command.

int times  - Number of times to flash the modules.

EXAMPLE        select_module("A1", "A3", NULL);

flash_selected_modules('A', 5);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will automatically reset flashed modules to the state

they were in before the call, ie: if the module was

originally at brightness level 5 it will be returned to

that level after flashing.

RETURNS        Nothing

NOTES          You send a character rather than a string for the house

code, 'A' and not "A".

flush_recv_buffer

FUNCTION       flush_recv_buffer

PURPOSE        Remove all received X10 commands from the buffer.

SYNTAX         flush_recv_buffer()

PARAMETERS     None

EXAMPLE        flush_recv_buffer();

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

force_group

FUNCTION       force_group

PURPOSE        Forces a group of modules to a certain state regardless

of what the library shows for those modules.

SYNTAX         force_group(GROUP group, int on_off)

PARAMETERS     GROUP group - Group structure of the modules.

int on_off  - Force the modules ON or OFF.

EXAMPLE        GROUP outside_lights;

add_to_group(&outside_lights, "O1", "O1", "P2", NULL);

force_group(outside_lights, ON);

ASSUMES        Nothing

SIDE EFFECTS   If you are forcing them ON then it will turn each module

off first then back on.  It will look like a flash.

RETURNS        Nothing

NOTES          To make your program more readable, constants ON and

OFF have been defined rather than using the raw commands

You do not pass address of the variable for this

function.  In other words DO NOT precede the variable

with the & operator.

The function will act on the group in the same order

that you added the modules to the group.

force_module

FUNCTION       force_module

PURPOSE        Forces a module to a certain state regardless of what

the library shows for that module.

SYNTAX         force_module(char *unit_addr, int on_off)

PARAMETERS     char *unit_addr - X10 address of the module.

int on_off      - Force the module ON or OFF.

EXAMPLE        force_module("A1", ON);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   If you are forcing it ON then it will turn the module

off first then back on.  It will look like a flash.

RETURNS        Nothing

NOTES          To make your program more readable, constants ON and

OFF have been defined rather than using the raw commands

force_selected_modules

FUNCTION       force_selected_modules

PURPOSE        Forces pre-selected modules for a house code to a

certain state regardless of what the library shows for

those modules.

SYNTAX         force_selected_modules(char house, int on_off)

PARAMETERS     char house - House code to send the command.

int on_off - Force the modules ON or OFF.

EXAMPLE        select_module("A1", "A3", NULL);

force_selected_modules('A', ON);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   If you are forcing them ON then it will turn the

selected modules off first then back on.  It will look

like a flash.

RETURNS        Nothing

NOTES          To make your program more readable, constants ON and

OFF have been defined rather than using raw commands.

You send a character rather than a string for the house

code, 'A' and not "A".

get_module_level

FUNCTION       get_module_level

PURPOSE        Get the level of a module from the status table.

SYNTAX         get_module_level(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        if (get_module_level("A1")) {

do_something();

do_something_else();

}

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        0 = OFF, 1 - 21 is ON showing the brightness level.

NOTES          For plug in lamp modules and appliance modules the max

level is 21.  Wall switches have a max of 17.

get_module_max

FUNCTION       get_module_max

PURPOSE        Return the maximum level a module is able to achieve

based on the type of module showing in the status table.

SYNTAX         get_module_max(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        max = get_module_max("A1");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Integer, either 17 for plug-in lamp modules or 21 for

the rest of the modules.

get_module_refresh

FUNCTION       get_module_refresh

PURPOSE        Return the refresh status for the passed X10 address.

SYNTAX         get_module_refresh(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        if (get_module_refresh("A1"))

do_something();

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        TRUE if refreshable, else FALSE.

get_module_select

FUNCTION       get_module_select

PURPOSE        Get the selected status of a module from the status

table.

SYNTAX         get_module_select(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        if (get_module_select("A1"))

do_something();

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        TRUE if selected, else FALSE.

get_module_slot1

FUNCTION       get_module_slot1

PURPOSE        Get the user defined value in slot1 for the passed X10

address.

SYNTAX         get_module_slot1(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        my_slot1 = get_module_slot1("A1");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Integer

NOTES          This is an integer slot for your own usage.

get_module_slot2

FUNCTION       get_module_slot2

PURPOSE        Get the user defined value in slot2 for the passed X10

address.

SYNTAX         get_module_slot2(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        my_char_pointer = get_module_slot2("A1");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Character pointer.

NOTES          This slot is for your own usage.

get_module_slot3

FUNCTION       get_module_slot3

PURPOSE        Get the user defined value in slot3 for the passed X10

address.

SYNTAX         get_module_slot3(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        my_char_pointer = get_module_slot3("A1");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Character pointer.

NOTES          This is a slot for your own usage.

get_module_type

FUNCTION       get_module_type

PURPOSE        Get the type of a module from the status table.

SYNTAX         get_module_type(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        if (get_module_type("A1") == LAMP)

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        1 = Wall switch, dimming

2 = Plug-in lamp module

3 = Appliance module

4 = Controller

NOTES          The entire status table defaults all the module types to

0, which identifies that there is no module at this

address.

get_x10cmd

FUNCTION       get_x10cmd

PURPOSE        Gets the next waiting X10 command from the receive

buffer.

SYNTAX         get_x10cmd()

PARAMETERS     None

EXAMPLE        get_x10cmd();

if (x10cmd_is("A1", OFF))

do_something();

or

if (get_x10cmd())

chk_ctrler_events();

ASSUMES        Nothing

SIDE EFFECTS   Stuffs the global variable X10Cmd with the next waiting

X10 command.  If there is no command waiting then it

stuffs a null into X10Cmd.unit and X10Cmd.funct.

RETURNS        TRUE if a command came in else FALSE.

NOTES          This function will also update the status of the

received address depending on what command was received:

"A2", ON.

The address part of the command, ie: "A1" is located in

the X10Cmd structure as X10Cmd.unit.

The function part is located in the X10Cmd structure as

X10Cmd.funct.

The received function can be one of these possibilities:

ALL_LIGHTS_OFF           HAIL_ACK

ALL_LIGHTS_ON            HAIL_REQ

ALL_UNITS_OFF            ON

BRIGHT                   OFF

DIM                      STATUS_OFF

EXTENDED_CODE            STATUS_ON

EXTENDED_DATA

get_x10date

FUNCTION       get_x10date

PURPOSE        Provide the date in string format to be used in X10

schedules.  Also stuffs the day of the week global.

SYNTAX         get_x10date()

PARAMETERS     None

EXAMPLE        get_x10date();

ASSUMES        Nothing

SIDE EFFECTS   Stuffs the global variable X10Date with a string

representing the current date based on the computers

system date and time and is formatted as mm-dd-yyyy.

Also stuffs the global variable X10Dow with a numeric

representing the current day of the week. Sunday = 0,

Saturday = 6.

RETURNS        Nothing

NOTES          Example of X10Date values - "05-11-1993"

"11-12-1993"

Example of X10Dow values - 0   /* Sunday */

1   /* Monday */

2   /* Tuesday */

You do not need to call this function directly if you

are using the function get_x10time() as this function

will automatically update the X10Date and X10Dow

variables when appropriate.

get_x10time

FUNCTION       get_x10time

PURPOSE        Provide the current time in string format to be used in

X10 schedules.

SYNTAX         get_x10time()

PARAMETERS     None

EXAMPLE        get_x10time();

ASSUMES        Nothing

SIDE EFFECTS   Stuffs the global variable X10Time with the time based

on the 24 hour clock formatted as hh:mm

RETURNS        Nothing

NOTES          Here is how the X10Time variable looks as your program

iterates through the main loop:

If its a new minute then X10Time will show hh:mm only

for example: "05:30".

If its not a new minute then X10Time will show hh:mm:ss

for example: "05:30:10"

This has been done so that you don't have to set flags

in each if statement to show that you have already done

that set of instructions.

increment_selected_levels

FUNCTION       increment_selected_levels

PURPOSE        Add or subtract a number from the status levels of the

selected modules of a house code.

SYNTAX         increment_selected_levels(char house, int steps)

PARAMETERS     char house - House code to adjust.

int steps  - Amount to add or subtract.

EXAMPLE        set_repeat(5);

send_x10cmd('A', DIM);

increment_selected_levels('A' -5);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Adjusts the status table.

RETURNS        Nothing

NOTES          Use with care.

increment_status_level

FUNCTION       increment_status_level

PURPOSE        Add or subtract a number from the status level of a

passed X10 address.

SYNTAX         increment_status_level(char *unit_addr, int steps)

PARAMETERS     char *unit_addr - X10 address of the module.

int steps       - Amount to add or subtract.

EXAMPLE        send_x10cmd('A', 1);

set_repeat(5);

send_x10cmd('A', DIM);

increment_status_level("A1", -5);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Adjusts the status table.

RETURNS        Nothing

NOTES          Use with care.

initialize_module

FUNCTION       initialize_module

PURPOSE        Initialize an X10 module to be used by the library.

SYNTAX         initialize_module(char *unit_addr, int type, int level,

int refresh, int slot1, char *slot2, char *slot3)

PARAMETERS     char *unit_addr - X10 address of the module.

int type        - Type of module at this address.

int level       - Initial level for the module.

int refresh     - Can this module be refreshed?

int slot1       - User defined slot.

char *slot2     - User defined pointer.

char *slot3     - User defined pointer.

EXAMPLE        initialize_module("A1", LAMP, 10, NO_REFRESH, 0,

"Living room", "table lamp");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          All modules need to be initialized with this function in

order for the library of functions to work correctly.

load_status

FUNCTION       load_status

PURPOSE        Load from disk a saved status table.

SYNTAX         load_status(char *filename)

PARAMETERS     char *filename - Name of the status file to load.

EXAMPLE        if (x10cmd_is("P1", ON)) {

load_status("default.sta");

refresh_entire_system();

}

ASSUMES        You are loading a file created by save_status()

SIDE EFFECTS   None

RETURNS        TRUE if the load was successful, else FALSE.

load_sun_times

FUNCTION       load_sun_times

PURPOSE        Load global variables with times representing dawn,

sunrise, sunset and dusk.

SYNTAX         load_sun_times()

PARAMETERS     None

EXAMPLE        if (time_is("00:01"))

load_sun_times();

ASSUMES        You have previously set your map coordinates with a call

to the function set_map_coord().

SIDE EFFECTS   Loads the following global variables with times based on

the current system time:

Dawn      When it just starts to get light.

Sunrise   When the sun rises above the horizon.

Sunset    When the sun goes below the horizon.

Dusk      When its actually dark out.

RETURNS        Nothing

NOTES          The times that are generated from this function are

accurate within about 1 minute year round.  But you have

to keep in mind that actual daylight is highly dependent

on weather conditions.  For example if it is a cloudy,

rainy day then it is going to be dark long before Sunset

or Dusk.

If you are using the function get_x10time() you do not

need to call this function directly as get_x10time will

do so when appropriate.

open_tw523

FUNCTION       open_tw523

PURPOSE        Opens communications to the TW523 module and initializes

internal variables.

SYNTAX         open_tw523(char *filename)

PARAMETERS     char *filename - Name of the file you will be using to

save the outage status table to.

EXAMPLE        #define OUTAGE_FILE "outage.sta";

open_tw523(OUTAGE_FILE);

or

open_tw523("outage.sta");

ASSUMES        Nothing

SIDE EFFECTS   Will exit application if anything is wrong.

Initializes the timer for schedule usage.

If the passed file is found on disk, global variable

PowerOutage will be set to TRUE.

RETURNS        Nothing

pause

FUNCTION       pause

PURPOSE        Pause the program for a specified number of seconds.

SYNTAX         pause(int secs)

PARAMETERS     int secs - Number of seconds to pause

EXAMPLE        dim_module("A1", 5);

pause(10);

dim_module("A1", 16);

ASSUMES        Nothing

SIDE EFFECTS   Suspends program execution until seconds have elapsed

RETURNS        Nothing

NOTES          Giving a value of 0 will suspend until a key is pressed.

I encourage you to use this function for waiting as this

function will eliminate the pause if the system is

catching up from a power outage and make the catchup

that much faster.

If you need a pause of some duration I suggest that you

set a timer and then wait for that timer to expire to

execute your functions.

play_catchup

FUNCTION       play_catchup

PURPOSE        Catch the system up to where it should be since the last

saved status table.

SYNTAX         play_catchup(char *filename)

PARAMETERS     char *filename - Name of the file that holds your saved

status tables for outages.

EXAMPLE        #define OUTAGE_FILE "outage.sta";

if (PowerOutage)

play_catchup(OUTAGE_FILE);

or

if (PowerOutage)

play_catchup("outage.sta");

ASSUMES        Nothing

SIDE EFFECTS   Sets flags to have the system run thru the schedule at

an accelerated pace until it reaches the current time.

RETURNS        Nothing

NOTES          This function will only work correctly if the power

outage was under 24 hours.

pop_x10cmd

FUNCTION       pop_x10cmd

PURPOSE        Removes X10 commands from the circular buffer.

SYNTAX         pop_x10cmd()

PARAMETERS     None

EXAMPLE        house = pop_x10cmd();

funct = pop_x10cmd();

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Integer representing the next command or -1 if buffer is

empty with no commands waiting.

push_x10cmd

FUNCTION       push_x10cmd

PURPOSE        Adds a command to the end of the circular buffer.

SYNTAX         push_x10cmd(int cmd)

PARAMETERS     int cmd - command to add to the end of the buffer.

EXAMPLE        push_x10cmd(6);       /* house code  A  */

push_x10cmd(6);       /* unit number 1  */

push_x10cmd(6);       /* house code  A  */

push_x10cmd(20);      /* funct code  On */

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          When you push commands into the buffer you need to use

the real numbering system for X10 commands.  Use your

reference to your TW523 for those codes.

refresh_all_modules

FUNCTION       refresh_all_modules

PURPOSE        Refresh all modules in a house code to the levels that

are shown in the status table.

SYNTAX         refresh_all_modules(char house)

PARAMETERS     char house - House code to refresh.

EXAMPLE        refresh_all_modules('A');

refresh_all_modules('D');

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will do what is necessary to set the modules to the

level that is shown in the internal status table.

RETURNS        Nothing

NOTES          You send a character rather than a string for the house

code, 'A' and not "A".

Will only refresh modules that have been originally set

with the function initialize_module().

Sequence of events are as follows:

Start with first module, ie: A1.

If the module type has been set.

Refresh individual module.

Go to the next module.

Repeat type check and refresh.

refresh_entire_system

FUNCTION       refresh_entire_system

PURPOSE        Refreshes all the modules in the entire system to the

levels that are shown in the status table.

SYNTAX         refresh_entire_system()

PARAMETERS     None

EXAMPLE        refresh_entire_system();

ASSUMES        Nothing

SIDE EFFECTS   All modules will be brought up to the level that is

shown in the internal status table.

RETURNS        Nothing

NOTES          Will only refresh modules that have been originally set

with the function initialize_module().

Sequence of events are as follows:

Start at the first house code, ie: A.

Turn all units off for that house code.

Start with the first module for that house code.

If the module type has been set.

Refresh the module.

Repeat module check for the next module & refresh.

Repeat for the next house code.

refresh_group

FUNCTION       refresh_group

PURPOSE        Refresh a group of modules to the levels that are shown

in the status table.

SYNTAX         refresh_group(GROUP group)

PARAMETERS     GROUP group - Group structure to refresh.

EXAMPLE        GROUP living_room;

add_to_group(&living_room, "L1", "L3", "F1", NULL);

refresh_group(living_room);

ASSUMES        Nothing

SIDE EFFECTS   Will do what is necessary to set the modules to the

level that is shown in the internal status table.

RETURNS        Nothing

NOTES          You do not pass address of the variable for this

function.  In other words DO NOT precede the variable

with the & operator.

Will only refresh modules that have been originally set

with the function initialize_module().

The function will act on the group in the same order

that you added the modules to the group.

Sequence of events are as follows:

Start with the first module in the group.

Refresh the module.

Go to the next module in the group.

Repeat refresh.

refresh_module

FUNCTION       refresh_module

PURPOSE        Refresh a module to the level that is shown in the

status table.

SYNTAX         refresh_module(char *unit_addr)

PARAMETERS     char *unit_addr - X10 address of the module.

EXAMPLE        refresh_module("A1");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Will do what is necessary to set the module to the level

that is shown in the internal status table.

RETURNS        Nothing

refresh_selected_modules

FUNCTION       refresh_selected_modules

PURPOSE        Refresh all pre-selected modules in a house code to the

levels that are shown in the status table.

SYNTAX         refresh_selected_modules(char house)

PARAMETERS     char house - House code to refresh.

EXAMPLE        refresh_selected_modules('A');

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will do what is necessary to set the modules to the

levels that are shown in the status table.

RETURNS        Nothing

NOTES          You send a character rather than a string for the house

code, 'A' and not "A".

Will only refresh modules that have been originally set

with the function initialize_module().

Sequence of events are as follows:

Start with the first module in the house code.

If it shows selected and module type has been set.

Refresh module.

Go to the next module.

Repeat if selected and type.

save_status

FUNCTION       save_status

PURPOSE        Saves the current status table to a file.

SYNTAX         save_status(char *filename)

PARAMETERS     char *filename - Name of the file for the saved status

table.

EXAMPLE        #define OUTAGE_FILE "outage.sta";

if (timer_expired(0)) {

save_status(OUTAGE_FILE);

start_timer(0, 60);

}

ASSUMES        Nothing

SIDE EFFECTS   The previous file, if it exists, will be overwritten.

RETURNS        TRUE if successful, else FALSE.

select_module

FUNCTION       select_module

PURPOSE        Sends out an X10 select command and wakes the module

up so it can respond to action commands.

SYNTAX         select_module(char *unit_addr, ...)

PARAMETERS     char *unit_addr - X10 address of the module.

...             - More addresses followed by a NULL.

EXAMPLE        select_module("A1", "A2", NULL);

turn_selected_modules('A', ON);

or

selecte_module("A1", NULL);

turn_selected_modules('A', ON);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Unpredictable if not terminated by a NULL.

RETURNS        Nothing

NOTES          When you send a select, the module responds by listening

to the line for commands to act on.  You can select

multiple modules and send commands to all of then in one

step.  All the modules you have selected will keep

responding to commands until another select is sent.

You can have selected modules in different house codes

and they will respond only to commands sent to that

house code.  For example you could have modules 1, 2,

and 3 selected in house code A, and modules 1, 3 and 4

selected in house code B.

Now keep in mind that other functions send select

commands out also, like the turn_module function.

send_x10cmd

FUNCTION       send_x10cmd

PURPOSE        Send out a raw X10 command thru the TW523 module.

SYNTAX         send_x10cmd(char house, int cmd)

PARAMETERS     char house - House code to send the command.

int cmd    - Command to send.

EXAMPLE        send_x10cmd('A', 1);

send_x10cmd('A', ON);

set_repeat(5);

send_x10cmd('A', DIM);

ASSUMES        House is within range of A-P and Command is within the

range 1-32.

SIDE EFFECTS   Sends command directly out the TW523 without updating

the status table.

RETURNS        Nothing

NOTES          If you use this command it is your responsibility to

keep the status table updated.

set_all_status_levels

FUNCTION       set_all_status_levels

PURPOSE        Set the level in the status table of all the modules

that are within a certain house code.

SYNTAX         set_all_status_levels(char house, int level)

PARAMETERS     char house - House code to set.

int level  - Actual level to set.

EXAMPLE        set_all_status_levels('L', 99);

set_all_status_levels('A', 0);

set_all_status_levels('A', 5);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          Not intended to be called unless you are creating your

own low level functions.  Status table can get out of

wack easily with this one.  Be careful!

You send a character rather than a string for the house

code, 'A' and not "A".

Giving a level of 99 tells the system the modules were

turned on and to give the module level a max value if it

was not already on.

*******************************************************

This is not a command that you would normally use.  It

has been included for those advanced programmers that

need to have access to the status table.

Be really careful with this as you can screw things up

in a big way by using this command incorrectly.

set_all_status_refresh

FUNCTION       set_all_status_refresh

PURPOSE        Set the refresh status of all modules within a house

code.

SYNTAX         set_all_status_refresh(char house, int refresh)

PARAMETERS     char house  - House code of the modules.

int refresh - TRUE to allow refreshing, or FALSE.

EXAMPLE        set_all_status_refresh('L', FALSE);

set_all_status_refresh('A', TRUE);

or

set_all_status_refresh('L', NO_REFRESH);

set_all_status_refresh('A', REFRESH);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will set a flag in the status table.

RETURNS        Nothing

set_all_status_selects

FUNCTION       set_all_status_selects

PURPOSE        Set the select flag of all the modules that are within a

certain house code.

SYNTAX         set_all_status_selects(char house, int select)

PARAMETERS     char house - House code to set.

int select - 0 to deselect, 1 to select.

EXAMPLE        set_all_status_selects('L', TRUE);

set_all_status_selects('A', FALSE);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          Not intended to be called unless you are creating your

own low level functions.  Status table can get out of

wack easily with this one.  Be careful!

You send a character rather than a string for the house

code, 'A' and not "A".

set_all_status_types

FUNCTION       set_all_status_types

PURPOSE        Set the type of all the modules that are within a

certain house code.

SYNTAX         set_all_status_types(char house, int mod_type)

PARAMETERS     char house   - House code to set.

int mod_type - Type of modules

EXAMPLE        set_all_status_types('L', LAMP);

set_all_status_types('A', APPL);

set_all_status_types('S', SWITCH);

set_all_status_types('C', CTRLER);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          You send a caracter rather than a string for the house

code, 'A' not "A".

Use the defined constants of:

LAMP   - Plug-in lamp modules

APPL   - Appliance or relay modules, on/off only

SWITCH - Incandescent wall switches

CTRLER - Controllers

set_daylight_time

FUNCTION       set_daylight_time

PURPOSE        Adjusts the computer time to reflect daylight savings

time.  Adds 1 hour to the current time.

SYNTAX         set_daylight_time()

PARAMETERS     None

EXAMPLE        if (date_is("04-04") && time_is("02:00"))

set_daylight_time();

ASSUMES        You have previously called the function set_map_coord()

to set internal variables and are using the global

variable X10Date.

SIDE EFFECTS   Sets the computer clock ahead 1 hour

RETURNS        Nothing

NOTES          If you are using the function get_x10time() you do not

need to call this function directly as get_x10time will

adjust the time when appropriate.

set_display_funct

FUNCTION       set_display_funct

PURPOSE        Set the function the library uses to display the

transmit or receive information.

SYNTAX         set_display_funct(int type, void (*funct)())

PARAMETERS     int type        - Type of function, TRANSMIT or RECEIVE.

void (*funct)() - Pointer to your function.

EXAMPLE        set_display_funct(TRANSMIT, my_transmit_funct);

set_display_funct(RECEIVE, my_receive_funct);

ASSUMES        Everything

SIDE EFFECTS   Will call your function after every transmit or receive.

RETURNS        Nothing

NOTES          Your functions that you create should be prototyped as:

void my_funct(char *unit, char *cmd,

char *time, char *date)

set_map_coord

FUNCTION       set_map_coord

PURPOSE        Tells the system what map coordinates you are at for

calculating sunrise/sunset times.

SYNTAX         set_map_coord(float lng, float lat, float tz, char *sdt

char *edt)

PARAMETERS     float lng - Longitude location of your city.

float lat - Latitude location of your city.

float tz  - Time Zone your city.

char *sdt - Start month-day of daylight savings.

char *edt - End month-day of daylight savings.

EXAMPLE        set_map_coord(122.2, 47.37, 8, "04-04", "10-25");

ASSUMES        All parameters are valid

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          Use the following time zones for your Tz location:

Pacific = 8, Mountain = 7, Central = 6, Eastern = 5

Sdt and Edt should be formatted as Month-Day string:

"04-04" and "10-25"

set_repeat

FUNCTION       set_repeat

PURPOSE        Sets the number of times the next command is to be sent

out.  The default is 2, which is what X10 recommends.

SYNTAX         set_repeat(int times)

PARAMETERS     int times - Number of times to repeat the next command

EXAMPLE        send_x10cmd('A', 1);

set_repeat(4);

send_x10cmd('A', DimIt);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        If you send a value of 0 it will return the current

value of the repeat variable.

set_repeat_default

FUNCTION       set_repeat_default

PURPOSE        Sets the default number of times the next command is to

be sent out.  The default is 2, which is what X10

recommends.

SYNTAX         set_repeat_default(int times)

PARAMETERS     int times - Number of times to set the default.

EXAMPLE        set_repeat_default(1);

ASSUMES        Nothing

SIDE EFFECTS   Will also set the repeat variable for the next send

command.

RETURNS        If you send a value of 0 it will return the current

value of the repeat variable.

NOTES          We have done extensive testing and have found out that

X10 modules will respond to only 1 set of commands sent

out.  So if you want to increase response time for your

system try setting the default repeat count to 1 and see

if it works fine for your house.  If commands seen to

get lost then go back to 2.

set_selected_status_levels

FUNCTION       set_selected_status_levels

PURPOSE        Set the level in the status table of the pre-selected

modules for a certain house code.

SYNTAX         set_selected_status_levels(char house, int level)

PARAMETERS     char house - House code of to set.

int steps  - Level to set.

EXAMPLE        set_selected_status_levels('A', 5);

set_selected_status_levels('B', 99);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Updates the level of all of the modules that the status

table shows are selected for the particular house code.

RETURNS        Nothing

NOTES          You send a character rather than a string for the house

code, 'A' and not "A".

Giving steps a value of 99 tells the system the modules

were turned on and to give the module level a max value

if it was not already on.

*******************************************************

This is not a command that you would normally use.  It

has been included for those advanced programmers that

need to have access to the status table.

Be really careful with this as you can screw things up

in a big way by using this command incorrectly.

set_standard_time

FUNCTION       set_standard_time

PURPOSE        Adjusts the computer time to reflect standard time.

Subtracts 1 hour from the current time.

SYNTAX         set_standard_time()

PARAMETERS     None

EXAMPLE        if (date_is("10-25") && time_is("02:00"))

set_standard_time();

ASSUMES        You have previously called the function set_map_coord()

to set internal variables and are using the global

variable X10Date.

SIDE EFFECTS   Sets the computer clock back 1 hour

RETURNS        Nothing

NOTES          If you are using the function get_x10time() you do not

need to call this function directly as get_x10time will

change the time when appropriate.

set_status_level

FUNCTION       set_status_level

PURPOSE        Set the level in the status table for a module.

SYNTAX         set_status_level(char *unit_addr, int level)

PARAMETERS     char *unit_addr - X10 address of the module.

int level       - Level to set

EXAMPLE        set_status_level("A1", 5);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Updates the level in the status table.

RETURNS        Nothing

NOTES          Giving steps a value of 99 tells the system the module

was turned on and to give the module level a max value

if it was not already on.

*******************************************************

This is not a command that you would normally use.  It

has been included for those advanced programmers that

need to have access to the status table.

Be really careful with this as you can screw things up

in a big way by using this command incorrectly.

set_status_refresh

FUNCTION       set_status_refresh

PURPOSE        Set the refresh status of the passed X10 module address.

SYNTAX         set_status_refresh(char *unit_addr, int refresh)

PARAMETERS     char *unit_addr - X10 address of the module

int refresh     - TRUE to allow refreshing, else FALSE.

EXAMPLE        set_status_refresh("A1", TRUE);

set_status_refresh("A1", FALSE);

or

set_status_refresh("A1", REFRESH);

set_status_refresh("A1", NO_REFRESH);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Sets a flag in the status table for this module.

RETURNS        Nothing

set_status_select

FUNCTION       set_status_select

PURPOSE        Set the selected flag in the status table for a module.

SYNTAX         set_status_select(char *unit_addr, int select)

PARAMETERS     char *unit_addr - X10 address of the module.

int select      - TRUE or FALSE

EXAMPLE        set_status_select("A1", TRUE);

set_status_select("A2", FALSE);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   Sets a flag in the status table for the module.

RETURNS        Nothing

set_status_slot1

FUNCTION       set_status_slot1

PURPOSE        Set the value for slot1 of a module in the status table.

SYNTAX         set_status_slot1(char *unit_addr, int value)

PARAMETERS     char *unit_addr - X10 address of the module.

int value       - Integer value to place in the slot.

EXAMPLE        set_status_slot1("A1", 5);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

set_status_slot2

FUNCTION       set_status_slot2

PURPOSE        Set the value for slot2 of a module in the status table.

SYNTAX         set_status_slot2(char *unit_addr, char *value)

PARAMETERS     char *unit_addr - X10 address of the module.

char *value     - Character pointer.

EXAMPLE        set_status_slot2("A1", "Living room");

set_status_slot3("A1", "coffee table lamp");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

set_status_slot3

FUNCTION       set_status_slot3

PURPOSE        Set the value for slot3 of a module in the status table.

SYNTAX         set_status_slot3(char *unit_addr, char *value)

PARAMETERS     char *unit_addr - X10 address of the module.

char *value     - Character pointer.

EXAMPLE        set_status_slot2("A1", "Living room");

set_status_slot3("A1", "coffee table lamp");

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

set_status_type

FUNCTION       set_status_type

PURPOSE        Set the type of module in the status table for a module

located at an X10 address.

SYNTAX         set_status_type(char *unit_addr, int mod_type)

PARAMETERS     char *unit_addr - X10 address of the module.

int mod_type    - Type of module

EXAMPLE        set_status_type("A1", LAMP);

set_status_type("A4", APPL);

set_status_type("A9", SWITCH);

set_status_type("C1", CTRLER);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          Use the defined constants of:

LAMP   - Plug-in lamp modules

APPL   - Appliance or relay modules, on/off only

SWITCH - Incandescent wall switches

CTRLER - Controller

set_time

FUNCTION       set_time

PURPOSE        Set the computer time to the passed time string.

SYNTAX         set_time(char *new_time)

PARAMETERS     char *new_time - String to set the time.

EXAMPLE        set_time("04:00");

ASSUMES        Nothing

SIDE EFFECTS   Changes the computer time to the passed time.

RETURNS        Nothing

start_timer

FUNCTION       start_timer

PURPOSE        Starts a minute timer for use in schedules.

SYNTAX         start_timer(int num, char *time)

PARAMETERS     int num    - Number of the timer to set, 0-20.

char *time - Time string for how long to set, hh:mm:ss.

EXAMPLE        start_timer(1, "00:02:30");

start_timer(2, "00:10:00");

start_timer(3, "00:00:05");

ASSUMES        Timer system has been initialized

Num is within range of 0-20

Time should be formatted as hr:mm:ss,

for example: "00:25:00".

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          When you call the function open_tw523 you automatically

initialize the timer system.

time_is

FUNCTION       time_is

PURPOSE        Checks to see if the passed time matches the current

time.

SYNTAX         time_is(char *time)

PARAMETERS     char *time - String containing the time to check.

EXAMPLE        if (time_is("05:00"))

turn_module("A1", ON);

ASSUMES        You have called the function get_x10time() to set the

global variable X10Time.

SIDE EFFECTS   None

RETURNS        TRUE if the passed time matches the current time being

held in the variable X10Time else FALSE.

time_is_after

FUNCTION       time_is_after

PURPOSE        Checks to see if the passed time is after the current

time.

SYNTAX         time_is_after(char *time)

PARAMETERS     char *time - String containing the time to check.

EXAMPLE        if (x10cmd_is("A1", ON) && time_is_after("05:00"))

turn_module("A1", ON);

ASSUMES        You have called the function get_x10time() to set the

global variable X10Time.

SIDE EFFECTS   None

RETURNS        TRUE if passed time is after the current time being held

in the variable X10Time else FALSE.

time_is_before

FUNCTION       time_is_before

PURPOSE        Checks to see if the passed time is before the current

time.

SYNTAX         time_is_before(char *time)

PARAMETERS     char *time - String containing the time to check.

EXAMPLE        if (x10cmd_is("A1", OFF) && time_is_before("14:00"))

turn_module("A1", ON);

ASSUMES        You have called the function get_x10time() to set the

global variable X10Time.

SIDE EFFECTS   None

RETURNS        TRUE if passed time is before the current time being

held in the variable X10Time else FALSE.

time_is_between

FUNCTION       time_is_between

PURPOSE        Checks to see if the current time is between or equal to

the passed times.

SYNTAX         time_is_between(char *time1, char *time2)

PARAMETERS     char *time1 - String containing the 1st time to check.

char *time2 - String containing the 2nd time to check.

EXAMPLE        if (x10cmd_is("A1", OFF) &&

time_is_between(Sunrise, Sunset))

turn_module("B1", ON);

ASSUMES        You have called the function get_x10time() to set the

global variable X10Time.

SIDE EFFECTS   None

RETURNS        TRUE if passed time is between or equal the passed

times else FALSE.

time_is_not

FUNCTION       time_is_not

PURPOSE        Checks to see if the passed time does not match the

current time.

SYNTAX         time_is_not(char *time)

PARAMETERS     char *time - String containing the time to check.

EXAMPLE        if (x10cmd_is("A1", OFF) && time_is_not("19:00"))

turn_module("A1", ON);

ASSUMES        You have called the function get_x10time() to set the

global variable X10Time.

SIDE EFFECTS   None

RETURNS        TRUE if passed time does not match the current time

being held in the variable X10Time else FALSE.

timer_expired

FUNCTION       timer_expired

PURPOSE        Checks to see if a scheduled X10 timer has expired.

SYNTAX         timer_expired(int num)

PARAMETERS     int num - Number of the timer to check.

EXAMPLE        if (timer_expired(1))

turn_module("A1", ON);

ASSUMES        num is within range of 0-20

SIDE EFFECTS   Clears the timer after it has expired

RETURNS        TRUE if timer has expired else FALSE

turn_all_modules

FUNCTION       turn_all_modules

PURPOSE        Turn all modules for a house address on or off.

SYNTAX         turn_all_modules(char house, int on_off)

PARAMETERS     char house - House code to send the command.

int on_off - Turn the modules ON or OFF

EXAMPLE        turn_all_modules('A', ON);

turn_all_modules('A', OFF);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   Will not turn on appliance modules.

RETURNS        Nothing

NOTES          For the parameter on_off its more readable to use the

constants ON and OFF rather than raw commands.

You send a character rather than a string for the house

code, 'A' and not "A".

turn_group

FUNCTION       turn_group

PURPOSE        Turn a group of modules on or off.

SYNTAX         turn_group(GROUP group, int on_off)

PARAMETERS     GROUP group - Group structure to turn.

int on_off  - Turn the modules ON or OFF

EXAMPLE        GROUP living_room;

add_to_group(&living_room, "L1", "L2", "L9", NULL);

turn_group(living_room, ON);

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          For the parameter on_off its more readable to use the

constants ON and OFF rather than raw commands.

You do not pass address of the variable for this

function.  In other words DO NOT precede the variable

with the & operator.

The function will act on the group in the same order

that you added the modules to the group.

turn_module

FUNCTION       turn_module

PURPOSE        Turns a lamp or appliance module on or off.

SYNTAX         turn_module(char *unit_addr, int on_off)

PARAMETERS     char *unit_addr - X10 address of the module.

int on_off      - Turn the module ON or OFF.

EXAMPLE        turn_module("A1", ON);

ASSUMES        Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          For the parameter on_off its more readable to use the

constants ON and OFF rather than raw commands.

turn_selected_modules

FUNCTION       turn_selected_modules

PURPOSE        Turns pre-selected modules for a certain house code

(lamp or appliance) on or off.

SYNTAX         turn_selected_modules(char house, int on_off)

PARAMETERS     char house - House code to send the command.

int on_off - Turn the modules ON or OFF

EXAMPLE        select_module("A1", "A2", NULL);

turn_selected_modules('A', ON);

ASSUMES        House code is within the range of A-P.

SIDE EFFECTS   None

RETURNS        Nothing

NOTES          For the parameter on_off its more readable to use the

constants ON and OFF rather than raw commands.

You send a character rather than a string for the house

code, 'A' and not "A".

x10cmd_is

FUNCTION       x10cmd_is

PURPOSE        Checks to see if the passed parameters match the

received x10 command.

SYNTAX         x10cmd_is(char *unit_addr, int cmd)

PARAMETERS     char *unit_addr - Address of the controller, ie: "A1"

int cmd         - Type of command that was pressed.

EXAMPLE        if (x10cmd_is("A1", OFF))

turn_module("B1", ON);

ASSUMES        You are using the function get_x10cmd() to update the

global variable X10Cmd.

Unit addr is within the range of A-P and 1-16.

SIDE EFFECTS   None

RETURNS        TRUE if passed parameters match the received x10 command

else FALSE.

NOTES          The global variable X10Cmd is a structure defined as

follows:

struct {

char unit[4];

int funct;

} X10Cmd;

X10Cmd.unit contains a string identifing the controller

address that was received, ie: "A1"  or "D16".

X10Cmd.funct contains an integer identifing the function

that was pressed on the controller, ie: ON or OFF.

Here is a listing of the possible functions:

ALL_LIGHTS_OFF           HAIL_ACK

ALL_LIGHTS_ON            HAIL_REQ

ALL_UNITS_OFF            ON

BRIGHT                   OFF

DIM                      STATUS_OFF

EXTENDED_CODE            STATUS_ON

EXTENDED_DATA

x10cmd_waiting

FUNCTION       x10cmd_waiting

PURPOSE        Checks to see there are any X10 commands waiting to be

processed.

SYNTAX         x10cmd_waiting()

PARAMETERS     None

EXAMPLE        if (x10cmd_waiting())

do_something();

ASSUMES        Nothing

SIDE EFFECTS   None

RETURNS        TRUE if there is a command waiting else FALSE.

NOTES          Don't really need to use this function directly as

get_x10cmd() takes care of whether there is a command

waiting or not.