X-10 Professional Developers Library

For

C

=========================================

|                                       |

|               HINTS.DOC               |

|                                       |

=========================================

Version 3.0

Manual and Software Copyright 1993

Smart Home Technology

24123 94th Ave. S.

Kent,  WA  98031

(206) 813-8410

ALL RIGHTS RESERVED

QUESTIONS, ANSWERS, AND HINTS

Q. I did the experiment with the lamp module and wall switch module that

you suggested, yet your library did not dim them both to the same

level either.  Why not?

A. In order for the library to know the differences in the modules you

have to identify the addresses of the plug-in lamp modules.  You need

to identify these modules in the status table either by a call to the

function initialize_module() or set_status_type().

Q. When I dim a lamp with a controller that has the same address as the

module, the library does not seem to keep track of its level.  What's

wrong?

A. Nothing is wrong.  Dim commands from a controller come out in a

continuous stream, yet the TW523 interface module was designed only to

tell the computer about every third dim command in a continuous

stream.  So if you hold the controller dim button down continuously

then the library will only recognize every third command and adjust

its values as such.  One way to get around this is to not hold the dim

button down continuously.  If you press it once it sends out one dim

command.  Press it again it sends out another. Press it again it sends

out another.  Do this until the lamp is at the level that you want. If

you do it this way the library can accurately keep track of the level.

H. When you want to dim a lamp from a controller press the on button for

that particular lamp first and then press the dim button repeatedly

until the level is reached.  This will make sure that the lamp you

want is really the currently selected one.  If you don't do this funny

things might happen since you really do not know what modules are

waiting for that dim command.  So play it safe and always press the on

button first.

H. Try to use controllers to turn things on and off.  It takes a little

while to get in the habit of doing this.  But modules themselves do

not send commands out telling what is happening to them.  So if you

turn on a module with its switch no commands are sent and the library

will not know what has happened.  Of course if your just going into a

room and you want the light on for a while then turn it on when you

enter and turn it off when you leave.  But if that module will be

controlled at some point then the library will need to keep track of

its status.  One way to reset things is to turn the module off from a

controller after you have finished with it.  This will accurately set

its status within the library.