X-10 Professional Developers Library

For

C

=========================================

|                                       |

|              TUTORIAL.DOC             |

|                                       |

=========================================

Version 3.0

Manual and Software Copyright 1993

Smart Home Technology

24123 94th Ave. S.

Kent,  WA  98031

(206) 813-8410

ALL RIGHTS RESERVED

INSTALLATION OF THE X10 PDL

What is required to use the X10 PDL?

Any IBM PC compatible computer with a serial port.

MS-DOS 3.0 or later.

A TW523 interface module.

A serial port interface adapter.

A Turbo C / Borland C / Quick C / Microsoft C compiler.

How do I install the X10 PDL?

1. Unzip the library into a temporary directory of your choosing.

ie: A:X10_C3 C:\TEMP

2. Copy the header (*.h) files to your include directory.

3. Copy the library (*.lib) files to your library directory.

4. Copy the examples (*.c & *.prj) to your working directory.

5. I suggest that you print and study the following files:

QREF.DOC        Quick reference to X10 PDL commands.

X10_TC.H        Include file for Turbo C.

X10_QC.H        Include file for Quick C.

X10_SHEL.C      A shell of a program for you to use.

X10_EXMP.C      An example of a real life working program.

How do I use the X10 PDL?

I suggest that you use the X10_SHEL.C source code as the starting

framework for all of your programs.  The X10_SHEL.C program has

all of the required initialization and setup sequences that are

needed to use this system.  At least study this program before

creating your own custom version.

AN X10 PDL TUTORIAL

STEP 1    Make a listing of all of the addresses of your X-10

controllers and modules.  When you make this listing identify

the modules with the following types:

SWITCH - Wall switch that includes dimming.

LAMP   - Plug-in lamp module.

APPL   - Appliance module or switch, ON/OFF only.

CTRLER - Controller.

Example of a listing for your modules might be:

Location                      Addr        Type

Living room table lamp         A1         LAMP

Living room ovhd lights        A2         SWITCH

Living room flourescents       A3         APPL

Kitchen coffee pot             D1         APPL

Kitchen dishwasher             D2         APPL

Example of a listing for your controllers might be:

Location                      Addr        Type

Living room lamp table        B1 - 4    Sundowner 4B

Living room wireless          C1 - 8    Hand held 8B

STEP 2    Locate the longitude and latitude for your city.  You can get

this from a local map or by calling your local weather

service.  You also will need to locate the dates that

daylight savings time starts and stops.  Again the easiest

way to get this info is from your local weather service.

STEP 3    Start your C compiler Integrated Development Environment,

IDE, as you normally would.  Open a new file from within your

IDE and select the file X10_SHEL.C. So that you don't change

the original file X10_SHEL.C, you should immediately change

the name of the file to a name of your choosing.  Maybe

X10_TUT.C.

STEP 4    First thing we need to do is let the system know some things

at start up.  Go to line #26 and change the include file to

the one you need to use. Either X10_TC.H or X10_QC.H.

Continue on down to about line #207 where we need to

initialize all the modules.  All modules need to be

identified to the library with a call to the function

initialize_module().  Add as many initialize_module() lines

as necessary. You don't need to initialize controllers.

STEP 5    Cursor down a couple more lines to one that looks like this:

set_map_coord(122.2, 47.37, 8, "04-04", "10-31");

This is the function to let the library know in what part of

the world it is operating in and the information it needs to

calculate sunrise and sunset times.  This set of coordinates

are for Seattle, WA.  Longitude of 122.2, latitude of 47.37,

we are in the pacific time zone which is #8, daylight savings

starts on April 4th and ends on October 31st.  Now replace

all the parameters with the specifics for your city.

STEP 6    Now cursor down about 12 more lines which is where we will

initialize all of your modules to a certain position.  I

suggest that all modules be turned to the off state.  The

lines you are looking for look like:

turn_all_modules('D', OFF);

turn_all_modules('E', OFF);

turn_all_modules('F', OFF);

Substitute your house codes where you have modules for the

ones that are here.  Add additional lines as needed.

STEP 7    Now we are ready to work on your schedule.  So lets go back

up to about line #116 which should deal with time of day

events.  The line we are looking for looks like this:

if (time_is("00:01")) {

We will be inserting some new lines before this one so cursor

up to the blank line before this one.

Insert the following block of text, replacing the time and

module address with your own choosing:

if (time_is("07:00"))

turn_module("A1", ON);

This block will is pretty self explanatory, if its 7 a.m.

then turn module A1 On.

STEP 8    Now lets go deal with received X10 commands.  Cursor on down

a few more lines to one that looks like this:

if (x10cmd_is("C1", ON)) {

Replace "C1" with one of your controller addresses then add

some functions you would like to happen when that controller

button is pressed on.

For example you might want to have something like this

happen:

if (x10cmd_is("C1", ON)) {

turn_all_modules('O', ON);    /* outside lights on   */

dim_module("A1", 8);          /* table lamp to lvl 8 */

start_timer(1, "00:10:00");   /* start 10 min timer */

}                                  /* to bring A1 to full*/

/* brightness          */

Again substitute your addresses for ours.  As you can see

this block of text should be self explanatory, if someone

presses C1 On then turn all modules with house code of O on,

dim the module at address A1 to level 8, and start a 10

minute timer.  Now we need to identify what we want to do

when the timer expires.

STEP 9    Cursor down a few more lines to the one that looks like:

if (timer_expired(0)) {

Now go to the blank line before as we are going to be adding

a new timer block for timer #1 that we set previously.  Enter

the following text:

if (timer_expired(1)) {

flash_module("A1", 5);        /* just for fun flash   */

dim_module("A1", 30);         /* bring to full bright */

}

Again this should be self explanatory, if timer #1 has

expired then flash the module at address A1 five times and

then bring the light to full brightness.

Notice that after the module flashes the library will reset

the module to the status it was before the flash, in this

case to level 8.

STEP 10   Save your program.  Before we go further you need to find out

what your C compiler is defaulting its memory model to.  Lets

assume that it is defaulting to the medium model.

Depending on your compiler you need to create a project file

that might look like this one, which is for Turbo C:

X10_TUT.C (X10_TC.H)

X10_TCM.LIB

You can now compile to an executable, exit your C compiler

and run your simple schedule from DOS.

Hopefully this gives you a little idea of how easy it really

is to create a schedule.  All you need to do is expand on

what we just went thru.  Take a look at X10_EXMP.C for an

example of a real schedule.  Any problems.... Call us!