HP ScanJet 6200C/6250C Series Scanners

HP PrecisionScan Pro version 1.0 scanning software

This is the README file for HP ScanJet 6200C/6250C scanners, HP PrecisionScan Pro scanning software, and other related software programs. It contains information not described elsewhere in the product.

Contents

1. 	Getting Web Updates

Change in Support Phone Numbers

2. 	HP ScanJet 6200C/6250C Scanner Setup

A. 	Recommended minimum system

B.	Minimum supported system

C. 	Autolaunching the CD

D.	Troubleshooting installation problems

E.	Things you can remove to free up more disk space

F.	Uninstalling the HP PrecisionScan Pro scanning software from your PC

G.      Scanner Lamp Warmup (Slow scan times)

3. 	HP PrecisionScan Pro Software

A.	Using a scanned image as wallpaper for your PC

B.	Grayed out tools for Output Types Text and Black & White Scalable (vector)

C. 	Click and Select (accuracy of Selection Area, finding images on scanner glass)

D. 	Printing Issues

E.	Unlimited Enhanced Resolution

F. 	OCR (scanning text so it can be edited)

G.	"Show Me" videos

H.	"Smart Friend" messages (enabling, disabling)

I.	ADF (Automatic Document Feeder)

J.	Lead Tools copyright notice

K.      Scanning from Microsoft applications using TWAIN

L.      Edges of scans get "clipped"

M.	Disabling Automatic Color Adjustment

4.	HP Utility Programs

A. 	HP Copy Utility

B.	Sharing a scanner (HP PrecisionScan LAN)

C.	HP ScanJet Button Manager

5.	Other Application Notes

A.	Scanned image is a different size in another application

B.	Using compressed tiff images

C.	Adobe Applications

D. 	Caere Applications

E.	Corel Applications

------------------------------------------------------------------------------------------------------------------------

1. Getting Web Updates

The Hewlett-Packard Customer Care web site provides the latest information regarding the use of your HP ScanJet    scanner and accompanying software.    At this web site you will find the latest software & driver downloads, answers to frequently asked questions, and support documents. The web site is located at the following URL:

http://www.hp.com/cposupport/eschome.html

You may also want to visit the Hewlett-Packard ScanJet Scanners web site.    This site has information about the latest ScanJet scanner models, as well as the Possibilities Workshop where you will find projects, "how-to" instructions, tutorials, and tips for using your new scanner.    Use the following URL to get to this site:

http://www.scanjet.hp.com/

Change in Support Phone Numbers

To request software drivers or a CD, call the following numbers based on the current date.

From 9/98 - 10/98, call:

phone:	 970 - 339-7009 or 303 - 739-4009

fax: 	 970 - 330-7655 or 303 - 739-4143

after 10/98, these numbers will refer you to the 805 area code numbers.

From 10/98 - 2/12/98, call:

phone:    	805 - 257-5565

fax:              805 - 257-6995

Starting 2/13/99, call:

phone:	661 - 257-5565

fax:              661 - 257-6995

2. 	HP ScanJet Scanner Setup

A.	Recommended minimum system:

Pentium, 100 MHz

32MB RAM

200MB available hard disk space

10x CD-ROM

800x600 video, set to millions of colors (True Color, 24 bit)

Sound Card

Modem

Windows 95, Windows NT 4, or Windows 98 operating system

USB or SCSI port

B.	Minimum supported system:

Pentium, 60MHz

16MB RAM

40MB available hard disk space

2x CD-ROM

640x480 video, set to 256 colors

Windows 95, Windows NT 4, or Windows 98 operating system

USB or SCSI port

C. 	Autolaunching the CD

After you install the HP PrecisionScan Pro software, the installation program no longer runs automatically when you place the HP PrecisionScan Pro CD in the CD-ROM drive. (This allows you to run the Show Me videos without having the install program start.)

D.	Troubleshooting installation problems

If you installed the HP ScanJet Scanner and the HP PrecisionScan Pro software but you are unable to scan, try these steps:

1.	Make sure the scanner has electrical power. To test this, disconnect the power cord from the scanner or from the electrical outlet. Then, plug in the power cord once again. The scanner lamp should come on. After you plug in the power cord, lift the scanner document lid immediately to verify the lamp is on.    Note: the scanner does not have an on/off switch.

2.	Make sure the cable connecting the scanner to the PC is firmly connected on both ends. To check, press each end of the cable firmly into its connector. If you are using USB, make sure that you are using the USB cable shipped with the scanner. HP cannot ensure the quality standards of other USB cables.

3.      Check the driver installation.

Win 95 and Win 98 Systems

a)	Click on Start -> Settings -> Control Panel, and choose System.

b)	Click on the Device Manager tab.

c)	Check the following entries in the list:

- Controllers: USB controllers or SCSI controllers should list your USB or SCSI card.

- HP ScanJet Scanners: Double-click to display the scanner model number.

d)	If either entry is missing, or if the entries are marked as conflicts (yellow circles and

exclamation points), uninstall the controller drivers or HP PrecisionScan Pro software

and reinstall it.

Win NT 4.0 Systems

a)	Click on Start -> Settings -> Control Panel.

b)      Click on SCSI Adapters. Windows displays the dialog for the adapters.

c)	Check the following entries in the list:

- Controllers: SCSI controllers should list your SCSI card.

- HP ScanJet Scanners. Double-click the adapter to display the scanner model

number.

d)      If either entry is missing, or if the entries are marked as conflicts (yellow circles and

exclamation points), uninstall the controller drivers or HP PrecisionScan Pro software

and reinstall it.

4.    Make sure the HP PrecisionScan Pro scanning software is installed. If you used the defaults in the installation program, the software should be installed in the following directory: Start -> Programs -> HP ScanJet Software -> HP PrecisionScan Pro.

If you entered a different location for the software, check the program group where you installed the HP PrecisionScan Pro scanning software.    If the software is not installed, run the Installation Program and install the HP PrecisionScan Pro software. (To run the installation program, run the Setup.exe on the HP Scanning Software CD).

5.    If you are using a SCSI connection, power down your PC and scanner. Then power up first the scanner and then the PC.

E.	Things you can remove to free up more disk space

If you have limited disk space, load only the HP scanning software.

- choose Custom install, and de-select programs such as Adobe PhotoDeluxe, or Caere

PageKeeper.

- during the scanning software installation, Cancel the Acrobat Reader installation. (NOTE:

you will not be able to display the HP PrecisionScan Pro User's Guide.)

F.	Removing the HP Scanning Software from your computer

You can run UNINSTAL.EXE to remove the HP Scanning Software from your computer.

It is located in

Start -> Programs -> HP ScanJet Software -> HP ScanJet Utilities -> HP ScanJet Uninstall

NOTES ABOUT UNINSTALL

*          Be sure the software is in the same directory where you originally installed it.    If you have moved the software to a different directory or drive, uninstall will not work properly.

*	The contents of the SCANJET directory will be deleted.    Only files that were part of your original install are deleted. Any files that you saved into these directories will be retained.

*	Nothing that was installed into the system directory will be removed.    This is because other programs may be using files that were installed here.    This includes files such as the TWAIN DLLs and the scanner driver files.

*	The registry entries for the HP PrecisionScan Pro software will be removed from your system settings.

*	The program group will be removed from the Start -> Programs menu.

* 	The electronic registration files will be removed.

*          If you want to uninstall other software (eg. Photodeluxe or PageKeeper) shipped with the scanner, you need to run their uninstall programs separately.

G.      Scanner Lamp Warmup (Slow scan times)

* If the scanner is taking a long time to start, you can change the lamp setting in the

Properties dialog. NOTE: You cannot change the Properties Lamp setting when the

scanner is in use.

3. 	HP PrecisionScan Pro Software

A. 	Using a scanned image as wallpaper for your PC

HP PrecisionScan Pro can make a scanned image appear as wallpaper on your PC:

a. Preview scan (start HP PrecisionScan Pro and click on the Preview icon).

b. Make a Selection Area.

c. Right click within the Selection Area.

d. From the popup, select "Set as Wallpaper." The selection area will be scanned and set as

the wallpaper in Windows.

B.	Grayed out tools for Output Types Text and Black & White Scalable (vector)

When you choose the Output Type called Text, or the Output Type called Black & White Scalable (vector), certain tools will not be available. Color Adjustment, Exposure Adjustment, and B&W Threshold will be grayed out. This is because the settings are optimized automatically for these Output Types.

C.	Click and Select (accuracy of Selection Area, finding images on scanner glass)

When you use Click and Select, the accuracy of the Selection Area depends on the size of the Preview scan. To increase accuracy make the Preview area large. Zoom in to fill the preview area with the selection.

Using Click and Select to Find images on the scanner glass

Click and select allows you to choose a selection area by clicking on an object.    If your images are not    found correctly, try one of the following:

* If you are placing multiple items on the scanner glass and they are not detected by the software, place a white page behind the items.

* If you are scanning material printed on both sides of the paper and it bleeds through, place a black page behind the original. Magazine pages often bleed through.

D.	Printing

Color scanned images that print as black and white on a color printer:

If the resolution for a scanned image is set higher than a printer can handle, the printer may print the scanned color image as black and white. For example, some printers can print at 600 dpi for black and white images, and can only print at 300 dpi for color images. If the scanned image is set higher than 300 dpi, the printer may print in black and white. To fix this problem:

* Check the document defaults on the printer. Make sure color printing is enabled. Refer to your printer manual for more information.

* Change the printer resolution. In HP PrecisionScan Pro, click on Print Setup on the Scan menu. Select the correct printer and click on the Properties button. Click on the Advanced tab of the Document Properties dialog box. Look for the Resolution setting (usually under Graphics) and change the resolution to the proper setting for color (for color photos, 150 dpi - 200 dpi is optimal). Click on OK to close the dialog. Print from HP PrecisionScan Pro.

MORE NOTES ABOUT PRINTING

* You cannot print from HP PrecisionScan Pro when the Output Type is set to Text or Black & White Scalable (vector). When you select these output types, it is expected that you will modify the scanned image in another application before you print it. For example, Text will appear in other applications in editable form so that you can edit it before you print.

Line art (such as a business logo that is scanned as Black and White Scalable) can be resized in another application or embedded in another file before you print. If you want to print the scanned image from HP PrecisionScan Pro, set the output type to Black & White Bitmap (raster).

* As in other applications, you can not drag and drop from the Preview window to printer icons. Only files can be dragged and dropped to printer icons.

E. Unlimited Enhanced Resolution

Enhanced resolution is a method of increasing the dpi that you can get in a scanned image.      Enhanced resolution is useful when you want to scan a small original and enlarge it.

HP PrecisionScan Pro gives you unlimited enhanced resolution by allowing resolution settings from 12 to 999,999 dpi. With this feature, the only limits to the scanning resolution are your time, hard disk space, and/or the amount of data that is accepted by other applications.

However, very high resolutions take longer to scan and create much larger files. For example, to scan an area about 1/2 inch by 1/2 inch at 999,999 dpi requires about 50 minutes on a 200 MHz Pentium. The resulting .bmp file is more than 155MB and is too large to be viewed by most image editors.    This applies no matter how you use the scan, including Drag and Drop, Copy and Paste, Save to file, Print, and TWAIN. For more information about setting resolutions, refer to the online Help or the online User's Guide.

F.	OCR (scanning text so it can be edited)

The HP PrecisionScan software comes with integrated OCR (Optical Character Recognition software).    OCR converts a graphic picture of text into text that can be edited. When you scan text, the scanning software uses OCR to automatically convert the text to appear in your applications as editable text. If you use OCR frequently, you may want to consider upgrading to the Caere OmniPage Pro version of the OCR. It will improve overall results of the text conversion.    Caere OmniPage Pro will run as integrated OCR with HP PrecisionScan Pro or you can use it as a stand-alone application.

G.	"Show Me" videos

The "Show Me" entry in Help is a series of videos that teach you about key features in the scanning software. They are NOT copied onto your hard disk during installation. If you have enough disk space, you may copy the Show Me videos onto your hard disk. Create a directory on your hard drive, and copy the files from the CD drive directory HPPSPRO\MultiMedia.

The first time you run Show Me, you get a "CD Not Found" message. Type the path to the Show Me directory files into the text box on this dialog, and click OK.

H.	"Smart Friend" messages (enabling and disabling)

"Smart Friend" messages can be enabled or disabled through Online Help. To turn them on

or off in HP PrecisionScan Pro, click on Help -> Contents and Index -> Settings -> HP

PrecisionScan Pro Smart Friend.

I.	ADF (Automatic Document Feeder)

IMPORTANT: The ADF dialog appears only when you place paper in the ADF and flip the blue lever forward. To cancel out of the ADF dialog box, flip the purple lever back and/or take the paper out of the ADF.

When scanning with the ADF and using a bitmap output type (true color, grayscale, black and

white bitmap, or one of the 256 color types), each page will be placed in a separate file. When

using a text output type, all text will be put into one file. When saving as Whole Page, all

output will be put into a single file.

J.	LeadTools copyright notice

Portions hereof copyright (c) LEAD Technologies, Inc.    1991 - 1997.    ALL RIGHTS RESERVED.

K.      Scanning from Microsoft applications using TWAIN

To use TWAIN, the following applications must have the MS Photo Editor installed from the Microsoft Office CD: Word, Excel, and PowerPoint.    Custom install MS Photo Editor by selecting Office Tools and checking the MS Photo Editor. If Office is already installed, you can add MS Photo Editor in Settings -> Add/Remove programs.

L.	Edges of scans get "clipped"

Always position the edge of the image in the top right-hand corner of the scanner. The scanner may clip images placed on the bottom and left side since it scans 8.5" x 11.69" (or A size).

M.	Disabling Automatic Color Adjustment

HP PrecisionScan Pro adjusts color automatically for the image type and media (such as a printer). It makes the white background white from originals that may be slightly off-white. When the scan changes the background to pure white, other colors in the scan may change slightly as well. Disable the Automatic Color Adjustment if you want a scanned image EXACTLY like the original.

N.    If you experience performance problems with dragging and dropping large files, try transferring the image by saving a file and then opening the file in the target application.

4.	HP Utility Programs

A. 	HP ScanJet Copy Utility

Notes about the HP ScanJet Copy Utility:

* The requirements listed for the Copy Utility in its online Help are different from the

requirements for HP PrecisionScan Pro. HP PrecisionScan Pro requires that you have a Pentium processor and one of the following operating systems: Win 95, NT 4.0, or Win 98.

*	Use HP ColorSmart printers and drivers.

*	When you have a choice, print to the PCL driver rather than the PostScript driver for faster copies.

B.	Sharing a scanner (HP PrecisionScan LAN)

* To install the Share Scanner feature:

1. Check that the network services are working properly (described below)

2. Install the host by installing PrecisionScan Pro on a PC connected to the network.

Connect the scanner to this PC. Test the scanner to ensure it is installed properly.

3. Run the Share Scanner program in the HP ScanJet Utilities menu on the Start menu.

Right click on the scanner icon in the Windows tray and select Automatically Start

to run Share Scanner whenever you power on your PC.

4. Install the client on another PC connected to the network by running Remote Scanning

Installation selection on the main HP PrecisionScan Pro installation screen.

5. Run the Remote Scanning Setup program on the HP ScanJet Utilities menu on the

Start menu.    Check Use Remote Scanner. Enter the host PC name and select a

network protocol supported by the host.

* Sharing a scanner uses network services and will not work if the network services are down.

If using TCP/IP, check that the client can ping the server. Before you share the scanner over

the network, make sure file sharing and printing are enabled as follows:

Win 95 and Win 98

In the control panel, double-click the Network icon. Under the Configuration tab, click on the

File and Print Sharing button. Click both checkboxes on.

Win NT 4.0

Contact your network administrator to verify setup.

* If the PC sharing the scanner goes into "suspend mode" or is shut down during a remote

scan, both PCs may need to be rebooted. To prevent the PC sharing the scanner from

going into suspend mode, refer to your PC's manual for instructions on turning suspend

mode option OFF.

* The host can force remote sessions to disconnect by selecting "Shut Down" from the menu

displayed when you right click the Share Scanner icon on the tray of the Windows task bar.

If you do this, you must restart Share Scanner before the scanner can be used by a remote

PC.

* Using HP PrecisionScan LAN, each PC can only communicate with one scanner at a time.

HP PrecisionScan LAN does not support connecting multiple scanners to a single PC and

having different users connect to them via Share Scanner.

* If you are accessing a remote scanner, you can enter its IP address instead of the

PC name, if you wish.

* If a client is scanning on a server and the server attempts to use the scanner, a scanner not

found message will display. To determine the user using the scanner, open the Share

Scanner application (right click on the scanner icon in the Windows tray and select Open).

* To share your scanner AND be able to connect remotely to another scanner, run the Remote

install. Then run a Typical or Custom installation on top of the first installation. Do

not run a Remote install on top of a Typical or Custom installation without first uninstalling

the HP ScanJet Software uninstaller. To switch between the local scanner and remote

scanner, run the Remote Scanner Setup.

C.	HP ScanJet Button Manager

Windows 98: Use Control Panel "Scanners and Cameras" to set the Button Manager. Select

HP 6200C and click on properties. Select the Events tab to set Button Manager properties.

If you install HP PrecisionScan Pro on Windows 95 and then upgrade to Windows 98, you

must run HP PrecisionScan Pro once before the button manager will start a scan when the

button is pressed.

5.	Other Application Notes

A. 	Scanned image is a different size in another application

Some applications do not check image resolution. For example, Paint and Publisher assume the file is 100 dpi (on Win 95) or 120 dpi (on Win NT). If your image file is a different resolution, it will be displayed at a different size.

B.	Using compressed tiff images

* In Lotus Word Pro 97 and Visioneer PaperPort Deluxe 2, compressed tiff images appear          as a solid black box. Use regular tiff format.

* in Microsoft Office, you may not be able to import a compressed tiff image. Use regular tiff.

C.	Adobe Applications

1. Adobe PhotoDeluxe templates require Adobe Type Manager (ATM) and fonts.

If the templates in PhotoDeluxe do not display in the correct typeface or do not print properly, you should install the included fonts and ATM.    To install these components, launch the ATM installer from the following directory on your HP Scanning Software CD:

\ADOBE\PD\install\ATM\WIN95\instATM.EXE	for Windows 95

\ADOBE\PD\install\ATM\WIN98\instATM.EXE	for Windows 98

\ADOBE\PD\install\ATM\WINNT\Setup.EXE		for Windows NT 4.0

2. Adobe PageMill versions 2.0 and 3.0 do not support RTF files. If you try to open a file

scanned as RTF, the text and graphics will be listed as separate paragraphs and you

cannot edit the file. A complex page may stretch out to eight or 10 pages.

D. Caere Applications

* Brightness options in Caere Scan Manager (in the TWAIN Scanner Settings) have no effect when you have scanned with HP PrecisionScan Pro. HP PrecisionScan Pro automatically adjusts brightness for all scans.

* The Scanner page in the Tools/Options menu of PageKeeper provides options to be used during TWAIN sessions. Use these options when you scan from PageKeeper. If you choose B&W for the Scan Mode, set the resolution to 300 dpi to scan text.    The text will be much    more readable.    Use the default resolution of 75 dpi when you scan for screen presentations or publishing on the web.

* You cannot drag and drop to Caere OmniForm 2. The drag and drop cursor appears in the scanning software but no image appears in the application.

E.	Corel Applications

* When you drag and drop an image into Corel PhotoPaint 7 or later HP PrecisionScan Pro will scan twice. To avoid scanning twice, drag the scanned image to the PhotoPaint icon.

* When you scan using Caere PageKeeper Standard, the Cancel button will not work until the scan is complete. The Cancel button does work when PageKeeper processes the image data after the final scan.