ZCPR Version 3.4 Resident Command Package

Source Code

Revised Documentation for Version 1.1

October 7, 1989 by Carson Wilson

Version 1.0 Documentation

June 17, 1988 by Carson Wilson

Contents

--------

1.  Files in this Library.

2.  Purpose of the Resident Command Package.

2.1.  "Resident" vs. "Transient" Commands.

2.2.  The Z-System Resident Command Package.

2.2.1.  Design Philosophy of the Z34 RCP.

3.  Structure and Operation of the RCP Segment.

4.  Generating an RCP for your System.

4.1.  The Z-Relocatable Approach.

4.1.1.  NZCOM and JetLDR.

4.1.2.  Z-Relocatable (ZRL) Code

4.2.  The Assembly Approach.

4.2.1.  Note on Assemblers.

4.2.2.  Assembly to Absolute Code.

4.2.3.  Assembly to Z-Relocatable Code.

4.2.4.  Adding Custom Commands to Z34RCP.

5.  Legal Use of These Files.

1.  Files in this Library.

This library of files contains the assembler source code and

accompanying documentation for the Z-System Resident Command

Package (RCP).  To use Z34RCP you must be running ZCPR versions

3.3 or above.  For pre-assembled Z-Relocatable (ZRL) RCPs, see

RCPZRL11.LBR.  Z34RCP11.LBR consists of the following files

(uncompress with UNCR.COM, QL.COM or LBRE.COM):

CLEDINST.CZM   - Installer for CLED command.

CLEDSAVE.CZM   - Command file utility for CLED.

RCPCLED .DZC   - Documentation on CLED.

RCPBASE .LZB   - Used to assemble an absolute address RCP.

RCPCMD  .LZB   - Used to include custom commands in the RCP.

RCP?????.LZB   - Code modules for the standard RCP commands.

SYSDEF  .LZB   - Commonly used equates.

Z34CMN  .LZB   - Used to assemble Z-Relocatable RCP's.

Z34DEFN .LZB   - Offsets in the ZCPR 3.4 code.

Z34MAC  .LZB   - Assembler macros.

Z34RCP11.DZC   - This file.

Z34RCP11.FOR   - Short description of the library.

Z34RCP11.HZP   - Help file for the RCP, gives command syntax and

examples.

Z34RCP  .LZB   - Used to select features when assembling the RCP.

Z34RCP11.NZW   - Changes since Z34RCP10.LBR.  Intended for former

RCP users, this will be only of passing interest

to new users.

Z34RCP11.ZZ0   - Main file for assembling the RCP.

ZSYSTEM .AZ    - General information about Z System.

2.  Purpose of the Resident Command Package.

2.1.  "Resident" vs. "Transient" Commands.

Most of the commands used under CP/M and Z-System are "transient"

commands.  The machine code for these commands (or "programs") is

stored in "command files," whose file type is normally .COM.

Thus, the machine code for WordStar is stored in a file called

"WS.COM".

To run WordStar, the user issues the command "WS" at the command

line, the operating system loads WS.COM from the disk drive to

memory, and WS.COM takes over control of the computer.  If the

user then exits WordStar and issues another command, the

operating system can load another command file from the disk,

reusing the area of memory previously occupied by WordStar.

Recycling memory in this way makes personal computers quite

versatile, as the number of commands available to the user is

only limited to the number of command files stored on disk.

There are some drawbacks to this approach, however.  Although

disk-based commands conserve memory by recycling it, they must be

loaded from disk each time they operate.  This creates its own

costs in terms of speed of execution, disk storage space, and

memory reallocation.

The most obvious disadvantage of loading a file from disk each

time a command is issued is that it takes time to find and load

disk files.  Especially with floppy diskettes, the amount of time

which elapses after the command is issued and before the command

executes is significant.  A second drawback to disk-based

commands is that commands will not execute unless their machine

code is available on the disk system.  This means that the user

must know what command files are available on disk before issuing

commands.  The final disadvantage of disk file based commands is

that some commands (e.g., SAVE) require that the area of memory

normally overwritten by command files (known as the "Transient

Program Area" or TPA) be preserved.

In answer to these difficulties, the creators of CP/M designed

some of its commands as "built-in" commands.  The DIR, REN, SAVE,

ERA, USER, and TYPE commands of CP/M 2.2 remain in memory and can

be issued without loading machine code files from disk.  Thus,

the DIR command provides a disk directory regardless of whether

the command file "DIR.COM" is present on disk, and the SAVE

command saves the contents of memory at 100 hex to a disk file.

Residing permanently in memory, these commands are simple in

order to leave memory for disk-based commands to load in.

2.2.  The Z-System Resident Command Package.

The creators of Z-System have followed the resident command

philosophy of CP/M, extending and enhancing the capabilities of

resident commands with the Z-System Resident Command Package.

Under CP/M 2.2, resident commands are tightly integrated with the

operating system.  The user is therefore limited to a fixed set

of resident commands.  The code in the Z-System Resident Command

Package is much more loosely linked with the operating system.

The RCP occupies a section of protected memory, but how that

memory is used is only very loosely constrained.  The Z-System

resident command package may be tailored for a wide variety of

installations, and easily altered within a single installation.

2.2.1.  Design Philosophy of the Z34 RCP.

Though any set of commands which will fit in the RCP memory area

can be loaded as an RCP, a standard set of Z-System RCP commands

have evolved over the years.  Distributed as the standard

Z-System RCP, this set of commands consists of universal,

single-purpose commands modeled after the resident command set of

CP/M 2.2, such as DIR, REN, and ERA.

Keeping these commands simple both conserves memory and allows

their use in custom command scripts.  None of the standard RCP

commands require interaction on the part of the user--they

perform a single function and quit.  Aliases or other command

scripts which build complex commands from simple RCP commands

therefore retain complete control over the user interface.

However, the standard RCP forms only one of many alternative uses

to which the Z-System RCP memory segment may be put.  Since the

RCP memory is protected, it can be used by background programs

which continue to function even while other applications have

been loaded into the transient program area.  Several such

programs, including key-redefiners, screen-trapping programs,

background numeric calculators, program assembly environments,

and memory utilities have been developed.

Until Z34RCP, all but the most advanced users had to surrender

their entire RCP to use the custom commands, even if the commands

needed far less than the available RCP space.  Thus, to use a

key-redefinition RCP, one was forced to give up all of his other

RCP commands.  Z34RCP solves this problem by acting as a base to

which programmers may easily add code.  By modifying the standard

file RCPCMD.LIB, programmers can implement and distribute custom

RCP commands which may be installed by those with only basic

knowledge of assembly language.  See "Adding Custom Commands,"

below, for details.

3.  Structure and Operation of the RCP Segment.

Typically, the Resident Command Package occupies 2 kilobytes of

memory, protected above the system's Basic Input Output System

(BIOS).  Its internal structure is very simple, consisting of the

five byte string "Z3RCP", a table of command names and their

addresses within the RCP, and the machine code of the commands

themselves.  This means that an RCP of a given length can contain

any number of resident commands which will fit in 2k, from one

large program to many shorter programs.

Further, since the RCP is independent of the rest of the

operating system, the resident commands may be changed at will.

On system startup, a default RCP is loaded into memory.

Thereafter the RCP's commands remain active until the system is

turned off or another RCP is loaded.

Using loader programs to load the RCP to memory allows another

important ability--it can exchange RCPs during a single session.

By allowing users to change their RCPs "on-the-fly," loader

programs provide some of the benefits of transient commands even

while preserving the benefits of resident commands.  In fact,

even what portion of memory is allocated to the RCP can be

altered in this way by advanced users! (see the Z-System User's

Manual for more on this).

4.  Generating an RCP for your System.

Since Z80 computers come with a wide variety of operating system

environments, it is impossible for a single segment of machine

code function as a universal RCP.  Due to differences in

operating system and hardware design, the size and location of

memory available to the Z-System RCP vary widely.  Until

Z34RCP, this meant that each type of Z80 computer required an RCP

segment assembled specifically for that computer.

Now, however, there are two basic means by which you may generate

an RCP for your system.  The simplest route is to load already-

assembled Z-Relocatable code modules.  This technique is

recommended for beginners and those with little experience in

assembly language.  See RCPZRL11.LBR for pre-assembled RCP

modules.  For more experienced users with an interest in

customizing or their RCPs or adding their own commands, custom

assembly of the files in this library is also an option.

4.1.  The Z-Relocatable Approach.

4.1.1.  NZCOM and JetLDR.

LDR.COM, the familiar Z-System segment loader first introduced by

Rick Conn as a means of loading the various segments of the ZCPR

environment from disk to memory, required that system segments be

assembled to run only at system-specific addresses.  For example,

if the system's RCP started at FA00 hex, only an RCP assembled to

run at that starting address would function on the system.

Therefore, separate SYS.RCP files were needed for each computer

installation.

Thanks to several advanced loader utilities, namely NZCOM by Joe

Wright and JetLDR and Z3PLUS by Bridger Mitchell, custom-

assembling an RCP for a given target machine is no longer

necessary.  These three programs can load specially assembled RCP

segments (known as Z-ReLocatable or "ZRL" segments) to any area

of memory.  Z-System users now need only obtain previously

assembled RCP segments, much as they need only obtain assembled

.COM or .OBJ files to run transient programs.

Using information in the system's environmental descriptor,

advanced loaders load pre-assembled segments to the proper

addresses in any system.  The only remaining constraint is the

amount of memory allocated to the RCP.  While the standard RCP

size is 2 kilobytes, many Z-System installations vary

considerably from this standard, so it is still necessary to

ensure that a ZRL segment does not exceed available RCP memory.

Fortunately, this is taken care of automatically by the advanced

loader programs.

Using NZCOM or JetLDR, it is possible to load an RCP directly

from RCPZRL11.LBR.  See NZCOM or JetLDR documentation for full

instructions on how to load Z-Relocatable RCPs with these

programs.

4.1.2.  Z-Relocatable (ZRL) Code

The technique used by NZCOM and JetLDR to load a single RCP to

any address in memory involves relocatable code.  Relocatable

code has been used for years by the "linker" programs available

to assembly language programmers.  Since only portions of a given

Z80 machine code routine are specific to one memory address, it

is possible to create "relocatable" files of Z80 routines (known

by their file type .REL) containing all but the address-specific

codes.  These files can then be very quickly relocated (or

"linked") by a linker program to operate at any memory address.

Z-Relocatable code is simply a more sophisticated form of

relocatable code.  ZRL files contain separately labeled segments,

known as "named common blocks."  Where normal relocatable code is

all linked to operate at one address at a time by the linker

program, named common blocks allow the linker to distinguish

between different sections of the file and to link each "block"

to operate at a different address.

This represents a major step forwards in user convenience, and

will certainly lead to more innovative work using the ZCPR system

segments.  There is a minor drawback, however, inherent in the

form of assembly required by NZCOM and JetLDR.  Before the advent

of these two programs, system segments were equivalent to .COM

files which were assembled to operate at addresses other than the

standard 100 hex.  The ability to load a single system segment to

any address depends partially on the form of the assembled system

segment itself.

While public domain assemblers will produce absolute object

files, more sophisticated assemblers, such as SLR's Z80ASM (a

bargain at $50) are required to assemble code with named common

addresses.  NZCOM and JetLDR thus greatly reduce the time needed

by most users to install Z-System segments, but those wishing to

assemble Z-Relocatable segments will probably need a commercial

assembler.

4.2.  The Assembly Approach.

Those with knowledge of assembly language may opt to assemble

their own custom RCPs, including only the commands and options

which match their exact requirements.  Z34RCP.LBR contains the

complete source to the ZCPR 3.4 RCP.  The file Z34RCP.Z80

contains include statements to combine the various library files

during assembly.  The file Z34RCP.LIB contains all of the equates

controlling assembly time options, and should be edited before

assembling Z34RCP.Z80.

4.2.1.  Note on Assemblers.

The development work on Z34RCP was done exclusively with the SLR

Systems line of Z80 assemblers.  Hence, some of the source code

is unavoidably specific to SLR and compatible assemblers.  Those

using incompatible assemblers such as public-domain Z80

assemblers should set the SLR equate in Z34RCP.LIB to FALSE.

This removes most SLR-specific pseudo-ops, but some additional

editing of the source files may be required for error-free

assembly.  Unless your assembler can handle named common blocks,

you will also want to set the ZRL equate in Z34RCP.LIB to FALSE.

4.2.2.  Assembly to Absolute Code.

If you are assembling a custom RCP for use at one memory address

only, or if your assembler can't handle named common blocks, you

can set the ZRL equate in Z34RCP.LIB to FALSE, and instruct your

assembler to produce absolute code (either a .HEX or a .COM file).

Before generating absolute code you must set addresses in the

file RCPBASE.LIB for the particular installation.  Once you have

set these addresses, you may proceed to assemble Z34RCP.Z80 to

either Z34RCP.COM or Z34RCP.HEX.  If your assembler generated a

.HEX file, use LOAD.COM to convert Z34RCP.HEX to Z34RCP.COM.

If you are running a version of ZCPR prior to 3.4, you must now

alter two bytes in your Z-System environment segment to point to

your system's CCP address.  The bytes at offset 39 and 40 hex

(formerly containing "width of printer 2" and "lines on printer

2") should contain a 2-byte address of your system's CCP, most

significant byte last.

Z34RCP.COM may now be renamed to SYS.RCP and loaded with the Z-

System loader program LDR.COM or Bridger Mitchell's JetLDR.

4.2.3.  Assembly to Z-Relocatable Code.

Unlike absolute RCP segments, Z-Relocatable segments may be

loaded to ANY address, provided enough RCP space to contain them

is available.  To assemble a Z-Relocatable RCP, you will need an

assembler which can handle named common blocks (see 4.2.1 above).

Set the ZRL equate in Z34RCP.LIB to TRUE, and instruct your

assembler to produce a relocatable (.REL) file.  The resulting

Z34RCP.REL may then be renamed to Z34RCP.ZRL and loaded with

NZCOM, Z3PLUS, JetLDR.

4.2.4.  Adding Custom Commands to Z34RCP.

As mentioned above, I have designed Z34RCP as a base to which

custom RCP commands may readily be added.  All code and command

names for custom commands may be included in RCPCMD.LIB.

Thereafter, whenever the RCP is assembled the commands added to

RCPCMD.LIB will be included automatically.  To remove the custom

commands, just replace the modified RCPCMD.LIB with an unmodified

copy.  See RCPCMD.LIB for detailed instructions on how to add

code and command names.

RCPCMD.LIB also facilitates distribution of custom commands to be

included by others in their RCPs.  Simply distribute a copy of

your modified RCPCMD.LIB along with instructions on how to use

your custom commands.  Users may then easily include the new

commands by substituting your modified RCPCMD.LIB for their

original copy and reassembling their RCPs.

Of course, users of NZCOM, Z3PLUS, or JetLDR may simply trade

custom RCPs in Z-Relocatable form and dispense with the need for

any assembly work on the part of the user.  All three of these

advanced loaders are available from Z Systems Associates (see

below).

5.  Legal Use of These Files.

Z34RCP is copyright 1989 by Z Systems Associates (ZSA), all

rights reserved.  Any commercial use of Z34RCP, defined as any

situation where the duplicator recieves revenue by duplicating or

distributing Z34RCP by itself or in conjunction with any hardware

or software product, is expressly prohibited unless authorized in

writing by ZSA.

Except for the file RCPCMD.LIB (see 4.2.4 above), you may

redistribute Z34RCP.LBR in its present form only.  I encourage

you to explore the source code, suggest improvements, and

document errors.  However, please obtain permission from Z

Systems Associates before redistributing any of these files in

altered form.  This will prevent confusion by allowing your work

to be coordinated with the efforts of others.

The Z Systems Associates are:

Sage Microsystems East

Selling & Supporting the Best in 8-Bit Software

1435 Centre St., Newton Centre, MA 02159-2469

Voice: 617/965-3552 (9:00am - 11:30pm)

Modem: 617/965-7259 (password = DDT)(MABOS on PC-Pursuit)

and:

Plu*Perfect Systems

"==World-Class Software"

410 23rd Street, Santa Monica, CA 90402

Voice: 213/393-6105 (evenings)

Modem: 213/670-9465 (leave message for "Bridger Mitchell")

For more information on Z System, check at the above bulletin

board systems, or consult The Computer Journal for excellent

articles on ZCPR and CP/M.  The Computer Journal is published six

times a year by Publishing Consultants, 190 Sullivan Crossroad,

Columbia Falls, MT 59912, phone 406/257-9119.  An issue averages

forty pages with few ads.  Subscription rates are $16 for one

year (6 issues), or $28 for two years (12 issues) in the U.S.,

$22 for one year Canada and Mexico, and $24 (surface) for one

year in other countries.

Carson Wilson is the operator of:

Antelope Freeway Remote Access System

Chicago, 312-764-5162

300-1200-2400 Baud, 24 Hours

#################################################################################