# NCP5201 DUAL OUTPUT DDR POWER CONTROLLER SPICE MODELING

# 5.4.1		Scope

The purpose of this design is to model NCP5201 DDR and VTT Power Controller.

| Spice Design: | Dual Output DDR Power Controller |
| --- | --- |
| Performed by: | Kehinde Omolayo |
| Model Last Rev Date: | 2-20-2004 |
| ON Semi Publication Order Number: | ## NCP5201/D |
| ON Semi Revision | ## May, 2003-Rev.5 |
| SPICE File | NCP5201S.LIB (for IsSpice version) NCP5201P.LIB (for PSpice version) |

# 5.4.2		Functional Description

NCP5201 is dual power controller for DDR memory VDDQ and VTT power supplies. It provides complete power control during normal mode (S0), Standby mode (S3), and off mode (S5).

The VDDQ section uses voltage mode synchronous PWM buck topology to regulation high power DDR memory supply. The VTT is a two quadrant linear regulator: it sinks and sources 1.8Adc. If the VTT input is connected to the VDDQ output, the VTT output is directly proportional to the VDDQ output. The controller operates at 250kHz during S0 state and at 500kHz during S3 state.

NCP5201 is equipped with VCC and Standby UVLO, and overload fault protections. During transition from S5 to S0 or S3, the system is in soft start mode. The feature prevents VDDQ output voltage from overshooting due to natural response of the output filter during.

# 5.4.3		Assumptions

- The temperature for this model is 25 degrees centigrade.

- Thermal Shutdown is not included

# 5.4.4		 Methods of modeling

IsSpice4 and Pspice were used to model the device. The IsSpice net list is as follows and the Pspice version is available upon request:

* IsSpice Model Editor

* |=======================================================

* |                     NCP5201        REV - 02/20/04

* |                (IsSPICE VERSION)

* |                ON SEMICONDUCTOR

* |          DUAL OUTPUT DDR POWER CONTROLLER

* |

* | This model was developed for ON Semiconductor by:

* | OK Power Systems

* |

* |

* |

* | Copyright 2004 OK Power Systems

* | All Rights Reserved

* |

* | The content of this model is subject to change

* | without notice and may not be modified or altered

* | without permission from ON Semiconductor. This model

* | has been carefully checked and is believed to be

* | accurate, however neither OK Power Systems nor

* | ON Semiconductor assume liability for the use of

* | this model or the results obtained from using it.

* |

* | For more information regarding modeling services,

* | model libraries or simulation productors, please

* | call at (631) 654-0253 ext 116 or 106.

* |

* |=======================================================

# *Designed by Kehinde Omolayo 2-20-04

# .SUBCKT NCP5201 1 2 3 4 5 6 7 8 10 11 12 13 14 15 16 17 18

# *DUAL OUTPUT DDR POWER CONTROLLER

# *Terminal Identifications

# *FBDDQ=1 FBVTT=2 PGND=3 VSTBY=4 VTT=5 & 6 OCDDQ=7 VDDQ=8

# *PWRGD=10 S3_EN=11 AGND=12 SDDQ=13 BGDDQ=14 TGDDQ=15 VCC=16

# *COMP=17 SS=18

# .MODEL NC5201D D CJO=2P

# .MODEL NC5201D1 D CJO=2P N=0.1

# .MODEL MOS5201N NMOS Level=1 CBD=8.44P CBS=10.1P CGBO=21.6N

# + CGDO=2N CGSO=2.4N GAMMA=.893 IS=5.1N KP=5.34M LAMBDA=22M

# + MJ=.46 PB=.8 PHI=.75 RD=13 RS=16 VTO=1.1

# .MODEL MOS5201P PMOS CBD=20.5P CBS=24.6P CGBO=84N CGDO=30N

# + CGSO=36N KP=72.5E-3 RD=1 RS=0.4 VTO=-3

# .MODEL MT5201N NMOS CBD=244P CBS=293P CGBO=454N CGDO=180N

# + CGSO=216N KP=90.5m RD=0 VTO=3

# .MODEL MB5201N NMOS CBD=244P CBS=293P CGBO=454N CGDO=180N

# + CGSO=216N KP=56m RD=0.4 VTO=3

# .SUBCKT RS1500 2 3 4 5

# C1 11 0 10P

# R3 10 8 144.269

# C3 8 0 10P

# B1 10 0 V=V(2)>1 ? V(5)>1 ? 0 : 2

# B2 16 0 V=V(3)>1 ? V(4)>1 ? 0 : 2

# B5 4 0 V=V(8)>1 ? 2 : 0

# B6 5 0 V=V(11)>1 ? 2 : 0

# R1 16 11 144.269

# .ENDS

# .SUBCKT T1500 18   17   15 16

# R2 3 19 144.269

# B1 2 0 V=V(3)>1 ? 0 : 2

# C4 0 7 10P

# R4 7 4 144.269

# B2 14 0 V=V(7)>1 ? 0 : 2

# C6 0 10 10P

# R6 10 5 144.269

# B3 15 0 V=V(10)>1 ? 0 : 2

# C8 0 13 10P

# R8 13 8 144.269

# B4 16 0 V=V(13)>1 ? 0 : 2

# B5 19 0 V=V(18)<1 ? 2 : V(17)<1 ? 0 : V(2)<1 ? V(14) :

# +  V(14)>1 ? V(16)>1 ? V(14) : 0

# B6 4 0 V=V(18)<1 ? 0 : V(17)<1 ? 0 : V(14)>1 ? V(16)>1 ? 0 : V(2)

# B7 5 0 V=V(18)<1 ? 0 : V(2)<1 ? 0 : V(16)

# B8 8 0 V=V(18)<1 ? 2 : V(2)<1 ? 2 : V(14)<1 ? 0 : V(15)

# C2 0 3 10P

# .ENDS

# R1 4 12 2.5K

# X1 70 20 22 66 RS1500

# V1 19 0 PULSE 0 2

# B1 70 0 V=V(20)>1 ? V(4)>(4.25+V(12)) ? 0 : 2

# R3 11 12 20MEG

# B2 20 0 V=V(19)>1 ? V(4)>(4.048+V(12)) ? 2 : 0

# X2 54 21 23 55 RS1500

# B3 54 0 V=V(21)>1 ? V(16)>(9.23+V(12)) ? 0 : 2

# B4 21 0 V=V(19)>1 ? V(16)>(8.096+V(12)) ? 2 : 0

# R2 16 12 100K

# B5 87 0 V=V(88)>1 ? V(17)>(1.277+V(12)) ? 0 : 2

# B6 24 0 V=V(11)>(0.95+V(12)) ? 2 : 0

# B7 26 0 V=(V(22)*V(24)*V(25))>4 ? 2 :

# +(V(22)*V(24))>2 ? V(23)<1 ? 2 : 0

# B8 27 0 V=V(22)<1 ? 2 : V(23)<1 ? V(24)<1 ? 2 : 0

# B9 28 0 V=V(26)<1 ? V(27)<1 ? 2 : 0

# M1 10 82 12 12 MOS5201N

# R4 82 85 50

# B10 85 0 V=V(29)>1 ? 0 : 5

# C1 29 0 10N

# R5 29 9 28.85K

# B11 9 0 V=V(32)*V(37)

# E1 67 0 64 5 10

# E2 0 60 64 5 10

# V2 67 79

# V3 83 60

# D1 79 62 NC5201D

# D2 83 62 NC5201D

# R40 95 64 100m

# R6 64 5 1MEG

# B13 64 5 I=10*(I(V2)+I(V3))

# R7 6 3 1MEG

# E3 58 3 74 0 1

# C2 74 0 1N

# R8 74 56 10K

# B14 56 0 V=(V(8)-V(12))/(1+EXP(V(30)))

# B15 0 30 V=70*(1-EXP(V(31)))/(1+EXP(V(31)))

# R9 6 5 1U

# B16 32 0 V=ABS(V(33)-V(2))<30M ? 1 : 0

# E4 0 31 33 2 3

# E5 0 36 71 1 3

# R11 33 12 25K

# R12 8 33 25K

# B18 0 35 V=69.417*(1-EXP(V(36)))/(1+EXP(V(36)))

# R13 2 12 125MEG

# B17 8 12 I=I(V5)>0 ? I(V5) : 0

# V5 58 95

# D3 5 8 NC5201D

# D4 12 5 NC5201D

# B19 0 34 V=69.417*(1-EXP(V(35)))/(1+EXP(V(35)))

# B20 59 12 V=0.7*(V(4)-V(12))/(1+EXP(V(34)))

# R10 59 17 3.36k

# C3 17 12 120.21p

# R14 12 1 10MEG

# V4 96 12 DC=1.3

# D5 78 73 NC5201D1

# R15 93 37 10K

# D6 18 96 NC5201D

# D7 12 18 NC5201D1

# B21 18 12 I=V(27)>1 ? 1M : -2.68U

# X7 90 91 77 93 T1500

# B22 90 0 V=ABS(V(36))<57m ? V(28)>1 ? V(18)>V(96) ? V(19) : 0

# V7 38 0 PULSE 0.5 0.6389 0 3.9U 0.05U 0.05U 4U

# V8 39 0 PULSE 0.5 0.6389 0 1.95U 25N 25N 2U

# B23 40 0 V=V(38)*V(28)+V(39)*V(26)

# B24 57 0 V=V(40)<(V(17)-V(12)) ? 2 : 0

# R16 57 41 7.2K

# C4 41 0 10P

# B25 42 0 V=V(41)>1 ? 2 : 0

# D8 3 7 NC5201D1

# I1 7 3 DC=10U

# B26 61 0 V=V(13)<V(7) ? (V(15)-V(13))>1 ? 2 : 0

# R17 61 43 7.21K

# C5 43 0 100P

# X3 63 44 45 65 RS1500

# B27 63 0 V=V(27)>1 ? 0 : V(19)

# B28 44 0 V=V(63)>1 ? V(43)>1 ? 0 : 2

# B29 4 68 I=2.036*(V(4)-V(13))*V(26)*V(48)*V(42)*V(45)

# D9 13 4 NC5201D

# V9 68 13

# B36 84 0 V=V(15)<(2+V(13)) ? 2 : 0

# X5 75 69 47 48 T1500

# B30 75 0 V=V(19)>1 ? V(46)<1 ? 2 : 0

# B31 69 0 V=(V(40)*V(47))<2.2 ? 2 : 0

# B32 73 0 V=I(V9)>2 ? 2 : 0

# R18 73 46 7.21K

# C6 46 0 100P

# M3 72 52 49 49 MOS5201P

# M4 76 53 49 49 MOS5201P

# M5 72 52 3 3 MT5201N

# M6 76 53 3 3 MB5201N

# V10 72 14

# V11 76 15

# R19 14 3 50k

# E7 49 3 16 3 1

# B33 16 3 I=V(52)<(V(49)-3) ? I(V10)>0 ? I(V10) : 0

# B34 80 0 V=V(51)*V(28)*V(45)>4 ? V(42)<1 ? V(3) : V(49)

# R20 52 80 100

# B35 81 0 V=V(50)*V(28)*V(45)>4 ? V(42)>1 ? V(3) : V(49)

# R21 15 13 50k

# R22 53 81 100

# C7 0 51 10P

# R23 51 84 5K

# B37 86 0 V=V(14)<(2+V(3)) ? 2 : 0

# R24 50 86 5K

# C8 50 0 10P

# B38 16 3 I=V(53)<(V(49)-3) ? I(V11)>0 ? I(V11) : 0

# B39 4 12 I=(V(4)-V(12))*0.2m*V(26)

# E6 100 12 18 12 1

# C17 37 0 1N

# R39 78 46 100

# G1 62 0 37 0 0.115

# D10 0 62 NC5201D

# R41 96 71 1K

# B12 91 0 V=(V(40)*V(77))<2.2 ? 2 : 0

# X8 87 88 25 94 RS1500

# B41 88 0 V=V(17)<(1.25+V(12)) ? 0 : V(19)

# D11 71 100 NC5201D1

# .ENDS

# 5.4.5		 Model Tests

The model was tested with the application diagram in the data sheet. The IsSpice test fixture for PWM normal mode is shown in figure 5.4.1, and the Spice net List is shown in figure 5.4.2. The SPICE simulation waveforms for normal mode are shown in figure 5.4.3 and.

Figure 5.4.1. PWM Normal Operation Schematic

Figure 5.4.2. PWM APPLICATION Net List

.TRAN 0.1U 2.4M 2.3M UIC

.PRINT  TRAN Vout

.PRINT  TRAN VTT

.PRINT  TRAN TGDDQ

.PRINT  TRAN BGDDQ

.PRINT  TRAN PWRGD

.PRINT  TRAN SWDDQ

.PRINT  TRAN IL

.PRINT  TRAN SS

X1 14 19 0 18 19 19 7 6 15 16 0 5 9 8 10 13 20 NCP5201

X3 17 23 3 NTD60N02R

.SUBCKT NTD60N02R 1 2 3

M1 4 2 3 3 NMAIN W =285M  L =1U

M2 3 2 3 1 PAUX W =0.35 L = 1U

R1 1 4 0.5M RTEMP

CGS 2 3 0.7N

D1 3 1 DBD

.MODEL NMAIN NMOS (LEVEL=3 ETA=9.5M

+VTO=1.5 KP=0.1M

+ RS=0.05M)

.MODEL PAUX PMOS (LEVEL = 3 TOX = 3E-8  VTO=2

+NSUB=5E16)

.MODEL DBD D (IS=249N N=1.5 RS=5.83M BV=24

+CJO=1.16N VJ=0.8 M=0.42 TT=15.5N)

.MODEL RTEMP R (TC1 = 7E-3 TC2 = 6E-6)

.ENDS

X2 3 4 5 NTD60N02R

R1 5 1 10m

L1 1 6 2.2u

C1 6 2 3000u

R2 2 0 10m

R3 6 0 0.265

R4 9 22 4.7

R5 8 4 4.7

R6 17 7 62k

R7 6 14 1.15K

R8 6 11 16

C2 11 14 100n

R9 14 0 1.1K

C3 14 13 10n

R10 14 12 20k

C4 12 13 6.8n

V1 17 0 DC=5

V2 10 0 DC=12

R11 17 15 10k

V3 18 0 DC=5

R12 16 17 10k

X4 5 22 0 NTD60N02R

C5 0 20 3N

C6 19 21 470u

R13 21 0 0.10

R14 19 0 1

R15 8 23 4.7

.END

Figure 5.4.3: PWM waveforms During Normal Mode @250kHz.

# 5.4.6 		Conclusions and Recommendations

When using this mode, I would recommend soft starting the controller to prevent output overshooting due to the natural response of the output filter.