# NCP5203 2-IN-1 DDR POWER CONTROLLER SPICE MODELING

# 5.4.1		Scope

The purpose of this design is to model NCP5203 DDR and VTT Power Controller.

| Spice Design: | 2-in_1 DDR Power Controller |
| --- | --- |
| Performed by: | Kehinde Omolayo |
| Model Last Rev Date: | 1-09-2004 |
| ON Semi Publication Order Number: | ## NCP5203/D |
| ON Semi Revision | ## October, 2003-Rev.0 |
| SPICE File | NCP5203S.LIB (for IsSpice version) NCP5203P.LIB (for PSpice version) |

# 5.4.2		Functional Description

NCP5203 is dual power controller for DDR memory VDDQ and VTT power supplies. It provides complete power control during normal mode (S0), sleep mode (S3), and off mode (S5).

The VDDQ section uses voltage mode synchronous PWM buck topology to regulation high power DDR memory supply. The VTT is a two quadrant linear regulator: it sinks and sources 2.0Adc. Both PWM and linear regulator outputs are adjustable. If the VTT input is connected to the VDDQ output, the VTT output is directly proportional to the VDDQ output.

The system also provides enable inputs for both VDDQ and VTT. When VDDQEN is OFF, the controller is in mode S5 - the PWM output drivers are low and VTT output is high impedance. When VDDQEN and VTTEN are ON, the controller is in normal mode S0 - the VDDQ operates as a synchronous buck converter with 300kH switching frequency, and VTT is ON when VDDQ is in regulation, and the system PGOOD pin is ON when VTT is in regulation. When VDDEN is ON and VTTEN is off, the controller is in sleep mode S3 - the VDDQ operates in asynchronous switch mode at 600kHz.

NCP5203 is equipped with input UVLO, output UVLO & OVLO, and overload fault protections. During transition from S5 to S0 or S3, the system is in soft start mode. The feature prevents VDDQ output voltage from overshooting due to natural response of the output filter during startup.

# 5.4.3		Assumptions

- The temperature for this model is 25 degrees centigrade.

- Thermal Shutdown is not included

- The controller is in S5 mode if FBDDQ is not within 65% and 130% of error amplifier VREF

- Upon output FBDDQ over voltage or under voltage, all outputs shall latch off to protect the load. The controller shall reset once 5VDUAL or VDDQEN is cycled.

# 5.4.4		 Methods of modeling

IsSpice4 and Pspice were used to model the device. The IsSpice net list is as follows and the Pspice version is available upon request:

* IsSpice Model Editor

* |=======================================================

* |                     NCP5203        REV - 01/6/04

* |                (IsSPICE VERSION)

* |                ON SEMICONDUCTOR

* |          2-IN-1 DDR POWER CONTROLLER

* |

* | This model was developed for ON Semiconductor by:

* | OK Power Systems

* |

* |

* |

* | Copyright 2004 OK Power Systems

* | All Rights Reserved

* |

* | The content of this model is subject to change

* | without notice and may not be modified or altered

* | without permission from ON Semiconductor. This model

* | has been carefully checked and is believed to be

* | accurate, however neither OK Power Systems nor

* | ON Semiconductor assume liability for the use of

* | this model or the results obtained from using it.

* |

* | For more information regarding modeling services,

* | model libraries or simulation productors, please

* | call at (631) 654-0253 ext 116 or 106.

* |

* |=======================================================

.SUBCKT NCP5203 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18

*2-IN-1 DDR POWER CONTROLLER

*Terminal Identification

*VDDQEN=1 VTTEN=2 PGOOD=3 REFSNS=4 FBVTT=5 AGND=6 SS=7 COMP=8

*FBDDQ=9 OCDDQ=10 SWDDQ=11 5VDUAL=12 TGDDQ=13 BGDDQ=14

*BST=15 PGND=16 VTT=17 VDDQ=18

.MODEL PG5203NM NMOS Level=1 CBD=8.44P CBS=10.1P CGBO=21.6N

+ CGDO=2N CGSO=2.4N GAMMA=.893 IS=5.18N KP=5.34M LAMBDA=22M

+ MJ=.46 PB=.8 PHI=.75 RD=13 RS=16 VTO=1.1

.MODEL DR5203P PMOS CBD=20.5P CBS=24.6P CGBO=84N CGDO=30N

+ CGSO=36N KP=0.197 RD=0 RS=0 VTO=-0.5

.MODEL DR5203N NMOS CBD=244P CBS=293P CGBO=454N CGDO=180N

+ CGSO=216N KP=0.197 RD=0 VTO=0.5

.MODEL HD5203 D CJO=2P

.MODEL LD5203 D CJO=2P N=0.1

.SUBCKT RS1500 2 3 4 5

C1 11 0 10P

R3 10 8 144.269

C3 8 0 10P

B1 10 0 V=V(2)>1 ? V(5)>1 ? 0 : 2

B2 16 0 V=V(3)>1 ? V(4)>1 ? 0 : 2

B5 4 0 V=V(8)>1 ? 2 : 0

B6 5 0 V=V(11)>1 ? 2 : 0

R1 16 11 144.269

.ENDS

*

R1 1 6 1MEG

R2 2 6 1MEG

R3 12 6 10K

M1 3 62 16 16 PG5203NM

B3 22 0 V=V(21)>1 ? V(1)>(0.95+V(6)) ? V(2)>(0.95+V(6)) ? 1 : 0

X1 56 20 21 53 RS1500

V1 19 0 PULSE 0 2

B4 23 0 V=V(21)>1 ? V(1)>(0.95+V(6)) ? V(2)<(0.95+V(6)) ? 1 : 0

B5 24 0 V=V(22)+V(23)

R4 62 70 20

B1 56 0 V=V(20)<1 ? 2 : V(12)>(4.15+V(6)) ? 0 : 2

B2 20 0 V=V(19)<1 ? 0 : V(12)<(3.95+V(6)) ? 0 : 2

B6 70 16 V=V(25)>1 ? 5 : 0

C1 25 0 10N

R5 25 55 28.85K

B7 55 0 V=V(7)<V(41) ? 0 : V(29)*V(37)

E1 0 26 4 5 3

R6 28 57 1K

C2 57 0 1N

B8 0 27 V=70*(1-EXP(V(26)))/(1+EXP(V(26)))

B9 28 0 V=(V(18)-V(16))/(1+EXP(V(27)))

R21 81 74 1.961

D5 16 17 HD5203

R7 51 75 1MEG

R8 65 75 7.5M

B12 29 0 V=ABS(V(27))<2.624 ? 2 : 0

E7 0 73 51 75 10

D6 17 18 HD5203

E2 0 31 30 9 3

V2 41 6 DC=1.25

B13 0 32 V=69.417*(1-EXP(V(31)))/(1+EXP(V(31)))

B14 0 33 V=69.417*(1-EXP(V(32)))/(1+EXP(V(32)))

B15 64 6 V=3.5/(1+EXP(V(33)))

R9 9 6 2.5MEG

R10 64 8 3.36k

C3 8 6 120.21p

V3 34 0 PULSE 1 1.261042 0 1.565U 1.5N 0.10017 1.66667U

V4 35 0 PULSE 1 1.261042 0 3.13U 3N 0.2003U 3.3333U

B16 36 6 V=V(22)*V(35)+V(23)*V(34)

B17 71 0 V=V(22)*(1/(1+EXP(-1.052*(V(32)+6.627)))-

+1/(1+EXP(-1.258*(V(32)-5.847))) )

B18 38 0 V=V(8)>V(36) ? 2 : 0

B19 39 0 V=V(36)>(1.25+V(6)) ? 0 : V(19)

B21 40 0 V=V(7)<V(41) ? 0 :

+V(9)>0.65*V(30) ? V(9)<1.3*V(30) ? 0 : 2

I1 10 6 DC=35U

D1 6 10 LD5203

B24 54 0 V=V(50)<(V(15)-0.5) ? V(10)>V(11) ? 2 : V(40)

R11 54 44 7.21K

C4 44 0 100P

X6 45 60 46 59 RS1500

B25 45 0 V=V(19)<1 ? 0 :  2*V(24)

B26 60 0 V=V(45)<1 ? 2 : V(44)>1 ? 0 : 2

D2 77 41 LD5203

D3 6 7 LD5203

B27 7 6 I=100U*(1-V(24))

I2 41 7 DC=5U

B34 12 6 I=1M*(V(12)-V(6))*V(24)

M3 69 50 42 42 DR5203P

M4 74 50 11 11 DR5203N

M5 72 49 16 16 DR5203N

M6 79 49 42 42 DR5203P

R12 13 11 100K

R13 14 16 50K

R14 11 16 50K

B28 66 16 V=V(48)>1 ? V(38)<1 ? (V(22)*V(46)*V(39))>2 ? 0 :

+(V(15)-V(16))

R15 66 49 50

R16 67 50 50

B29 67 11 V=V(38)>1 ? (V(24)*V(46)*V(39))>2 ? 0 :

+(V(15)-V(11))

E3 42 16 15 16 1

V5 76 14

V6 81 13

B30 15 16 I=V(49)<(V(15)-0.5) ? I(V5) : 0

B31 15 11 I=V(50)<(V(15)-0.5) ? I(V6) : 0

B33 43 0 V=V(11)<(2+V(16)) ? V(19) : 0

R18 43 48 4.328K

C6 48 0 10P

R24 69 81 2.961

E4 47 6 7 6 1

E5 51 16 57 0 1

E6 61 0 51 75 10

V17 65 17

V18 61 63

V19 52 73

D7 63 58 HD5203

D8 52 58 HD5203

B10 51 75 I=10*(I(V18)+I(V19))

B20 18 16 I=I(V17)>0 ? I(V17) : 0

V10 7 77

R17 17 16 100MEG

R25 79 76 2.961

R26 76 72 0.761

D4 30 47 LD5203

R32 41 30 15k

R37 71 37 10K

C13 37 0 1N

B22 58 0 I=52.5k*V(37)*I(V10)

*Designed by Kehinde Omolayo 1-06-2004

.ENDS

# 5.4.5		 Model Tests

The model was tested and the results were correlated to the actual data points. The IsSpice test fixture for PWM normal mode is shown in figure 5.4.1, and the Spice net List is shown in figure 5.4.2. The SPICE simulation waveforms for normal and sleep modes are shown in figure 5.4.3 and 5.4.4.

Figure 5.4.1. PWM Normal Operation Schematic

Figure 5.4.2. PWM APPLICATION Net List

.TRAN 0.1u 2M 1.98M

.PRINT  TRAN VDDQ

.PRINT  TRAN VTT

.PRINT  TRAN PGOOD

.PRINT  TRAN SWDDQ

X1 17 18 19 14 15 0 16 8 7 10 2 13 12 11 1 0 15 9 NCP5203

D1 13 1 DN5817

.MODEL DN5817 D BV=2.66E+01 CJO=2.31E-10 EG=0.69

+ IBV=3.00E-04 IS=3.53E-05 M=.588 N=1.30 RS=4.47E-02

+ TT=7.20E-11 VJ=2.42 XTI=2

X2 2 11 0 NTD60N02R

.SUBCKT NTD60N02R 1 2 3

M1 4 2 3 3 NMAIN W =285M  L =1U

M2 3 2 3 1 PAUX W =0.35 L = 1U

R1 1 4 0.5M RTEMP

CGS 2 3 0.7N

D1 3 1 DBD

.MODEL NMAIN NMOS (LEVEL=3 ETA=9.5M

+VTO=1.5 KP=0.1M

+ RS=0.05M)

.MODEL PAUX PMOS (LEVEL = 3 TOX = 3E-8  VTO=2

+NSUB=5E16)

.MODEL DBD D (IS=249N N=1.5 RS=5.83M BV=24

+CJO=1.16N VJ=0.8 M=0.42 TT=15.5N)

.MODEL RTEMP R (TC1 = 7E-3 TC2 = 6E-6)

.ENDS

X4 13 12 2 NTD60N02R

C1 1 2 0.1u

R1 2 3 10m

L1 3 9 9.375U

C2 9 4 300u

R2 4 0 50m

C3 8 5 28.647N

R3 5 7 1.851k

C4 8 7 3.184N

C5 9 6 4.773N

R4 6 7 1.112k

R5 9 7 10k

R6 7 0 10k

R7 9 0 0.131

R8 13 10 6.8k

R9 14 9 10K

R10 0 14 10K

C6 0 15 1U

C7 0 16 5N

R11 15 0 1.25

V1 18 0 DC=5

V2 17 0 DC=5

R12 13 19 10K

V3 13 0 DC=5

.END

Figure 5.4.3: PWM waveforms During Normal Mode @300kHz.

Figure 5.4.4: PWM waveforms During Sleep mode @600kHz.

# 5.4.6 		Conclusions and Recommendations

When using this mode, I would recommend initializing the output filter capacitor to zero so that the output can be soft started from zero.